|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2025-2028 |  | SG2-TD119R2/PLEN |
| --- | --- | --- | --- |
|  |  |  | STUDY GROUP 2 |
|  |  |  | Original: English |
| Question(s): |  | 6/2 | Geneva, 5-14 February 2025 |
| TD |  |  |  |
| Source: |  | Editor |  |
| Title: |  | Revised baseline text for ITU-T M.3042 (M.fcnhe): Framework of communication network health evaluation (for consent) |  |
| Contact: |  | Shuangchun LIANG China Mobile | Tel: +86 13701123512 E-mail: liangshuangchun@chinamobile.com |
| Contact: |  | Weiwei Zhang Huawei Technologies Co., Ltd | Tel: +86 13122310608 E-mail: weiweicute.zhang@huawei.com |

| Abstract: | This document gives a revised baseline text for ITU-T M.3042(ex M.fcnhe) "Framework of communication network health evaluation", based upon SG2-C16R1/P and discussion of Q6/2 meeting on 7 February 2025. |
| --- | --- |

Summary:

This document provides the updated baseline text for draft Recommendation ITU-T M.3042(ex M.fcnhe) "Framework of communication network health evaluation", based on the Contribution SG2-C16R1 and the discussion result of Q6/2 meeting on 7 February 2024. The updated text is as following:

1) Table of contents is added.

2) Clause 4 description is updated according to discussion in Q6/2 meeting.

3) Clause 7, 8 and 9 are updated to make the description much clear.

Draft new Recommendation ITU-T M.3042(ex M.fcnhe)

Framework of communication network health evaluation

# Summary

This Recommendation introduces the framework of communication network health evaluation, definition of communication network health index, evaluation objects, and the weighted value of index scoring method. This Recommendation also provides the communication network health evaluation process.

# Keywords

health evaluation, framework, index

Table of Contents

1.	Scope	4

2.	References	4

3.	Definitions	5

3.1.	Terms defined elsewhere	5

3.2.	Terms defined in this Recommendation	6

4.	Abbreviations and acronyms	6

5.	Conventions	7

6.	Communication network health index evaluation framework	7

7.	Communication network health evaluation method	9

7.1.	First level index scoring method	9

7.2.	Second level index scoring method	9

7.3.	Thirdlevel index scoring method	9

8.	Communication network health evaluation process	10

8.1.	Overview	10

8.2.	Evaluation preparation	11

8.3.	Data processing	11

8.4.	Health evaluation	12

8.5.	Evaluation report issuance	13

9.	Definition of third level communication network health index	13

9.1.	Template of communication network health index definition	13

9.2.	Definition and scoring method of the third level index to evaluate availability	13

9.2.1.	Definition and scoring of Resource Utilization Qualified Rate	13

9.2.2.	Definition and scoring of Network Availability Rate	14

9.2.3.	Definition and scoring of Network Performance Qualified Rate	15

9.2.4.	Definition and scoring of Service Successful Accessibility Qualified Rate	16

9.2.5.	Definition and scoring of Service Retainability Performance Qualified Rate	16

9.2.6.	Definition and scoring of Service Integrity Performance Qualified Rate	17

9.3.	Definition and scoring method of the third level index to evaluate reliability	17

9.3.1.	Definition and scoring of Network Entity Fault Report Rate	18

9.3.2.	Definition and scoring of Network Toughness	18

9.3.3.	Definition and scoring of Network Redundancy Protection Rate	19

9.3.4.	Definition and scoring of Core Equipment Room Remote Disaster Recovery Protection Rate	20

9.3.5.	Definition and scoring of Reliabilityrelated Configuration Management Compliance Rate	21

9.3.6.	Definition and scoring ofSupply Chain Qualified Rate	22

9.4.	Definition and scoring method of the third level index to evaluate  security	22

9.4.1.	Definition and scoring of Infrastructure Security Qualified Rate	22

9.4.2.	Definition and scoring of Network Security Qualified Rate	23

9.4.3.	Definition and scoring of VNF Security Qualified Rate	24

9.5.	Definition and scoring method of the third level index to evaluate maintainability	24

9.5.1.	Definition and scoring of Proportion of Capacity Forecasting	24

9.5.2.	Definition and scoring of Proportion of Fault Prevention	25

9.5.3.	Definition and scoring of Mean Time to Repair (MTTR)	25

9.5.4.	Definition and scoring of Operationrelated Configuration Management Compliance Rate	26

9.5.5.	Definition and scoring of Version Compliance Rate	27

9.5.6.	Definition and scoring of Maintenance Efficiency	27

Appendix I	29

Appendix II	30

Appendix III	32

Appendix IV	33

Appendix V	34

Bibliography	35

Draft new Recommendation ITU-T M.3042(ex M.fcnhe)

Framework of communication network health evaluation

- Scope

This Recommendation specifies the framework of communication network health evaluation, including index definition, common indexes, evaluation method and evaluation process.

It is applicable to evaluate the risk of operator’s public networks and industry specific private networks.

- References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU- T E.800]	Recommendation ITU-T E.800, Definitions of terms related to quality of service.

[ITU-T E.802]	Recommendation ITU-T E.802, Framework and methodologies for the determination and application of QoS parameters.

[ITU- T E.840]	Recommendation ITU-T E.840, Statistical framework for end-to-end network performance benchmark scoring and ranking.

[ITU- T E.4110]	Recommendation ITU-T E.4110, Framework for operations requirements of nextgeneration networks and services.

[ITU- T G.808]	Recommendation ITU-T G.808, Terms and definitions for network protection and restoration.

[ITU-T I.113]	Recommendation ITU-T I.113, Vocabulary of terms for broadband aspects of ISDN.

[ITU- T M.60]	Recommendation ITU-T M.60, Maintenance terminology and definitions.

[ITU- T X.641]	Recommendation ITU-T X.641,  Information technology – Quality of Service: Framework

[ITU- T X.739] 	Recommendation ITU-T X.739, Information technology –Information technology - Open Systems Interconnection - Systems Management: Metric objects and attributes

[ITU- T X.1812]     Recommendation ITU-T X.1812, Security framework based on trust relationships for the IMT-2020 ecosystem

- Definitions

  - Terms defined elsewhere

This recommendation uses the following terms defined elsewhere:

3.1.1	availability [ITU-T E.802]: Availability of an item to be in a state to perform a required function at a given instant of time or at any instant of time within a given time interval, assuming that the external resources, if required, are provided.

3.1.2	capacity [ITU-T X.641]: The amount of service that can be provided in a specified period of time. Since the capacity characteristic can be applied to different types of resource, it is quantified using various units. The quantification of capacity also depends on the time unit used for the measurement.

3.1.3	configuration management [ITU-T M.60]: A set of TMN management functions which exercise control over the extension or reduction of a system, the status of the constituent parts and the identity of their allocation.

3.1.4	disaster recovery [ITU-T E.800]: All activities associated with the restoration of a network provided service after disasters. Examples of such disasters are fire, earthquakes, vandalism, bombings, or software malfunctioning.

3.1.5	entity [ITU-T G.911]: Any part, device, subsystem, functional unit, equipment or system that can be individually considered.

3.1.6	fault [ITU-T E.800]: The inability of an item to perform a required function, excluding that inability due to preventive maintenance, lack of external resources or planned actions.

3.1.7	fault localization [ITU-T I.113]: A specific action for locating a faulty entity, by internal or external test systems when fault information is insufficient.

3.1.8	GlobalNetScore [ITU-T E.840]: This represents global network score, i.e., the relative overall quality across various networks or operators against the best performing network. GlobalNetScore is calculated across all services.

3.1.9	indicator [ITU-T E.800]: Value calculated from observed attribute/s of a measure.

3.1.10	maintainability performance [ITU-T E.800]: The ability of an item under stated conditions of use, to be retained in, or restored to, a state in which it can perform a required function, when maintenance is performed under given conditions and using stated procedures and resources.

3.1.11	Mean time to repair (MTTR) [ITU-T E.800]: The expectation of repair time for a statistically significant number of repairs carried out from the instant a fault has been reported to the instant the service restored for use by the customer and usually expressed as the arithmetic mean.

3.1.12 network operator [ITU-T I.113]: An organization that provides and operates a telecommunication network for the purpose of transporting bearers of telecommunication services. NOTE – If the same organization also offers services, it also becomes a service provider.

3.1.13	network performance [ITU-T E.800]: The ability of a network or network portion to provide the functions related to communications between users.

3.1.14 	operation [ITU-T E.4110]: The operations process is the traditional heart of the service provider (SP)enterprise for ensuring across the value chain within and between service providers that all key business objectives and customer experience are supported. The operational environment includes provisioning, configuration, activation and billing of products and services both on the network stratum as well as on the service stratum. It includes all operations processes that support the customer (and the network) operations and management, as well as those that enable direct customer operations with the customer. These processes include both day-to-day and operations support and readiness processes. The view of operations also includes sales management and supplier/partner relationship management.

3.1.15	protection [ITU-T G.808]: The use of pre-assigned capacity between nodes.

3.1.16	recovery [ITU-T M.60]: That event when the item regains the ability to perform a required function after a fault.

3.1.17	reliability [ITU-T E.800]: The ability of an item to perform a required function under given conditions for a given time interval.

3.1.18	repair [ITU-T E.800]: The maintenance carried out after fault recognition and intended to restore an item to a state in which it can perform a required function.

3.1.19	resource utilization [ITU-T X.739]: The amount of capacity in use. Resource utilization can be measured as instantaneous or as an estimate of the mean over a time interval. Instantaneous resource utilization is the amount of capacity in use at a given point in time. Estimated mean resource utilization is an estimate of the mean resource utilization, taken over a time interval.

3.1.20	security [ITU-T E.800]: The term 'security' is used in the sense of minimizing the vulnerabilities of assets and resources. An asset is anything of value. A vulnerability is any weakness that could be exploited to violate a system or the information it contains.

3.1.21	service accessibility performance [ITU-T E.800]: The ability of a service to be obtained, within specified tolerances and other given conditions, when requested by the user.

3.1.22	service integrity performance [ITU-T E.800]: The degree to which a service is provided without excessive impairments, once obtained.

3.2.23 service provider [ITU-T I.113]: An organization that offers a telecommunication service to the customer and/or users.

3.1.24	service retainability performance [ITU-T M.60]: The ability of a service, once obtained, to continue to be provided under given conditions for a requested duration.

3.1.25	supply chain [ITU-T X.1812]: A network of organizations that are involved, through upstream and downstream linkages, in the processes and activities that produce value in the form of products and services in the hands of the ultimate consumer.

  - Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	communication network health: Operating state of a communication network based on availability, reliability, security and maintainability.

3.2.2	communication network health index: A single summary indicator expressed in quantitative terms indicating the degree of communication network health.

3.2.3	network toughness: The ability of a network to preserve its normal operating state.

3.2.4	network toughness index: A single summary indicator expressed in quantitative terms indicating the degree of network toughness.

- Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| AHP CERRDRPR | Analytic Hierarchy Process Core Equipment Room Remote Disaster Recovery Protection Rate |
| --- | --- |
| CPU | Central Processing Unit |
| DCGW | Data Center Gateway |
| EMS | Element Management System |
| EoR | End of Row Switches |
| ETL | Extract Transform Load |
| IMT-2020 ISQR | International Mobile Telecommunications-2020 Infrastructure Security Qualified Rate |
| IP ME | Internet Protocol Maintenance Efficiency |
| MTTR NAR | Mean Time to Repair Network Availability Rate |
| NE NEFRR NPQR NRPR | Network Element Network Entity Fault Report Rate Network Performance Qualified Rat Network Redundancy Protection Rate |
| NSA NSQR NT ORCMCR | Non-Stand Alone Network Security Qualified Rate Network Toughness Operation related Configuration Management Compliance Rate |
| OMU PoCF PoFP RRCMCR | Operation and Management Unit Proportion of Capacity Forecasting Proportion of Fault Prevention Reliability related Configuration Management Compliance Rate |
| RUQR | Resource Utilization Qualified Rate |
| SA SCQR SIPQR SRPQR SSAQR | Stand Alone Supply Chain Qualified Rate Service Integrity Performance Qualified Rate Service Retainability Performance Qualified Rate Service Successful Accessibility Qualified Rate |
| ToR VCR | Top of Rack Switches Version Compliance Rate |
| VNF | Virtual Network Function |
| VNFM VSQR | Virtual Network Function Manager VNF Security Qualified Rate |

- Conventions

In this Recommendation:

The keywords "is required to" indicate a requirement which must be strictly followed and from which no deviation is permitted, if conformance to this Recommendation is to be claimed.

The keywords "is recommended" indicate a requirement which is recommended but which is not absolutely required. Thus, this requirement need not be present to claim conformance.

The keywords "can optionally" indicate an optional requirement which is permissible, without implying any sense of being recommended. This term is not intended to imply that the vendor's implementation must provide the option, and the feature can be optionally enabled by the network operator or service provider which defined in Recommendation [ITU-T I.113]. Rather, it means the vendor may optionally provide the feature and still claim conformance with this Recommendation.

- Communication network health index evaluation framework

With the rapid development of telecommunications industry, complexity of communication network increasing greatly, the difficulty of ensuring network stability is growing significantly. Therefore, communication network health is necessary to be evaluated to resist potential risks by communication network health index. Communication network health index framework contains 4 levels of indexes, as shown in Figure 1.

Figure 1 Communication Network Health Evaluation Index Framework

  - First level index

The first level index includes one index named as “Communication network health”, which is calculated by the sum of weighted second level indexes.

  - Second level indexes

The second level indexes include 4 indexes naming as Availability, Reliability, Security and Maintainability. Each second level index is calculated by the sum of weighted third level indexes that are related.

  - Third level indexes

The third level indexes include 21 indexes altogether which belongs to the different second-level index.

- Availability index is calculated by the sum of third-level indexes such as Resource Utilization Qualified Rate, Network Availability Rate, Network Performance Qualified Rate, Service Successful Accessibility Qualified Rate, Service Retainability Performance Qualified Rate, Service Integrity Performance Qualified Rate.

- Reliability index is calculated by the sum of third-level indexes such as Network Entity Fault Report Rate, Network Toughness, Network Redundancy Protection Rate, Core Equipment Room Remote Disaster Recovery Protection Rate, Reliability related Configuration Management Compliance Rate, Supply Chain Qualified Rate.

- Security index is calculated by the sum of third-level indexes such as Infrastructure Security Qualified Rate index, Network Security Qualified Rate index, VNF Security Qualified Rate index.

- Maintainability index is calculated by the sum of third-level indexes such as Proportion of Capacity forecasting index, Proportion of Fault Prevention index, Mean Time to Repair index, Operation related Configuration Management Compliance Rate index, Version Compliance Rate index and Maintenance Efficiency index.

Each third level index is calculated by the sum of weighted fourth level indexes that is related.

  - Fourth level indexes

The fourth level indexes include CPU usage qualified rate index, memory usage qualified rate index, disk usage qualified rate index, etc. The fourth level indexes are specified according to the evaluated network object such as IMT-2020 SA network or NSA network.

- Communication network health evaluation method

  - First level index scoring method

Communication network health index is calculated by the sum of weighted second level indexes, scoring method is described in Formula (1).

T=w1K1+w2K2+w3K3+w4K4(1)

Where：

T represents Communication network health index score.

K1, K2, K3, K4 represent second level indexes that denote availability, reliability, maintainability and security respectively.

w1, w2, w3, w4 represent weight, the definition of the weights for the T is outside the scope of this Recommendation. The example of weights determined by Analytic Hierarchy Process (AHP) is provided in Appendix IV. Weights can also optionally be determined by statistical significance defined in [ITU- T E.840].

  - Second level index scoring method

Second level index is calculated by the sum of weighted third level indexes, and the scoring method is described in Formula (2).

Km=r=1nμm.r sm.r                           (2)

Where:

Km——represents second level index m,m=1,,2,,3,,4. K1,K2,K3,,K4represent availability, reliability, maintainability and security respectively.

μm.r represents weight of third level index, r is the number of third level indexes, r=1,,2,,…,,n. The definition of the weights is outside the scope of this Recommendation. The example of weights determined by Analytic Hierarchy Process (AHP) is provided in Appendix IV. Weights can also optionally be determined by statistical significance defined in Recommendation [ITU- T E.840].

sm.r represents third level index，r=1,,2,,…,,n.

∑ represents sum of the weighted third level indexes.

Especially for the availability index and reliability index, scoring method can optionally refer to GlobalNetScore defined in Recommendation [ITU-T E.840].

  - Third level index scoring method

Normalization method is used to calculate the third level index score to eliminate the effects of the variation of different third level index data sets. The normalized data value is within a certain range between 0 and 1 with linear scaling, log scaling or z-score scaling method etc. Taking normalization method with linear scaling as an example, for every index value that is below or equal to the lower limit value is changed to lower limit value for example 0, index value that is above or equal to the upper limit value is changed to upper limit value for example 1, and other index value is changed to a value between 0 and 1. A positive correlation linear scaling example is presented in Figure 2.

Figure 2 Communication Network Health Index Scoring Method

For linear scaling method, indexes are divided into positive correlation indexes and negative correlation indexes. The positive correlation index scoring method is provided in Formula (3) and the negative correlation index scoring method is provided in Formula (4).

si=&0,, si∈-∞,,qmin&qi-qminqmax-qmin,, si∈qmin,,qmax&1,, si∈qmax,,∞                                             (3)

sj=&0, sj∈[qmax,,∞)&qmax-qjqmax-qmin, sj∈(qmin,,qmax)&1,, sj∈[0,,qmin)                                                (4)

Where：

si  represents positive correlation index score, i=1,,2,,………,,n.

sj  represents negative correlation index score, j=1,,2,,………,,n.

qi  represents positive correlation index value, i=1,,2,,………,,n.

qj  represents negative correlation index value, j=1,,2,,………,,n.

qmin  represents minimum index value.

qmax represents maximum index value.

- Communication network health evaluation process

  - Overview

Health evaluation process includes evaluation preparation, data processing, health evaluation and evaluation report issuance, as shown in Figure 3.

Figure 3 Communication Network Health Index Evaluation Process

  - Evaluation preparation

Evaluation preparation includes definition of evaluation object, scope, plan and index with corresponding index weight.

Evaluation objects can optionally include one or more communication networks, such as wireless network, core network, wired access network, transmission network, data communication network.

Each evaluation objects correspond to a set of evaluation indexes based on custormized evaluation requirements.

  - Data processing

Data processing includes data collection, Extract Transform Load (ETL), and weight design. The details are as follows.

Data collection indicates collecting data from relevant communication network operation and management system. The collected data includes, but is not limited to network performance data, service quality data, usage measurement data, maintenance data (for example, topology data, configuration data, alarm data) and log data from related reports. The detail functions of data collection are as follows:

- Real-time or quasi-real-time data collection can optionally be provided.

- Three data collection modes can optionally be provided: automatic collection, batch import, and manual input.

- Automatic collection: supports interconnection with the Network Management System and Operational Support System provided by vendors and supports interface protocols such as Socket, File Transfer Protocol, Simple Object Access Protocol, and Simple Network Management Protocol.

- Batch import: the data can be imported based on tool collection or template writing, and imported data can be verified.

- Manual input: data is manually entered into fields provided by a user interface of evaluation system or tool.

- Data extraction indicates a process of extracting required data for network health evaluation from a variety of data sources, which can optionally be structured or unstructured. The following functions are required: database interface and file interface, full and incremental extraction modes, configuration of data extraction rules, such as the extraction mode, extraction time and extraction period.

Data transforming indicates the data is transformed and consolidated for network health evaluation use case. The following functions are required:

- Filtering, cleansing, de-duplicating, validating, and authenticating the data. According to statistical significance in Recommendation [ITU-T E.840], data reliability and confidence level are improved by data filtering, data cleaning, data validating, the upper and lower limits of confidence intervals designed.

- Calculations, translations, or summation based on the raw data, which include changing row and column headers for consistency, converting units of measurement, text changed to number and more.

- Auditing to ensure data quality and compliance.

- Removing, encrypting, or protecting data according to conversion rules.

- Formatting the data into tables or joined tables to match the schema configuration.

Data loading indicates the transformed data is moved into a target data warehouse or data storage. Typically, this involves an initial loading of all data, followed by periodic loading of incremental data changes or full refreshes to erase and replace data. Real-time loading and batch loading can optionally be supported.

The common methods of weight design are subjective weighting and objective weighting. The detailed descriptions are as follows:

- Subjective weighting includes the Delphi method and Analytic Hierarchy Process (AHP).

- Delphi method: Experts in related fields determine the importance of each index based on their own experience and knowledge. The final weight of an index can be obtained by averaging the weight values provided by K experts or by using the frequency statistics method.

- AHP: The indexes of a complex problem are divided into several ordered hierarchies according to the subordinate relationship among the indexes. Domain experts compare the internal indexes of each layer according to a certain ratio scale, and quantify the subjective judgment to form a judgment matrix. Then, the weight of each index in the judgment matrix of each layer relative to the upper layer is calculated by a mathematical method. Finally, the overall ranking of layers is performed, and the weight coefficients of all indexes relative to the overall target are calculated.

- Objective weighting includes coefficient of variation, principal component analysis, and entropy weight method.

- Coefficient of variation: it is calculated as the ratio of the standard deviation of index to the mean of index.

- Principal component analysis: it is a statistical procedure that uses an orthogonal transformation to onvert a group of correlated indexes into another group of irrelevant comprehensive indexes (principal components) based on dimension reduction and variance contribution rate of indexes.

- Entropy weight method: determines the weights of various criteria in a way that reflects the degree of uncertainty associated with each criterion of index with different evaluation objects.

  - Health evaluation

Communication network health evaluation includes calculation of communication network health score which described in chapter 7, and determination of communication network health index level in chapter 6. The score of communication network health index is calculated based the weighted value of index scoring method defined in section 7.1.

The communication network health evaluation objects can optionally include one or more of communication network scenarios such as a core network, bearer network and IP bearer network. Finally, communication network health scores and levels are required to be included in the output of health evaluation results.

  - Evaluation report issuance

Evaluation report. is required to provide summary, evaluation result and guidance. The following elements are necessary to be covered at least: evaluation object, appraiser information, evaluation time and place, 	evaluation methods, communication network health index evaluation score, communication network health level, 	potential risks identification, improvement actions.

- Definition of third level communication network health index

  - Template of communication network health index definition

- Definition (is required): This field contains the long name and description of the communication network health index.

- Scoring method (is required): This field contains the formula or calculation method of the communication network health index.

- Data type of the communication network health index (is required), such as integer, real etc.

- Unit of the communication network health index (is required): The unit can be percentage, second etc.

- Time period of the communication network health index (is required): The period can be week, month, quarter, and year.

  - Definition and scoring method of the third level index to evaluate availability

The third level index to evaluate availability includes Resource Utilization Qualified Rate, Network Availability Rate, Network Performance Qualified Rate, Service Successful Accessibility Qualified Rate, Service Retainability Performance Qualified Rate and Service Integrity Performance Qualified Rate.

    - Definition and scoring of Resource Utilization Qualified Rate

- Definition

Resource utilization refers to the ratio of actual resource usage to all existing resources defined in Recommendation [ITU-T X.739].

Resource utilization qualification rate describes the percentage of resources whose resource usage meets the threshold requirements to the total resources.

- Scoring method

Resource Utilization Qualified Rate is calculated by the fourth level index aggregation calculation of each NE type in the same network scenario within a statistical period. The calculation formulas are as follows:

Resource Utilization Qualified RateRUQR score=minRUQRi

RUQRi=r=1nRUQR_μi.rRUQR_si.r

RUQR_si.r=Score [(the percentage of resources whose resource usage meets the threshold requirements / total resources) *100%]

where:

i represents NE type, r represents fourth level index sample, n is number of fourth level index samples.

RUQRi represents weighted score of the fourth level index RUQR for NE type i

min represents minimum value of weighted scores

RUQR_μi.r  represents weight of the fourth level index RUQR for NE type i and sample number r

RUQR_si.r represents score of the fourth level index RUQR for NE type i and sample number r

∑ represents sum of weighted scores of fourth level indexes

Score represents calculated by deviation standardization method, binomial distribution method, logarithmic function transformation method, deviation method, specific weight method, etc. (Note: For details, see section 7.3. Third level index scoring method.)

- Real value

- percentage

- Week, Month, Quarter, Year

    - Definition and scoring of Network Availability Rate

- Definition

Network Availability Rate describes the evaluation of network services and functions availability. Availability is defined in Recommendation [ITU-T E.802].

- Scoring method

Network Availability Rate is calculated by the fourth level index aggregation calculation of each NE type in the same network scenario within a statistical period. The calculation formulas are as follows:

Network Availability Rate (NAR) score=min NARi

NARi=r=1nNAR_μi.rNAR_si.r

NAR_si.r=Score [(Normal running duration of a communication network entity / total duration) *100%]

where:

i represents NE type, r represents fourth level index sample, n is number of fourth level index samples.

NARi represents weighted score of the fourth level index NAR for NE type i

min represents minimum value of weighted scores

NAR_μi.r represents weight of the fourth level index NARfor NE type i and sample number r

NAR_si.r represents score of the fourth level index NARfor NE type i and sample number r

∑ represents sum of weighted scores of fourth level indexes

Score represents calculated by deviation standardization method, binomial distribution method, logarithmic function transformation method, deviation method, specific weight method, etc. (Note: For details, see section 7.3. Third level index scoring method.)

- Real value

- Percentage

- Week, Month, Quarter, Year

    - Definition and scoring of Network Performance Qualified Rate

- Definition

Network Performance Qualified Rate describes the evaluation of Network Performance qualified by operator or vendor according to predefined threshold. Network performance is defined in Recommendation [ITU-T E.800].

- Scoring method

Network Performance Qualified Rate is calculated by the fourth level index aggregation calculation of each NE type in the same network scenario within a statistical period. The calculation formulas are as follows:

Network Performance Qualified Rate (NPQR) score= min NPQRi

NPQRi=r=1nNPQR_μi.rNPQR_si.r

NPQR_si.r=Score [(Number of communication network entities whose network performance meet the threshold requirements / Total number of communication network entities) *100%]

where:

i represents NE type, r represents fourth level index sample, n is number of fourth level index samples.

NPQRi represents weighted score of the fourth level index NPQR for NE type i

min represents minimum value of weighted scores of the fourth level index

NPQR_μi.r  represents weight of the fourth level index NPQRfor NE type i and sample number r

NPQR_si.r represents score of the fourth level index NPQR for NE type i and sample number r

∑ represents sum of weighted scores of fourth level indexes

Score represents calculated by deviation standardization method, binomial distribution method, logarithmic function transformation method, deviation method, specific weight method, etc. (Note: For details, see section 7.3. Third level index scoring method.)

- Real value

- Percentage

- Week, Month, Quarter, Year

    - Definition and scoring of Service Successful Accessibility Qualified Rate

- Definition

Service Successful Accessibility Qualified Rate describes the evaluation of Service Successful Accessibility qualified by operator or vendor according to predefined threshold. Service accessibility performance is defined in Recommendation [ITU-T E.800].

- Scoring method

Service Successful Accessibility Qualified Rate is calculated by the fourth level index aggregation calculation of each NE type in the same network scenario within a statistical period. The calculation formulas are as follows:

Service Successful Accessibility Qualified Rate (SSAQR) score=r=1nSSAQR_μr SSAQR_sr

SSAQR_sr=Score [(Number of communication network entities whose service access success meets the threshold requirements /Total number of communication network entities) *100%]

where:

r represents fourth level index sample, n is number of fourth level index samples.

SSAQR_μr  represents weight of the fourth level index SSAQR for sample number r

SSAQR_sr represents score of the fourth level index SSAQR for sample number r

∑ represents sum of weighted scores of fourth level indexes

Score represents calculated by deviation standardization method, binomial distribution method, logarithmic function transformation method, deviation method, specific weight method, etc. (Note: For details, see section 7.3. Third level index scoring method.)

- Real value

- Percentage

- Week, Month, Quarter, Year

    - Definition and scoring of Service Retainability Performance Qualified Rate

- Definition

Retainability Performance Qualified Rate describes the evaluation of Service Retainability Performance qualified by operator or vendor according to predefined threshold. Service retainability performance is defined in Recommendation [ITU-T M.60].

- Scoring method

Retainability Performance Qualified Rate is calculated by the fourth level index aggregation calculation of each NE type in the same network scenario within a statistical period. The calculation formulas are as follows:

Service Retainability Performance Qualified Rate (SRPQR) score

=r=1nSRPQR_μr SRPQR_sr

SRPQR_sr= Score [(Number of communication network entities whose service retention meets the threshold requirements / Total number of communication network entities) *100%]

where:

r represents fourth level index sample, n is number of fourth level index samples.

SRPQR_μr  represents weight of the fourth level index SRPQR for sample number r

SRPQR_sr represents score of the fourth level index SRPQR for sample number r

∑ represents sum of weighted scores of fourth level indexes

Score represents calculated by deviation standardization method, binomial distribution method, logarithmic function transformation method, deviation method, specific weight method, etc. (Note: For details, see section 7.3. Third level index scoring method.)

- Real value

- Percentage

- Week, Month, Quarter, Year

    - Definition and scoring of Service Integrity Performance Qualified Rate

- Definition

Service Integrity Performance Qualified Rate describes evaluation of Service Integrity Performance qualified by operator or vendor according to predefined threshold. Service integrity performance is defined in Recommendation [ITU-T E.800].

- Scoring method

Service Integrity Performance Qualified Rate is calculated by the fourth level index aggregation calculation of each NE type in the same network scenario within a statistical period. The calculation formulas are as follows:

Service Integrity Performance Qualified Rate(SIPQR) score=r=1nSIPQR_μr SIPQR_sr

SIPQR_sr= Score [(Number of communication network entities whose service integrity meets the threshold requirements / Total number of communication network entities) *100%]

where:

r represents fourth level index sample, n is number of fourth level index samples.

SIPQR_μr  represents weight of the fourth level index SIPQR for sample number r

SIPQR_sr represents score of the fourth level index SIPQR for sample number r

∑ represents sum of weighted scores of fourth level indexes

Score represents calculated by deviation standardization method, binomial distribution method, logarithmic function transformation method, deviation method, specific weight method, etc. (Note: For details, see section 7.3. Third level index scoring method.)

- Real value

- Percentage

- Week, Month, Quarter, Year

  - Definition and scoring method of the third level indexes to evaluate reliability

The third level indexes to evaluate reliability includes Network Entity Fault Report Rate, Network Toughness, Network Redundancy Protection Rate, Core Equipment Room Remote Disaster Recovery Protection Rate, Reliability related Configuration Management Compliance Rate and Supply Chain Qualified Rate.

    - Definition and scoring of Network Entity Fault Report Rate

- Definition

Network Entity Fault Report Rate indicates the number of faults occurred for a single communication network entity during the period of measurement. Fault is defined in Recommendation [ITU-T E.800].

- Scoring method

Network Entity Fault Report Rate is calculated by the fourth level index aggregation calculation of each NE type in the same network scenario within a statistical period. The calculation formulas are as follows:

Network Entity Fault Report Rate NEFRR score=minNEFRRi

NEFRRi=r=1nNEFRR_μi.rNEFRR_si.r

NEFRR_si.r= Score [(Number of failed communication network entities / Total number of communication network entities) *100%]

where:

i represents NE type, r represents fourth level index sample, n is number of fourth level index samples.

NEFRRi represents weighted score of the fourth level index NEFRR for the same NE type i

min represents minimum value of weighted scores of the fourth level index

NEFRR_μi.r  represents weight of the fourth level index NEFRR for type i and sample number r

NEFRR_si.r represents score of the fourth level index NEFRR for type i and sample number r

This index is a reverse evaluation index and is scored using the min-max normalization method.

∑ represents sum of weighted scores of fourth level index

Score represents calculated by deviation standardization method, binomial distribution method, logarithmic function transformation method, deviation method, specific weight method, etc. (Note: For details, see section 7.3. Third level index scoring method.)

- Real value

- Percentage

- Week, Month, Quarter, Year

    - Definition and scoring of Network Toughness

- Definition

NFV network toughness indicates the ability of a network to preserve its normal operating state.
The Network Toughness can be measured with chaos engineering method. In the case of NFV network toughness evaluation, firstly service performance indicators of NFV network for evaluation is required to be confirmed, which is required to be collected and monitored before, during and after fault injection. Network toughness evaluation indicators is required to be collected during fault injection. When fault injection is finished, recovery value of the performance indicator of NFV network is required to be in the range of values before fault injection.

- Scoring method

Network Toughness is calculated by the fourth level index aggregation calculation of each NE type in the same network scenario within a statistical period. The calculation formulas are as follows:

Network Toughness NT score=minNTi

NTi=r=1nNT_μi.rNT_si.r

NT_si.r= Score [(Number of faulty communication network entities whose key performance indicators fluctuate within an acceptable range / Total number of communication network entities) *100%]

where:

i represents NE type, r represents fourth level index sample, n is number of fourth level index samples.

NTi represents weighted score of the fourth level index NT for the same NE type i

min represents minimum value of weighted scores of the fourth level index

NT_μi.r  represents weight of the fourth level index NT for type i and sample number r

NT_si.r represents score of the fourth level index NT for type i and sample number r

∑ represents sum of weighted scores of fourth level indexes

Score represents calculated by deviation standardization method, binomial distribution method, logarithmic function transformation method, deviation method, specific weight method, etc. (Note: For details, see section 7.3. Third level index scoring method.)

- Real value

- Percentage

- Month, Quarter, Year

    - Definition and scoring of Network Redundancy Protection Rate

- Definition

Network Redundancy Protection Rate indicates the ratio of the number of communication network entities with redundancy protection to the total number of communication network entities during the period of measurement. Protection is defined in Recommendation [ITU-T G.808].

- Scoring method

Network Redundancy Protection Rate is calculated by the fourth level index aggregation calculation of each NE type in the same network scenario within a statistical period. The calculation formulas are as follows:

Network Redundancy Protection RateNRPRscore=minNRPRi

NRPRi=r=1nNRPR_μi.rNRPRi.r

NRPR_si.r= Score [(Number of communication network entities with redundancy protection / Total number of communication network entities) *100%]

where:

i represents NE type, r represents fourth level index sample, n is number of fourth level index samples.

NRPRi represents weighted score of the fourth level index NRPR for the same NE type i

min represents minimum value of weighted scores of the fourth level index

NRPR_μi.r  represents weight of the fourth level index NRPR for type i and sample number r

NRPR_si.r represents score of the fourth level index NRPR for type i and sample number r

∑ represents sum of weighted scores of fourth level indexes

Score represents calculated by deviation standardization method, binomial distribution method, logarithmic function transformation method, deviation method, specific weight method, etc. (Note: For details, see section 7.3. Third level index scoring method.)

- Real value

- Percentage

- Quarter, Year

    - Definition and scoring of Core Equipment Room Remote Disaster Recovery Protection Rate

- Definition

Core Equipment Room Remote Disaster Recovery Protection Rate evaluates the ability of keeping network normal operating state when a disaster occurs in the room where communication network core equipment is located. Disaster recovery is defined in Recommendation [ITU-T E.800].

Remote Disaster Recovery Protection Rate indicates the ratio of the number of communication network entities with remote disaster recovery protection to the total number of communication network entities during the period of measurement.

- Scoring method

Core Equipment Room Remote Disaster Recovery Protection Rate is calculated by the fourth level index aggregation calculation of each NE type in the same network scenario within a statistical period. The calculation formulas are as follows:

Core Equipment Room Remote Disaster Recovery Protection Rate (CERRDRPR) score

=minCERRDRPRi

CERRDRPRi=r=1nCERRDRPR_μi.rCERRDRPR_si.r

CERRDRPR_si.r= Score [(Number of communication network entities with remote disaster recovery protection in the core equipment room / Total number of communication network entities) *100%]

where:

i represents NE type, r represents fourth level index sample, n is number of fourth level index samples.

CERRDRPRi represents weighted score of the fourth level index CERRDRPR for NE type i

min represents minimum value of weighted scores of the fourth level index

CERRDRPR_μi.r  represents weight of the fourth level index CERRDRPR for type i and sample number r

CERRDRPR_si.r represents score of the fourth level index CERRDRPRfor type i and sample number r

∑ represents sum of weighted scores of fourth level indexes

Score represents calculated by deviation standardization method, binomial distribution method, logarithmic function transformation method, deviation method, specific weight method, etc. (Note: For details, see section 7.3. Third level index scoring method.)

- Real value

- Percentage

- Quarter, Year

    - Definition and scoring of Reliability related Configuration Management Compliance Rate

- Definition

Reliability related Configuration Management Compliance Rate indicates the ratio of the number of communication network entities with reliability related configuration management compliance to the total number of communication network entities during the period of measurement. Reliability is defined in Recommendation [ITU-T E.800]. Configuration management is defined in Recommendation [ITU-T M.60].

- Scoring method

Reliability related Configuration Management Compliance Rate is calculated by the fourth level index aggregation calculation of each NE type in the same network scenario within a statistical period. The calculation formulas are as follows:

Reliability related Configuration Management Compliance Rate (RRCMCR) score

=minRRCMCRi

RRCMCRi=r=1nRRCMCR_μi.r RRCMCR_si.r

RRCMCR_si.r= Score [(Number of communication network entities that meet the technical requirements for reliability configuration / Total number of communication network entities) *100%]

where:

i represents NE type, r represents fourth level index sample, n is number of fourth level index samples.

RRCMCRi represents weighted score of the fourth level index RRCMCR for NE type i

min represents minimum value of weighted scores of the fourth level index

RRCMCR_μi.r  represents weight of the fourth level index RRCMCR for type i and sample number r

RRCMCR_si.r represents score of the fourth level index RRCMCR for type i and sample number r

∑ represents sum of weighted scores of fourth level indexes

Score represents calculated by deviation standardization method, binomial distribution method, logarithmic function transformation method, deviation method, specific weight method, etc. (Note: For details, see section 7.3. Third level index scoring method.)

- Real value

- Percentage

- Week, Month, Quarter, Year

    - Definition and scoring of Supply Chain Qualified Rate

- Definition

Supply Chain Qualified Rate indicates ratio of the number of communication network entities with qualified equipment supply availability to the total number of communication network entities during the period of measurement. Supply Chain is defined in Recommendation [ITU-T X.1812].

- Scoring method

Supply Chain Qualified Rate is calculated by the fourth level index aggregation calculation of each NE type in the same network scenario within a statistical period. The calculation formulas are as follows:

Supply Chain Qualified Rate (SCQR) score=r=1nSCQR_μr SCQR_sr

SCQR_sr= Score [(Number of communication network entities with more than two suppliers for core components / Total number of communication network entities) *100%]

where:

r represents fourth level index sample, n is number of fourth level index samples.

SCQR_μr  represents weight of the fourth level index SCQR for sample number r

SCQR_sr represents score of the fourth level index SCQR for sample number r

∑ represents sum of average weighted scores of fourth level indexes of all NE types

Score represents calculated by deviation standardization method, binomial distribution method, logarithmic function transformation method, deviation method, specific weight method, etc. (Note: For details, see section 7.3. Third level index scoring method.)

- Real value

- Percentage

- quarterly and yearly

  - Definition and scoring method of the third level index to evaluate security

The third level index to evaluate security includes Infrastructure Security Qualified Rate, Network Security Qualified Rate and VNF Security Qualified Rate.

    - Definition and scoring of Infrastructure Security Qualified Rate

- Definition

Infrastructure Security Qualified Rate indicates ratio of communication network entities with qualified infrastructure security to the total communication network entities during the period of measurement.

- Scoring method

Infrastructure Security Qualified Rate is calculated by the fourth level index aggregation calculation of each NE type in the same network scenario within a statistical period. The calculation formulas are as follows:

Infrastructure Security Qualified Rate (ISQR) score=r=1nISQR_μr ISQR_sr

ISQR_sr= Score [(Number of communication network entities that pass the infrastructure security check /Total number of communication network entities) *100%]

where:

r represents fourth level index sample, n is number of fourth level index samples.

ISQR_μr  represents weight of the fourth level index ISQR for sample number r

ISQR_sr represents score of the fourth level index ISQR for sample number r

∑ represents sum of average weighted scores of fourth level indexes

Score represents calculated by deviation standardization method, binomial distribution method, logarithmic function transformation method, deviation method, specific weight method, etc. (Note: For details, see section 7.3. Third level index scoring method.)

- Real value

- Percentage

- Week, Month, Quarter, Year

    - Definition and scoring of Network Security Qualified Rate

- Definition

Network Security Qualified Rate indicates ratio of communication network entities with qualified network security to the total communication network entities during the period of measurement.

- Scoring method

Network Security Qualified Rate is calculated by the fourth level index aggregation calculation of each NE type in the same network scenario within a statistical period. The calculation formulas are as follows:

Network Security Qualified Rate (NSQR) score=r=1nNSQR_μr NSQR_sr

NSQR_sr= Score [(Number of communication network entities that pass the network security check / Total number of communication network entities)*100%]

where:

r represents fourth level index sample, n is number of fourth level index samples.

NSQR_μr  represents weight of the fourth level index NSQR for sample number r

NSQR_sr represents score of the fourth level index NSQR for sample number r

∑ represents sum of average weighted scores of fourth level indexes

Score represents calculated by deviation standardization method, binomial distribution method, logarithmic function transformation method, deviation method, specific weight method, etc. (Note: For details, see section 7.3. Third level index scoring method.)

- Real value

- Percentage

- Week, Month, Quarter, Year

    - Definition and scoring of VNF Security Qualified Rate

- Definition

VNF Security Qualified Rate indicates ratio of communication network entities with qualified VNF security to the total communication network entities during the period of measurement.

- Scoring method

VNF Security Qualified Rate is calculated by the fourth level index aggregation calculation of each NE type in the same network scenario within a statistical period. The calculation formulas are as follows:

VNF Security Qualified Rate (VSQR) score=r=1nVSQR_μr VSQR_sr

VSQR_sr= Score [(Number of communication network entities which passed the VNF security check / Total number of VNFs) *100%]

where:

r represents fourth level index sample, n is number of fourth level index samples.

VSQR_μr  represents weight of the fourth level index VSQR for sample number r

VSQR_sr represents score of the fourth level index VSQR for sample number r

∑ represents sum of average weighted scores of fourth level indexes

Score represents calculated by deviation standardization method, binomial distribution method, logarithmic function transformation method, deviation method, specific weight method, etc. (Note: For details, see section 7.3. Third level index scoring method.)

- Real value

- Percentage

- Week, Month, Quarter, Year

  - Definition and scoring method of the third level index to evaluate maintainability

The third level index to evaluate maintainability includes Proportion of Capacity Forecasting, Proportion of Fault Prevention, MTTR, Operation related Configuration Management Compliance Rate, Version Compliance Rate and Maintenance Efficiency.

    - Definition and scoring of Proportion of Capacity Forecasting

- Definition

Proportion of Capacity Forecasting indicates ratio of communication network entities with function of capacity prediction to the total communication network entities during the period of measurement. Capacity is defined in Recommendation [ITU-T X.641].

- Scoring method

The index is calculated by the fourth level index aggregation calculation of each NE type in the same network scenario within a statistical period. The calculation formulas are as follows:

Proportion of Capacity Forecasting (PoCF) score=r=1nPoCF_μr PoCF_sr

PoCF_sr= Score [(Number of communication network entities with the capacity prediction function/Total number of communication network entities) *100%]

where:

r represents fourth level index sample, n is number of fourth level index samples.

PoCF_μr  represents weight of the fourth level index PoCF for sample number r

PoCF_sr represents score of the fourth level index PoCF for sample number r

∑ represents sum of average weighted scores of fourth level indexes

Score represents calculated by deviation standardization method, binomial distribution method, logarithmic function transformation method, deviation method, specific weight method, etc. (Note: For details, see section 7.3. Third level index scoring method.)

- Real value

- Percentage

- Week, Month, Quarter, Year

    - Definition and scoring of Proportion of Fault Prevention

- Definition

Proportion of Fault Prevention indicates ratio of faults that preventive actions are taken to total faults. Fault is defined in Recommendation [ITU-T E.800].

- Scoring method

Proportion of Fault Prevention is calculated by the fourth level index aggregation calculation of each NE type in the same network scenario within a statistical period. The calculation formulas are as follows:

Proportion of Fault Prevention (POoFP) score=r=1nPOoFP_μr POoFP_sr

POoFP_sr= Score [(Number of faults that are proactively prevented/Total number of faults) *100%]

where:

r represents fourth level index sample, n is number of fourth level index samples.

POoFP_μr  represents weight of the fourth level index PoFP for sample number r

POoFP_sr represents score of the fourth level index PoFP for sample number r

∑ represents sum of average weighted scores of fourth level indexes

Score represents calculated by deviation standardization method, binomial distribution method, logarithmic function transformation method, deviation method, specific weight method, etc. (Note: For details, see section 7.3. Third level index scoring method.)

- Real value

- Percentage

- Week, Month, Quarter, Year

    - Definition and scoring of Mean Time to Repair (MTTR)

- Definition

MTTR is defined in Recommendation [ITU-T E.800].

- Scoring method

MTTR is calculated by the fourth level index aggregation calculation of each NE type in the same network scenario within a statistical period. The calculation formulas are as follows:

Mean Time to Repair (MTTR) score=r=1nMTTR_μr MTTR_sr

MTTR_sr= Score [((Fault rectification time – Fault occurrence time)/Total number of faults) *100%]

where:

r represents fourth level index sample, n is number of fourth level index samples.

MTTR_μr represents weight of the fourth level index MTTR for sample number r

MTTR_sr represents score of the fourth level index MTTR for sample number r

This index is a reverse assessment index and is scored using the min-max normalization.

∑ represents sum of average weighted scores of fourth level indexes

Score represents calculated by deviation standardization method, binomial distribution method, logarithmic function transformation method, deviation method, specific weight method, etc. (Note: For details, see section 7.3. Third level index scoring method.)

- Real value

- Percentage

- Week, Month, Quarter, Year

    - Definition and scoring of Operation related Configuration Management Compliance Rate

- Definition

This index indicates the ratio of communication network entities that comply with the operation and maintenance configuration specifications to the total communication network entities during the period of measurement. Operation is defined in Recommendation [ITU-T E.4110]. Configuration management is defined in Recommendation [ITU-T M.60].

- Scoring method

Operation related Configuration Management Compliance Rate is calculated by the fourth level index aggregation calculation of each NE type in the same network scenario within a statistical period. The calculation formulas are as follows:

Operation related Configuration Management Compliance Rate(ORCMCR) score

=r=1nORCMCR_μr ORCMCR_sr

ORCMCR_sr= Score [(Number of communication network entities that meet the technical requirements for operation and maintenance configuration/Total number of communication network entities) *100%]

where:

r represents fourth level index sample, occmcrn is number of fourth level index samples.

ORCMCR_μr represents weight of the fourth level index ORCMCR for sample number r

ORCMCR_sr represents score of the fourth level index ORCMCR for sample number r

∑ represents sum of average weighted scores of fourth level indexes

Score represents calculated by deviation standardization method, binomial distribution method, logarithmic function transformation method, deviation method, specific weight method, etc. (Note: For details, see section 7.3. Third level index scoring method.)

- Real value

- Percentage

- Week, Month, Quarter, Year

    - Definition and scoring of Version Compliance Rate

- Definition

Version Compliance Rate indicates the ratio of communication network entities of the software version that comply with the release specifications to the total communication network entities during the period of measurement.

- Scoring method

Version Compliance Rate is calculated by the fourth level index aggregation calculation of each NE type in the same network scenario within a statistical period. The calculation formulas are as follows:

Version Compliance Rate (VCR) score=r=1nVCR_μr VCR_sr

VCR_sr= Score [(Number of communication network entities whose software version meets the configuration technical requirements/Total number of communication network entities) *100%]

where:

r represents fourth level index sample, n is number of fourth level index samples.

VCR_μr represents weight of the fourth level index VCR for sample number r

VCR_sr represents score of the fourth level index VCR for sample number r

∑ represents sum of average weighted scores of fourth level indexes

Score represents calculated by deviation standardization method, binomial distribution method, logarithmic function transformation method, deviation method, specific weight method, etc. (Note: For details, see section 7.3. Third level index scoring method.)

- Real value

- Percentage

- Week, Month, Quarter, Year

    - Definition and scoring of Maintenance Efficiency

- Definition

This index indicates the ratio of successful one-off maintenance operation to all maintenance operations.

- Scoring method

Maintenance Efficiency is calculated by the fourth level index aggregation calculation of each NE type in the same network scenario within a statistical period. The calculation formulas are as follows:

Maintenance Efficiency (ME) score=r=1nME_μrME_sr

ME_sr= Score [(Number of successful one-off maintenance operations /Total number of maintenance operations) *100%]

where:

r represents fourth level index sample, n is number of fourth level index samples.

ME_μr  represents weight of the fourth level index ME for sample number r

ME_sr represents score of the fourth level index ME for sample number r

∑ represents sum of average weighted scores of fourth level indexes

Score represents calculated by deviation standardization method, binomial distribution method, logarithmic function transformation method, deviation method, specific weight method, etc. (Note: For details, see section 7.3. Third level index scoring method.)

- Real value

- Percentage

- Week, Month, Quarter, Year

# Appendix I

Examples of scoring method for third level indexes

(This appendix does not form an integral part of this Recommendation.)

Examples of the Upper and Lower Limit (described in clause 7.3 Third level index scoring method), and scoring method for indexes in IMT-2020 core network scenario are given. The specific values in the tables are just for reference.

Table I-1 List of examples of scoring method for third level indexes

| Section No. | Third level index Name | NE Type | Fourth level index Name | Lower Limit | Upper Limit | Scoring Method |
| --- | --- | --- | --- | --- | --- | --- |
| 9.2.1 | Resource Utilization Qualified Rate | Server and Storage | CPU Utilization Qualified Rate | 90% | 100% | (+) Min-max normalization |
|  |  | Server and Storage | Memory Utilization Qualified Rate | 90% | 100% | (+) Min-max normalization |
|  |  | Server and Storage | Disk Space Utilization Qualified Rate | 90% | 100% | (+) Min-max normalization |
| 9.3.1 | Network Entity Fault Report Rate | Server | Server Fault Rate | 10% | 20% | (–) Min-max normalization |
|  |  | Storage | Storage Fault Rate | 0% | 3% | Binomial distribution |
| Note: (+) indicates the index is a positive one while (–) indicates the index is a negative one. |  |  |  |  |  |  |

# Appendix II

Examples of scoring method for fourth level indexes

(This appendix does not form an integral part of this Recommendation.)

The examples of scoring method for fourth level indexes which are included in the third level index such as Resource Utilization Qualified Rate and Network Entity Fault Report Rate are given below. The specific values in the tables are reference, not mandatory to be implemented.

Table II-1 List of fourth level indexes included in Resource Utilization Qualified Rate

| NE Type i (i=2) | Assessment Object | Fourth level index Name | Formula for Score of the Fourth level index (RUQR_si.r) (where: i represents NE type, r represents fourth level indexes of the NE type i) |
| --- | --- | --- | --- |
| Server | CPU | CPU Usage Qualified Rate | RUQR_s1.1= Min-Max normalization (Σ(Number of servers whose CPU usage is below the threshold (for example, 80%)/Total number of servers * 100%) |
| Server | Memory | Memory Usage Qualified Rate | RUQR_s1.2= Min-Max normalization (Σ(Number of servers whose memory usage is below the threshold (for example, 80%)/Total number of servers * 100%) |
| Server | Disk | Disk Usage Qualified Rate | RUQR_s1.3= Min-Max normalization (Σ(Number of servers whose disk usage is below the threshold (for example, 80%)/Total number of servers * 100%) |
| Storage | CPU | CPU Usage Qualified Rate | RUQR_s2.1= Min-Max normalization Σ(Number of storage devices whose CPU usage is below the threshold (for example, 80%)/Total number of storage devices * 100%) |
| Storage | Memory | Memory Usage Qualified Rate | RUQR_s2.2= Min-Max normalization Σ(Number of storage devices whose memory usage is below the threshold (for example, 80%)/Total number of storage devices * 100%) |
| Storage | Disk | Disk Usage Qualified Rate | RUQR_s2.3= Min-Max normalization Σ(Number of storage devices whose disk usage is below the threshold (for example, 80%)/Total number of storage devices * 100%) |

Table II-2 List of fourth level indexes included in Network Entity Fault Report Rate

| NE Type i (i=2) | Assessment Object | Fourth level index Name | Formula for Score of the Fourth level index (frr_si.r) (where: i represents NE type, r represents fourth level indexes of the NE type i) |
| --- | --- | --- | --- |
| Server | Server | Server fault rate | frr_s1.1 =Min-Max normalization (Σ(Number of faulty servers)/Total number of servers * 100%) |
| Storage | Storage | Storage fault rate | frr_s2.1 =Binomial distribution (Σ(Number of faulty storage devices/Total number of storage devices * 100%) |

# Appendix III

Example of communication network health level

(This appendix does not form an integral part of this Recommendation.)

It is determined by operators how to use communication network health index score for communication network health classification. For example, communication network health classification includes healthy, sub-healthy, and unhealthy. If the score of communication network health index is T, the maximum of T is 1 without loss of generality. If T is equal to or greater than 90% (threshold determined by operators), the communication network health is considered as healthy. If T is less than 90% and equal to or greater than 80% (threshold determined by operators), the communication network health is considered as sub-healthy. If T is less than 80%, the communication network health is considered as unhealthy.

Table III-1 Definition of Communication Network Health Level

| Communication Network Health Index Level | Explanation |
| --- | --- |
| Healthy | The communication network is available and reliable. Network performance meets service requirements. Network outage risk is low. |
| Sub-healthy | Correcting actions are required to reduce network operation risks and potential network fault. |
| Unhealthy | Repair and recovery actions must be taken immediately. The communication network is out of service. |

# Appendix IV

Analytic hierarchy process for weight determination

(This appendix does not form an integral part of this Recommendation.)

Analytic Hierarchy Process (AHP) is used for communication network health index to assign weight for indexes and dimension. Taking index weight as an example, over three experts (chosen determined by operators) in the field of communication network were questioned to compare the relative importance of the index. The relative importance is measured by scale of 1-9. Then the pairwise comparison matrixDkis constructed, based on the answers of expert as follows:

Dk=di,jn×n

Where：

Dk represents comparison matrix provided by expertk.

di,j represents the ith row jth column of matrix D ,  i,j=1,2,…n.

n×n represents n by nmatrix.

Square root method is recommended to calculate the column vectors of the index’s relative weights provided by each expertλ=λ1，λ2…λn. Weighted average method is recommended to calculate the column vectors of the index weight based on multiple expert’s opinions as follows. The results should be passed the AHP consistency test.

λn=k=1pμk(mj=1mdnj/j=1mmj=1mdnj)

Where：

λn represents weight of indexn.

Σ represents sum of the weighted average weight.

μk represents weight of expert k.

dnj represents the ith row jth column of matrix D,  j=1,2,…n

m represent number of parameters.

Π represents product over a set of relative importance scale provided by expert.

# Appendix V

Proactive Defence for Cloud-Based Network

(This appendix does not form an integral part of this Recommendation.)

V.1	Overview

In the new era of cloud-network synergy, chaos engineering could be used in communication network such as fault injection and fault drill to explore its weak spots and hidden dangers. Transform the fault handling from reactive response to proactive prevention. Manually or automatically inject fault to proactively test communication network’s resilience, potential risks are exposed and shorten the latency of fault incubation period. The minimum blast radius is set before fault injection. By observing and analyzing the expected or unexpected performance of the communication network, organization could verify the effectiveness of fault tolerance mechanisms, such as fault detection, fault localization, fault recovery, and fault isolation.

V.2	Injection Fault Type

Injection fault types are classified as follows:

- Management plane faults, including VNFM/EMS faults and OMU faults.

- VNF layer faults, including service process faults and VNF faults.

- Cloud operating system layer faults, including core components, security attacks, virtual machine faults, switch exceptions, common components and cloud database fault.

- Computing and storage faults, including CPU, memory, or network interface card faults, shared storage faults, multi-point server faults, and storage faults.

- Data center network faults, including ToR faults, EoR faults, and core DCGW faults.

Based on the blast radius, faults can be classified into the following types: network-wide faults affecting a large number of services, partial service NE faults, and management plane faults affect management functions but services are not affected.

V.3	Proactive prevention process

- Solution Preparation: solution preparation should follow four steps as follows:

- set project objects;

- classify project role and responsibility;

- break the object down into executive tasks;

- define owner for each task.

- Solution implementation: inject faults in the specified environment according to the specified solution, record the operation procedure in detail, monitor the impact of faults, and perform network defence to minimize the impact on services.

- Retrospective: retrospective issues, and take actions to reinforce weak spots exposed in the fault drill.

# Bibliography

[b-ITU-T M.3040]	Recommendation ITU-T M.3040 (2019), Principles for on-site telecommunication smart maintenance.

[b-ITU-T Y.2614]	Recommendation ITU-T Y.2614 (2011), Network reliability in public telecommunication data networks.