3GPP TSG-CT Meeting #94-e	CP-213077

# E-Meeting, 13th – 15th December 2021

(revision of CP-yyxxxx)

# Source:	CT4

# Title:	New WID on enhancement on the GTP-U entity restart

# Document for:	Approval

Agenda Item:	17.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	Enhancement on the GTP-U entity restart

## Acronym: EGTPUR

## Unique identifier: 940003

Potential target Release: Rel-17

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

## 3	Justification

About 20 years ago, in Rel-8, 3GPP had decided to remove the usage of Recovery from the GTP Echo Response message for user plane (GTP-U), where the Recovery is defined as the restart counter for a GTP entity, since at that time, a GTP entity comprises a Control Plane part and User Plane part, it is redundant to communicate the restart counter for a GTP entity via both the control plane signalling path and user plane payload path.

However, it is not valid any longer after 3GPP introduces Control Plane and User Plane separation since Rel-14. CP function and UP function must maintain their own restart counter/recovery timestamp, as specified in clause 19a in 3GPP TS 23.007 (PFCP based restart).

More importantly, a restarted GTP-U entity, if its GTP-U contexts can't be restored by the control plane, will send massive amount of GTP Error Indication messages for the unknown incoming GTP-U packets to the peer User Plane function sending those GTP-U packets; and receiving those GTP Error Indication message leads further massive signalling over Sx/N4 interface to report it to the Control Plane function, and subsequently, the Control Plane function will trigger PFCP Session Modification signaling for all affected PDU sessions/PDN connections associated with the restarted GTP-U entity. Such massive signalling over GTP-U interface and Sx/N4 interface should be avoided. In addition, forwarding GTP-U packets towards a restarted GTP-U entity will result in over-charging.

## 4	Objective

The objective of this work is to specify relevant requirements and procedures upon detection of a GTU-entity restart to avoid massive signalling (for GTP Error Indication messages) over GTP-U interface and massive signalling (for reporting GTP-U Error Indication and PFCP Session Modification Signalling messages) over Sx/N4 interface.

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.007 | Specify the mechanism to detect a GTP-U entity restart and the corresponding procedures when the peer GTP-U entity has restarted. | TSG#95 | CT4 responsibility |
| 23.527 | Specify the relevant procedures when the peer GTP-U entity has restarted | TSG#95 | CT4 responsibility |
| 29.060 | Specify potential new IE needed to populate GTP-U recovery timestamp | TSG#95 | CT4 responsibility |
| 29.244 | Specify the protocol impact over Sx/N4 interface when peer GTP-U entity has restarted | TSG#95 | CT4 responsibility |
| 29.281 | Specify the protocol impact to enable the detection of a GTP-U entity restart | TSG#95 | CT4 responsibility |

## 6	Work item Rapporteur(s)

YONG, YANG, Ericsson, frank.yong.yang@ericsson.com

## 7	Work item leadership

CT4

## 8	Aspects that involve other WGs

None

## 9	Supporting Individual Members

Supporting IM name

Ericsson

T-Mobile US

Samsung

Nokia

Nokia Shanghai Bell

AT&T