3GPP TSG CT Meeting #108	CP-251217

Prague, CZ, 9 - 10 June, 2025

# Source:	CT1

# Title:	New WID on CT aspects for ATSSS Rule Provisioning via 3GPP access connected to EPC

# Document for:	Approval

# Agenda Item:	19.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: 	CT aspects for ATSSS Rule Provisioning via 3GPP access connected to EPC

Acronym: 	TEI19_ARP3E-CT

Unique identifier:	1080020

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  | X |  | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
| X | Normative – Stage 3 |
|  | Normative – Other* |

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| TEI19_ ARP3E | SA2 | 1060029 | ATSSS Rule Provisioning via 3GPP access connected to EPC |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| N/A |  |  |

# 3	Justification

After the establishment of a MA PDU Session, the UE receives a prioritized list of ATSSS rules from the SMF. For MA PDU Session establishment with 3GPP access connected to EPC and non-3GPP access connected to 5GC, the ATSSS rules are provided from the PGW-C+SMF to the UE via SM NAS signalling over 5GC, but ATSSS rules are not provided via 3GPP access connected to EPC.

To make the ATSSS Rules provisioning solution complete, it is beneficial that ATSSS rules can be provided to the UE via 3GPP access connected to EPC from Rel-19.

The normative work on ATSSS Rule Provisioning via 3GPP access connected to EPC is specified by SA2 in Rel-19 (SP-241500) and is documented in 3GPP TS 23.502 CR#5184 (SP-241501) to specify that ATSSS rules may be provided to the UE via 3GPP access connected to EPC.

It is proposed to start a new work item to specify the protocol aspects for TEI19_ ARP3E in CT1.

# 4	Objective

Define the protocol aspects based on Stage 2 technical specification 3GPP TS 23.502 developed by SA2 in Rel-19 under the TEI_ ARP3E work item.

For CT1, the expected work includes:

- Specify the protocol aspects on ATSSS rules provisioning via 3GPP access connected to EPC.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| N/A |  |  |  |  |  |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 24.193 | Specify ATSSS rules provisioning via 3GPP access connected to EPC. | TSG CT #108 (June 2025) | CT1 |

# 6	Work item Rapporteur(s)

Chunhui Zhu (NEC), zhu_chunhui@nec.cn

# 7	Work item leadership

CT1

# 8	Aspects that involve other WGs

None

# 9	Supporting Individual Members

Supporting IM name

NEC

Apple

Charter Communications

InterDigital

Tencent

ZTE