3GPP TSG CT Meeting #108	CP-251226

Prague, CZ, 9 - 10 June, 2025	(revision of CP-253265)

# Source:	CT1

# Title:	Revised WID on CT aspects of Multi-Access (ATSSS_Ph4)

# Document for:	Approval

# Agenda item:	19.3

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: CT aspects of Multi-Access (ATSSS_Ph4)

Acronym: MASSS

Unique identifier: 1050020

Potential target Release: Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | x |  | x |  | x |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
| X | Normative – Stage 3 |
|  | Normative – Other* |

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| MASSS | SA2 | 1040034 | Multi-Access (ATSSS_Ph4) |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 940070 | Access Traffic Steering, Switch and Splitting support in the 5G system architecture; Phase 3 | SA2 Rel-18 Work Item |

Dependency on non-3GPP (draft) specification: None

# 3	Justification

In Rel-19 SA2 is working on Multi-Access (MASSS) which covers support for phase 4 of Access Traffic Steering, Switching and Splitting feature (ATSSS_Ph4). The ATSSS_Ph4, involves specifying how the MPQUIC steering functionality defined in Rel-18 as part of phase 3 of Access Traffic Steering, Switching and Splitting feature (ATSSS_Ph3) is extended to be able to steer, switch, and split the IP traffic and the Ethernet traffic. Furthermore, a new SA2 work item (MASSS) "Multi-Access (ATSSS_Ph4)" was approved at SA#104.

Considering the above, impacts on protocols and interfaces under CT WGs' responsibilities are foreseen. The CT WGs will need to carry out stage-3 work in Rel-19 to satisfy the normative requirements arising out of stage-2 work.

Based on progress in SA2, this WID will be updated as needed.

# 4	Objective

The objective of this work item is to specify the CT aspects of (MASSS). The stage-3 work shall be started after the applicable normative stage-2 requirements in SA2 are available.

The stage-3 aspects will include the following (CT WGs impact areas will be identified based on the progress in the normative work in SA2):

CT1:

1) Support the Multipath QUIC-IP (MPQUIC-IP) and Multipath QUIC-Ethernet (MPQUIC-E) steering functionalities that includes:

a)	defining the UE capabilities for supporting the steering functionalities when using MPQUIC for proxying UDP, IP, and Ethernet packets as HTTP datagram payload;

b)	updating the ATSSS rules to support the new steering functionalities when using MPQUIC for proxying IP and Ethernet packets as HTTP datagram payload and specifying the potential impacts of the new steering functionalities on the NAS signalling;

c) 	defining the co-existence of MPQUIC-IP and MPQUIC-E steering functionalities with existing steering functionalities; and

d) 	defining AT Commands to provision the new steering functionalities and steering modes in MA PDU session.

CT3:

1)	Support the new steering functionalities when using MPQUIC for proxying IP and Ethernet packets as HTTP datagram payload in Npcf_SMPolicyControl service.

CT4

1)	Updates to the N4 rules to support the new steering functionalities when using MPQUIC for proxying IP and Ethernet packets as HTTP datagram payload.

2)	Impacts due to the UE capabilities for supporting the steering functionalities when using MPQUIC for proxying UDP, IP, and Ethernet packets as HTTP datagram payload.

# 5	Expected Output and Time scale

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 24.193 | Support for (MPQUIC-IP) and (MPQUIC-E) steering functionalities. | TSG#109 (September 2025) | CT1 Responsibility |
| 24.501 | Update of NAS signalling (PDU Session Establishment procedure) to support any new ATSSS capabilities caused by the new steering functionalities. | TSG#109 (September 2025) | CT1 Responsibility |
| 24.302 | Support for (MPQUIC-IP) and (MPQUIC-E) steering functionalities. | TSG#109 (September 2025) | CT1 Responsibility |
| 27.007 | Support for AT Commands to set new steering functionalities and steering modes in MA PDU session. | TSG#109 (September 2025) | CT1 Responsibility |
| 29.512 | Update of Npcf_SMPolicyControl service for the UE to support MA-PDU session for IP and ethernet traffic flows. | TSG#109 (September 2025) | CT3 Responsibility |
| 29.244 | Updates to N4 rules and impacts due to the UE capabilities to support the new steering functionalities. | TSG#109 (September 2025) | CT4 Responsibility |
| 29.571 | Updates to AtsssCapability attribute to support the new steering functionalities. | TSG#109 (September 2025) | CT4 Responsibility |

# 6	Work item Rapporteur(s)

Gupta, Vivek, Apple (vivek_g_gupta@apple.com)

# 7	Work item leadership

CT1

# 8	Aspects that involve other WGs

None

# 9	Supporting Individual Members

Supporting IM name

Apple

InterDigital Inc.

Nokia

Ericsson

LG Electronics

China Telecom

Intel

CATT

Xiaomi

NEC

Lenovo

ZTE

Qualcomm Incorporated