3GPP TSG-SA Meeting #108	SP-250870

Prague, Czech Republic, June 10 – 13, 2025	was SP-250582

# Source:	SA6

# Title:	New SID on application enablement for Ambient IoT services Phase 2

# Document for:	Approval

# Agenda Item:	6.4.6

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: 	Study on application enablement for Ambient IoT services Phase 2

Acronym: 	FS_AmbientIoT_Ph2_APP

Unique identifier:	1080023

Potential target Release:	Rel-20

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  |  |
| Don't know |  |  |  |  | X |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 950004 | Study on Ambient power-enabled Internet of Things | Rel-19 stage-1 study on Ambient power-enabled Internet of Things |
| 1020030 | Stage 1 of Ambient power-enabled Internet of Things | Rel-19 stage-1 Normative Work on Ambient power-enabled Internet of Things |
| 1020071 | Study on Architecture support of Ambient power-enabled Internet of Things | Rel-19 stage-2 study on Ambient power-enabled Internet of Things in SA2 |
| 1070010 | Architecture support of Ambient power-enabled Internet of Things | Rel-19 stage-2 Normative Work on Ambient power-enabled Internet of Things in SA2 |
| 1020085 | Study on solutions for Ambient IoT (Internet of Things) in NR | Study on Ambient IoT (Internet of Things) in RAN in Rel-19 |
| 1030031 | Study on Security Aspect of Ambient IoT Services in 5G | Study on Architecture support of Ambient power-enabled Internet of Things in SA3 |

# 3	Justification

SA1 (via 3GPP TS 22.369) has specified the service and performance requirements for Ambient IoT since Rel-19, which is defined as an IoT device powered by energy harvesting, being either battery-less or with limited energy storage capability (e.g. using a capacitor), and the energy is provided through the harvesting of radio waves, light, motion, heat, or any other power source that could be seen suitable. An Ambient IoT device has low complexity, small size and lower capabilities and lower power consumption than previously defined 3GPP IoT devices (e.g. NB-IoT/eMTC devices).

Based on SA1 functional service requirements, some potential SA6 impacts maybe as follows:

- Collected information and network capability exposure:

- Subject to user consent, operator policy and 3rd party request, the 5G system shall be able to obtain data from Ambient IoT devices (e.g. sensor data) and provide it to a trusted 3rd party via the 5G network.

- Subject to user consent, operator’s policy and 3rd party request, the 5G system shall provide information about an Ambient IoT device or a group of Ambient IoT devices (e.g. position) to the trusted 3rd party via the 5G network.

- The 5G system shall enable an authorized 3rd party to instruct the 5G network to trigger a group of Ambient IoT devices in a specific area and which action the Ambient IoT devices need to perform when triggered (e.g. send ID, receive further information, send measurement value).

Furthermore, aligned with the same scope as RAN study and considering the SA1 requirements, SA2 (SP-250377) has started to specify the following functions which are focusing on Device 1 in Topology 1 in Rel-19 and plans to finalize the normative work in Q2 2025.

- Architecture to support Ambient IoT

- Ambient IoT Device management

- Ambient IoT Service enabler for Inventory and Command

At present, SA2 could provide the requested AIoT Devices ID(s) to the AF. However, these AIoT Devices ID(s) are so many and may not being consolidated/aggregated before sending to the AF especially for the periodic report. But the 3rd parties (e.g. application server) are more interested in direct and rich results. E.g., the 3rd parties expect to receive the changed AIoT Devices ID(s) compared to last time, the aggregated AIoT Devices ID(s) per different services, or the AIoT Devices ID(s) combined with the e.g. location data, etc. And how to provide this value-added information to the 3rd parties should be in the scope of the SA6 study.

What’s more, SA2 just defined the Permanent ID format for the AIoT Devices. But how to associate these IDs with different services and monitoring the AIoT devices’ status (e.g., enable, disabled, location) continually to inform the application server in time to to minimize the service impact should also be studied.

In summary, Ambient IoT has the potential to benefit a large number of vertical industries, so it is important for SA6 to study how to support the Ambient IoT services in the application enabled layer, aligned with SA1 requirements and SA2 architecture and functionalities, to open up new markets and opportunities for 3GPP systems.

# 4	Objective

This SID studies the support of ambient power-enabled IoT devices in the application enabled layer, based on the services requirements defined in SA1 and architecture and functions defined in SA2 of Rel-19.

The SA6 objectives of this work item include the following:

- Identify the service impacts to the application enabled layer from SA1 use cases and/or other application server’s requirements.

- Study new architecture or enhancements on existing SA6 application enablers for supporting Ambient IoT services in application enabled layer based on bullet 1.

- Expose the value-added service/information of Ambient IoT devices to the consumer

- Study how to enrich the processing (e.g., classification, consolidation) of the collected Ambient IoT device information.

- Study how to utilize the exposure from other working group (e.g., SA2) and provide the value-added Ambient IoT device information to the consumer (e.g., VAL server).

- Monitoring of Ambient IoT devices

- Study how to monitor the status of the Ambient IoT devices to minimize the service impact.

- Identify potential key issues and related solutions, to satisfy the architectural requirements and enhancements identified in bullet 2.

NOTE:	The objectives of bullet 2. depend on the exposure output from SA2 in Rel-19.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 23.700-26 | Study on application enablement for Ambient IoT services | SA#110(Dec 2025) | SA#111 (Mar 2026) | Liping Wu, CATT, wuliping@cictmobile.com |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

# 6	Work item Rapporteur(s)

Liping Wu, CATT, wuliping@cictmobile.com

# 7	Work item leadership

SA6

# 8	Aspects that involve other WGs

SA2 for core network architecture aspects, SA3 for security aspects, and SA5 for management and charging aspects.

# 9	Supporting Individual Members

Supporting IM name

CATT

Huawei

ZTE

China Mobile

KPN

InterDigital

NTT DOCOMO