3GPP TSG-CT WG1 Meeting #157	C1-256011

# Sophia Antipolis, France 13 – 17 October 2025

Source:	ZTE

Title:	Pseudo-CR on command error report procedure

Spec:	3GPP TS 24.369 v1.0.0

Agenda item:	19.70

Document for:	Agreement

1. Introduction

The current specification defines separate rejection messages for different command procedures (e.g., READ COMMAND REJECT, WRITE COMMAND REJECT).

This p-CR proposes the introduction of a unified COMMAND ERROR REPORT message to replace the individual command-specific rejection messages.

2. Reason for Change

This approach using "unified COMMAND ERROR REPORT message" represents the minimal implementation complexity for the device, and from CT1 perspective, this could be a valid and sufficient way to meet the stage 2 requirement, especially if the requirement is primarily about ensuring an explicit rejection when a device cannot process a command.

The specific command that triggered the error is indicated by any command type information element within this unified message, and the detailed failure reason is conveyed using the Cause IE with appropriate cause value.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369 v1.0.0.

* * * First Change * * * *

#### 5.3.2.1	General

The purpose of the read command procedure is to retrieve AIoT data from the AIoT device (see example in figure 5.3.2.1.1).

AIoT device

AIOTF

READ COMMAND

READ COMPLETE

OR

READ COMMAND REJECT

AIoT device

AIOTF

READ COMMAND

READ COMPLETE

OR

COMMAND ERROR REPORT

Figure 5.3.2.1.1: Read command procedure

* * * Next Change * * * *

#### 5.3.2.5	Read command procedure not accepted by AIoT device

Upon receiving a READ COMMAND message, if read command is not supported by the AIoT device, the AIoT device shall send a COMMAND ERROR REPORTmessage with a cause set to #xx "TBD".

Upon receiving a READ COMMAND message, if the command type specific parameters (e.g. the offset to read application data from the AIoT Device user memory, the length of the application data to read.) is invalid, the AIoT device shall send a COMMAND ERROR REPORT message with a cause set to #xx "TBD".

* * * Next Change * * * *

#### 5.3.2.6	Abnormal cases in the AIoT device

The following abnormal cases can be identified:

a)	The read command is unsuccessfully implemented (e.g. user memory operation error).

The AIoT device shall send a COMMAND ERROR REPORT message with a cause set to #xx "TBD".

b)	The application data cannot be found in the requested place of the user memory.

The AIoT device shall send a COMMAND ERROR REPORT message with a cause set to #xx "TBD".

Editor's note:	It is FFS the command and inventory procedure collision case.

* * * Next Change * * * *

#### 5.3.3.1	General

The purpose of the write command procedure is to write the AIoT data to the AIoT device's user memory (see example in figure 5.3.3.1.1).

AIoT device

AIOTF

WRITE COMMAND

WRITE COMPLETE

OR

WRITE COMMAND REJECT

AIoT device

AIOTF

WRITE COMMAND

WRITE COMPLETE

OR

COMMAND ERROR REPORT

Figure 5.3.3.1.1: Write command procedure

* * * Next Change * * * *

#### 5.3.3.5	Write command procedure not accepted by AIoT device

Upon receiving a WRITE COMMAND message, if write command is not supported by the AIoT device, the AIoT device shall send a COMMAND ERROR REPORT message with a cause set to #xx "TBD".

Upon receiving a WRITE COMMAND message, if the command type specific parameters (e.g. the offset where to write the application data, the length of the application data to write) is invalid, the AIoT device shall send a COMMAND ERROR REPORT message with a cause set to #xx "TBD".

Upon receiving a WRITE COMMAND message, if the write command procedure cannot be implemented because the energy will be run out, the AIoT device shall send a COMMAND ERROR REPORT message with a cause set to #xx "TBD".

* * * Next Change * * * *

#### 5.3.3.6	Abnormal cases in the AIoT device

The following abnormal cases can be identified:

a)	The write command is unsuccessfully implemented (e.g. user memory operation error)

The AIoT device shall send a COMMAND ERROR REPORT message with a cause set to #xx "TBD".

* * * Next Change * * * *

## 6.3	Unknown or unforeseen message type

If the AIoT device or the network receives a AIoT NAS message with message type not defined for the AIoT NAS protocol or not implemented by the receiver, it shall return an COMMAND ERROR REPORT message with cause #xx "message type non-existent or not implemented".

NOTE 1:	A message type not defined for the AIoT NAS protocol in the given direction is regarded by the receiver as a message type not defined for the AIoT NAS protocol, see 3GPP TS 24.007 [3].

NOTE 2:	If a UE does not support read command or write command, the UE can be considered as not implementing the respective read command or write command message type.

* * * Next Change * * * *

* * * Next Change * * * *

* * * Next Change * * * *

### 7.1.xx	Command error report

#### 7.1.xx.1	Message definition

The COMMAND ERROR REPORT message is sent by the AIoT device to the AIOTF to indicate that a received command (Read, Write, or Permanent Disable) is not accepted and to provide the cause of the failure..

See table 7.1.xx.1-1.

Message type:	COMMAND ERROR REPORT

Significance:	dual

Direction:	AIoT device to AIOTF

Table 7.1.xx.1-1: COMMAND ERROR REPORT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Command error report message identity | Message type 7.2.2 | M | V | 1 |
|  | Cause | Cause 7.2.8 | M | V | 1 |

* * * Next Change * * * *

### 7.2.2	Message type

The Message type IE and its use are defined in 3GPP TS 24.007 [3]. Table 7.2.2.1 defines the value part of the Message type IE used in the AIoT NAS protocol.

Table 7.2.2-1: Message types for AIoT NAS

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |  | Inventory report |
| 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 |  | Read command |
| 0 | 1 | 0 | 0 | 0 | 0 | 1 | 1 |  | Read complete |
| 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 |  | Write command |
| 0 | 1 | 0 | 0 | 0 | 1 | 0 | 1 |  | Write complete |
| 0 | 1 | 0 | 0 | 0 | 1 | 1 | 0 |  | Permanent disable command |
| 0 | 1 | 0 | 0 | 0 | 1 | 1 | 1 |  | Permanent disable complete |
| x | x | x | x | x | x | x | x |  | Command error report |

Editor's note:	Whether the range for message type of 8 bits is needed or a shorter range is sufficient is FFS. If a shorter range is sufficient, remaining bits in the IE can be used for other mandatory parameters, e.g. part of the security parameters.

* * * Next Change * * * *

### 7.2.8	Cause

The purpose of the cause information element is to indicate the cause of the failure of the read or write operation.

The cause information element is coded as shown in figure 7.2.8-1 and table 7.2.8-1.

The cause is a type 3 information element with length of 2 octets.

Editor's note:	the possible cause values are FFS.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cause IEI |  |  |  |  |  |  |  | octet 1 |
| Cause value |  |  |  |  |  |  |  | octet 2 |

Figure 7.2.8-1: Cause information element

Table 7.2.8-1: Cause information element

| Cause value (octet 2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bits |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| x | x | x | x | x | x | x | x |  | Message type non-existent or not implemented |
| 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 |  | Error, unspecified |
| All other values are spare and if received shall be interpreted as "error, unspecified". |  |  |  |  |  |  |  |  |  |

* * * End of Changes * * * *