3GPP TSG-CT WG1 Meeting #157	C1-256013

# Sophia Antipolis, France 13 – 17 October 2025

Source:	ZTE

Title:	Pseudo-CR on introduction of status procedure

Spec:	3GPP TS 24.369 v1.0.0

Agenda item:	19.70

Document for:	Agreement

1. Introduction

<Introduction part (optional)>

2. Reason for Change

According to following texts quoted from TS 23.369, it mandates an explicit reject response when the AIoT device does not support the read or write command.

"5.2.2.2	Read and Write Commands

It is optional for an AIoT Device to support of Read and Write Commands….

……

… …

If an AIoT Device does not support Read Request or Write Request commands, or the parameters in the commands are invalid the AIoT Device shall reject the command.

"

If a UE does not support read command or write command, the UE can be considered as not implementing the respective read command or write command message type.

Therefore, it proposes to introduce a STATUS message which is similar to the status procedure mechanism used in normal NAS to fulfil the requirement above.

In addition, existing statement in clause 6.3 for unknown or unforeseen message type needs to be revised since it specifies the AIoT NAS message shall be ignored other than rejected.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369 v1.0.0.

* * * First Change * * * *

#### 5.3.2.5	Read command procedure not accepted by AIoT device

Upon receiving a READ COMMAND message, if the command type specific parameters (e.g. the offset to read application data from the AIoT Device user memory, the length of the application data to read.) is invalid, the AIoT device shall send a READ COMMAND REJECT message with a cause set to #xx "TBD".

* * * Next Change * * * *

#### 5.3.3.5	Write command procedure not accepted by AIoT device

Upon receiving a WRITE COMMAND message, if the command type specific parameters (e.g. the offset where to write the application data, the length of the application data to write) is invalid, the AIoT device shall send a WRITE COMMAND REJECT message with a cause set to #xx "TBD".

Upon receiving a WRITE COMMAND message, if the write command procedure cannot be implemented because the energy will be run out, the AIoT device shall send a WRITE COMMAND REJECT message with a cause set to #xx "TBD".

* * * Next Change * * * *

## 5.x	Status procedure

### 5.x.1	General

The purpose of the status procedure is to report in the STATUS message certain error (e.g. unknown message type) detected upon receipt of the AIoT NAS message from the AIOTF (see example in figure 5.x.1).

AIoT device

AIOTF

STATUS

Figure 5.x.1: Status procedure

### 5.x.2	Status received in the AIOTF

On receipt of a STATUS message in the AIOTF, the local actions to be taken by the AIOTF are implementation dependent.

* * * Next Change * * * *

## 6.3	Unknown or unforeseen message type

If the AIoT device or the network receives a AIoT NAS message with message type not defined for the AIoT NAS protocol or not implemented by the receiver, it shall return a STATUS message with cause #xx "message type non-existent or not implemented".

NOTE 1:	A message type not defined for the AIoT NAS protocol in the given direction is regarded by the receiver as a message type not defined for the AIoT NAS protocol, see 3GPP TS 24.007 [3].

NOTE 2:	If a UE does not support read command or write command, the UE can be considered as not implementing the respective read command or write command message type.

* * * Next Change * * * *

### 7.1.xx	Status

#### 7.1.xx.1	Message definition

The STATUS message is sent by the AIoT device to the AIOTF to report certain error, e.g. unknown message type.

See table 7.1.xx.1-1.

Message type:	STATUS

Significance:	dual

Direction:	AIoT device to AIOTF

Table 7.1.xx.1-1: STATUS message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Status message identity | Message type 7.2.2 | M | V | 1 |
|  | Cause | Cause 7.2.8 | M | V | 1 |

* * * Next Change * * * *

### 7.2.2	Message type

The Message type IE and its use are defined in 3GPP TS 24.007 [3]. Table 7.2.2.1 defines the value part of the Message type IE used in the AIoT NAS protocol.

Table 7.2.2-1: Message types for AIoT NAS

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |  | Inventory report |
| 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 |  | Read command |
| 0 | 1 | 0 | 0 | 0 | 0 | 1 | 1 |  | Read complete |
| 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 |  | Read command reject |
| 0 | 1 | 0 | 0 | 0 | 1 | 0 | 1 |  | Write command |
| 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 |  | Write complete |
| 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 |  | Write command reject |
| 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 |  | Permanent disable command |
| 0 | 0 | 0 | 0 | 1 | 0 | 0 | 1 |  | Permanent disable complete |
| x | x | x | x | x | x | x | x |  | Status |

Editor's note:	Whether the range for message type of 8 bits is needed or a shorter range is sufficient is FFS. If a shorter range is sufficient, remaining bits in the IE can be used for other mandatory parameters, e.g. part of the security parameters.

* * * Next Change * * * *

### 7.2.8	Cause

The purpose of the cause information element is to indicate the cause of the failure of the read or write operation.

The cause information element is coded as shown in figure 7.2.8-1 and table 7.2.8-1.

The cause is a type 3 information element with length of 2 octets.

Editor's note:	the possible cause values are FFS.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cause IEI |  |  |  |  |  |  |  | octet 1 |
| Cause value |  |  |  |  |  |  |  | octet 2 |

Figure 7.2.8-1: Cause information element

Table 7.2.8-1: Cause information element

| Cause value (octet 2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bits |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| x | x | x | x | x | x | x | x |  | Message type non-existent or not implemented |
| 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 |  | Error, unspecified |
| All other values are spare and if received shall be interpreted as "error, unspecified". |  |  |  |  |  |  |  |  |  |

* * * End of Changes * * * *