3GPP TSG-CT WG1 Meeting #157	C1-256019

Sophia Antipolis, France, 13-17 October 2025

Source:	China Mobile

Title:	pCR on the command abnormal cases

Spec:	3GPP TS 24.369 v1.0.0

Agenda item:	19.70

Document for:	Agreement

1. Introduction

This p-CR proposes to consider the case the disable command procedure not fulfilled by the AIoT device.

2. Reason for Change

In the subclause 5.3.4.3/5.3.4.4, the permanent disable command procedure both accepted by AIoT device and completion by the AIOTF are specified, and CT1 agreed in CT1#156 that retransmission for command messages is not needed in no response case. But it is unclear whether the AIOTF determines the AIoT device is disable or not after unsuccessful permanent disable command procedure, especially for the case permanent disable command procedure can be triggered by the NW, according to TS 23.369 subclause 5.2.2.3.

In TS 24.501, no response case of Network-initiated de-registration procedure is specified for the network side as follows. i.e. The network shall handle the UE as de-registrated.

a)	T3522 time-out

On the first expiry of the timer, the network shall retransmit the DEREGISTRATION REQUEST message and shall start timer T3522. This retransmission is repeated four times, i.e. on the fifth expiry of timer T3522, the de-registration procedure shall be aborted. The network shall change to the state 5GMM-DEREGISTERED for the access type which the de-registration procedure is intended for.

It is suggested to specify the requirement of the AIOTF for unsuccessful permanent disable command procedure in the similar way as 24.501.

Moreover, the AIOFT needs to handle the case the AIoT device does not receive the disable command and still response to the AIoT paging.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369 v1.0.0.

* * * First Change * * * *

5.2.6	Abnormal cases in the AIOTF

The following abnormal cases can be identified:

a)	Lower layer failure

In the case the lower layer indication of an inventory failure received as specified by 3GPP TS 38.413 [10] from all the selected NG-RAN(s), the AIOTF shall abort the inventory procedure.

In the case the lower layer indication of no inventory response from all the selected NG-RAN(s), the AIOTF shall abort the inventory procedure.

b)	Inventory report received from a permanent disabled AIoT device

In the case an INVENTORY REPORT message received from a permanent disabled AIoT device during an inventory procedure for a group of AIoT devices, the AIOTF shall discard the INVENTORY REPORT message.

* * * Next Change * * * *

5.3.4.4	Permanent disable command procedure completion by the AIOTF

Upon receiving the PERMANENT DISABLE COMPLETE message, the AIOTF shall consider the permanent disable command procedure as successfully completed.

In the case of no PERMANENT DISABLE COMPLETE message received, the AIOTF shall consider the communication capability of the AIoT device is permanent disabled.

* * * End of Changes * * * *