3GPP TSG-CT WG1 Meeting #157	C1-256034

Sophia-Antipolis, France 13-17 October 2025

Source:	CEWiT

Title:	Pseudo-CR on Timer for command procedures

Spec:	3GPP TS 24.369

Agenda item:	19.70

Document for:	Approval

1. Reason for Change

AIOTF needs a timer to abort the command procedures in case it receives no response from NG-RAN for the specific device.

The NG-RAN may not send a command response to AIOTF due to lower layer failure between NG-RAN and AIOTF or no response received at NG-RAN from the device.

2. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369.

*******

* * * First Change * * *

### 5.3.1	General

This clause describes command procedures between the AIOTF and AIoT device. Command procedures are initiated by the AIOTF to perform read, write, or permanent disable operation on the AIoT device. The AIOTF shall start timer Tx after initiating the command procedure.

* * * Next Change * * *

#### 5.3.2.4	Read command procedure completion by the AIOTF

Upon receiving the READ COMPLETE message, the AIOTF shall consider the read command procedure as successfully completed and stop timer Tx if running.

* * * Next Change * * *

#### 5.3.3.4	Write command procedure completion by the AIOTF

Upon receiving the WRITE COMPLETE message, the AIOTF shall consider the write command procedure as successfully completed and stop timer Tx if running.

* * * Next Change * * *

#### 5.3.4.4	Permanent disable command procedure completion by the AIOTF

Upon receiving the PERMANENT DISABLE COMPLETE message, the AIOTF shall consider the permanent disable command procedure as successfully completed and stop timer Tx if running.

* * * Next Change * * *

## 8.1	General

This clause describes the definition of the timers for AIoT.

## 8.2	Timers of command procedure

Timers of command procedure is shown in table 8.2.1.

Table 8.2.1: Timers of command procedure – AIOTF side

| TIMER NUM. |  | TIMER VALUE |  | CAUSE OF START | NORMAL STOP |  | ON EXPIRY |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tx |  | NOTE 1 |  | Transmission of READ COMMAND message Transmission of WRITE COMMAND message Transmission of PERMANENT DISABLE COMMAND message | READ COMPLETE message received or READ COMMAND REJECT message received WRITE COMPLETE message received or WRITE COMMAND REJECT message received PERMANENT DISABLE COMPLETE message received |  | Abort the procedure |  |
| NOTE 1: The value of this timer is network implementation specific. |  |  |  |  |

* * * End of Change(s) * * *