3GPP TSG-CT WG1 Meeting #157	C1-256051

# Sophia Antipolis, France, 13-17 October 2025

Title:	LS on information provided by the lower layers for AIoT device

Response to:	-

Release:	Release19

Work Item:	AmbientIoT-CT

Source:	CT1

To:	RAN2, RAN3

Cc:	SA2, SA3

Contact Person:

Name:	Yizhong Zhang

Tel. Number:	-

E-mail Address:	yizhong.zhang@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

1. Overall Description:

CT1 is working on the stage 3 aspects of AmbientIoT-CT, and find the following issues that may need coordination with RAN2 and RAN3.

Issue 1

According to TS 23.369, the AIoT identification information can include a single AIoT device identity to discover an AIoT device or a filtering information to discover a group of AIoT devices. The AIoT identification information is provided by the lower layers, is not a part of the AIoT NAS message, and is used to determine whether the AIoT device is matched or not by the AIoT NAS layer.

Question 1: CT1 kindly asks RAN2 and RAN3 to confirm whether the lower layers can indicate which information (i.e. single device ID or filtering information) is included in the AIoT identification information?

Issue 2

According to TS 33.369, the T-ID is used to protect the privacy of AIoT device during the inventory procedure. CT1 notices that the latest SA3 requirement states a T-ID handling indication can be provided by the lower layers, see follows:

-	In step 2 and 3 the AIOTF includes indication of type of T-ID handling. T-ID can be either concealed type or stored type. The concealed type can be based on either the stored T-ID or the permanent identifier. If needed the handling also indicates whether the stored T-ID type shall be updated with or without a command. NG-RAN includes the T-ID handling in the paging message.

The T-ID handling indication is not a part of the AIoT NAS message. Hence CT1 assume that the T-ID handling indication can be provided by lower layers and indicates one of the following: "stored type", "concealed type based on stored T-ID", "concealed type based on permanent ID", and "update the stored T-ID".

Question 2: CT1 kindly asks RAN2 and RAN3 to confirm whether the lower layers can provide the T-ID handling indication to the AIoT NAS layer.

Issue 3

According to TS 33.369, the ciphering is optionally supported for the AIoT command messages. CT1 notices that the latest SA3 requirement states that, when the cyphering is activated, the AIoT NAS layer can obtain an indication from the lower layers, see follows:

Confidentiality protection of AIoT NAS messages between the AIoT device and the AIOTF is optional to use.

The AIoT device shall support the following ciphering algorithms:

NEA0 and 128-NEA2 as specified in Annex D of TS 33.501 [5].

and

3.	The AIOTF shall construct a AIOT NAS Command Request and protect the message based on the KCommand_enc, KCommand_int, the confidentiality and integrity algorithms for the AIoT device. The AIOTF shall send the protected Command Request containing an indication on whether cyphering is activated to NG-RAN.

The cyphering activation indication is not a part of the AIoT NAS message. Hence CT1 assumes that the cyphering activation indication can be provided by lower layers and indicates one of the following: "NEA2", "NEA0", and "Not activated".

Question 3: CT1 kindly asks RAN2 and RAN3 to confirm whether the lower layers can provide the cyphering activation indication to the AIoT NAS layer.

CT1 kindly asks RAN2 and RAN3 to provide answers to the above questions.

2. Actions:

To RAN2, RAN3 group.

ACTION: 	CT1 kindly asks RAN2 and RAN3 to provide answers to the above questions.

3. Date of Next CT1 Meetings:

CT1#158                           17th November- 21st November 2025............Dallas, US

CT1#159                           9th February- 13th February 2026..................India