3GPP TSG-CT WG1 Meeting #157	C1-256196

# Sophia-Antipolis, France, 13-17 October 2025

Source:	Qualcomm Incorporated, LG Electronics

Title:	Indication to lower layers when the AIoT NAS message is ignored.

Spec:	3GPP TS 24.369 v1.0.0

Agenda item:	19.70

Document for:	Decision

1. Introduction

Regarding the issue of delayed AIoT command procedure, RAN2 has agreed a solution, the so called AS based solution: AIoT MAC entity sends the D2R message with indicating 0 SDU, and the reader sends another D2R scheduling info.

TS 38.391 version 1.0.0:

### 5.4.2	D2R message transmission

Upon initiation of the procedure corresponding to the A-IoT access procedure or reception of an R2D Upper Layer Data Transfer message which contains either the Data SDU field or the Received Data Size field set to 0, the A-IoT MAC entity shall:

1>	apply the D2R Scheduling Info, received in the A-IoT Paging message with Access Type set to CFA or in the Random ID Response message or in the R2D Upper Layer Data Transfer message containing either the Data SDU field or the Received Data Size field set to 0, whichever triggered the initiation of the procedure;

1>	if upper layer data is available to be transmitted:

2>	if the size of the resulting MAC PDU including the total upper layer data is smaller than or equal to the resource size given by the D2R TBS in the D2R Scheduling Info:

3>	generate the D2R Upper Layer Data Transfer message, as follows:

4>	set the More Data Indication field to value 0;

4>	include SDU Length field and Data SDU field;

4>	if the size of the resulting MAC PDU including the total upper layer data is smaller than the resource size given by the D2R TBS in the D2R Scheduling Info:

5>	include the MAC Padding field;

3>	instruct the physical layer to transmit the D2R Upper Layer Data Transfer message and indicate the L1 parameters to the physical layer, as specified in clause 6.2.1.6;

2>	else (the size of the resulting MAC PDU including the total upper layer data is larger than the resource size given by the D2R TBS in the D2R Scheduling Info):

3>	initiate the segmentation procedure for the upper layer data SDU as specified in 5.4.4;

1>	else (i.e., upper layer data is not available to be transmitted):

2>	generate the D2R Upper Layer Data Transfer message, as follows:

3>	set the More Data Indication field to 0;

3>	set the SDU Length field to 0;

3>	if the size of the resulting MAC PDU including no upper layer data is smaller than the resource size given by the D2R TBS in the D2R Scheduling Info:

4>	include the MAC Padding field;

2>	instruct the physical layer to transmit the D2R Upper Layer Data Transfer message and indicate the L1 parameters to the physical layer, as specified in clause 6.2.1.6.

NOTE:	It is up to reader’s implementation to avoid segmentation for the D2R Upper Layer Data Transfer message for inventory response.

2. Reason for Change

However, there is still an open issue. If the AIoT NAS layer does not provide any response—for example, if an AIoT device chooses to ignore the AIoT NAS message because it is e.g., too short, and the AIoT NAS layer generates no reply to the network — then the lower layers (especially the MAC layer) cannot independently produce a “command failure” response. In such cases, the lower layers must find a way to prevent the Reader from continuously sending R2D messages while waiting for a D2R response. The Reader may assume the NAS response is simply delayed, when in reality, no response is coming at all.

Therefore, it is proposed to clarify the behavior of the AIoT NAS layer in cases where an AIoT NAS message is ignored due to a protocol error or other relevant reasons. In such cases, the AIoT NAS layer shall provide an explicit indication to the lower layers that the message has been ignored.

3. Conclusions

The AIoT device shall indicate to the lower layers that the AIoT NAS message was ignored, so that the lower layers stop AIoT data transmission.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369.

***** First change *****

## 6.1	General

The procedures specified in the present document apply to those messages which pass the checks described in this clause.

This clause also specifies procedures for the handling of unknown, unforeseen, and erroneous protocol data by the receiving entity. These procedures are called "error handling procedures", but in addition to providing recovery mechanisms for error situations they define a compatibility mechanism for future extensions of the protocols.

Clauses 6.2 to 6.7 shall be applied in order of precedence.

Detailed error handling procedures in the network are implementation dependent and may vary from network to network. However, when extensions of this protocol are developed, networks will be assumed to have the error handling that is indicated in this clause as mandatory ("shall") and that is indicated as strongly recommended ("should").

Also, the error handling of the network is only considered as mandatory or strongly recommended when certain thresholds for errors are not reached during a dedicated connection.

For definition of semantical and syntactical errors see 3GPP TS 24.007 [3], clause 11.4.2.

In case of a received AIoT NAS message being ignored by the AIoT device as specified in clause 6.2, the AIoT device shall indicate to the lower layers that the AIoT NAS message was ignored, so that the lower layers stop AIoT data transmission as specified in TS 38.391 [8].

Editor's note:	Whether AIoT NAS protocol supports conditional IEs and comprehension required IEs is FFS.

Editor's note:	Whether AIoT NAS protocol supports TLV-E format is FFS.

***** End of change *****