3GPP TSG-CT WG1 Meeting #157	C1-256233

# Sophia-Antipolis, France 13 – 17 October 2025

Source:	Nokia

Title:	Digital asset media management service operation definition for DOWNLOAD

Spec:	3GPP TS 24.550 v1.1.0

Agenda item:	19.42

Document for:	Agreement

1. Introduction

TS 23.448 specifies services for digital asset media management. This p-CR proposes the download of digital asset media management service operation to 3GPP TS 24.550.

2. Reason for Change

The pCR defines the digital asset media management service operations for download as part of Digital Asset Media Management Service specified in TS 23.438. This service operation shall enable a digital asset client to download the digital asset media information at the at the digital asset server matching the discovery filters provided in the download request.

3. Proposal

It is to agree the proposed enhancement of spec for 3GPP TS 24.550 v1.1.0.

* * * First Change * * * *

###### 5.4.3.2.b.2	DA client downloading digital asset media request at DA server using SS_DAMediaManagement_Download operation

Upon receiving the request from the VAL client for the downloading of digital asset media information, the DA client shall send an HTTP POST request to the DA server on the resource URIs identifying the "Digital Asset Lists" and the "Digital Asset Media Information" resource collection as specified in clause "TBD". The body of the HTTP POST request shall include the data structure as specified in clause "TBD".

Upon reception of the HTTP POST request, the DA server shall perform the downloading of the stored digital asset media information of the spatial map profile as per the sent digital asset media identifier, if the DA client is authorized. If the digital asset media download operation:

1)	is successful, the DA server shall send the POST response with HTTP "200 OK" status code and the POST response body including the data structure as described in the clause "TBD", which includes the downloaded digital asset media information; or

2)	failed, then the POST response is sent to the DA client with the appropriate HTTP status code indicating the "failure" and the data structure as described in the clause "TBD".

Editor’s note: The reference to clauses under "TBD" to be resolved with correct reference number.

* * * End of Changes * * * *