3GPP TSG-CT WG1 Meeting #157	C1-256241

Sophia-Antipolis, France 13-17 October 2025	was C1-xxxxxx

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.229 | CR | 6752 | rev | - | Current version: | 19.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the parameter for satellite id |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2025-10-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to CR1656(S2-2507635),Npcf_PolicyAuthorization_Notify and Nsmf_EventExposure_Notify include only the DNAI in UP path change notifications, which is derived from the satellite identifier, rather than sending the satellite identifier redundantly. P-CSCF derives the satellite ID from the DNAI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that DNAI is received by P-CSCF from PCF, not satellite identifier. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect use of satellite identifier |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Y.2.1, Y.2.2.1, Y.2.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

Y.2.1	Introduction

The optimized media routing via IMS described in this annex refers to a routing of media between UEs under the coverage of the same or of different serving satellites, using IMS satellite media plane optimization and with the user plane traffic not transiting through any network elements on the ground as described in 3GPP TS 23.228 [7].

NOTE 1:	In this Release of the specification, this feature is supported only for IMS voice/video service and for UEs belonging to the same PLMN and in the non-roaming scenario.

Subject to operator policy, the network may support UE-Satellite-UE communication in IMS, i.e. optimized media routing via IMS.

If the network supports optimized media routing via IMS, then at call set-up, the P-CSCF determines if the activation of optimized media routing via IMS is possible. This is performed by the P-CSCF on the UE originating side determines the network support of the optimized media routing via IMS and negotiate with the P-CSCF on the terminating side if it supports optimized media routing via IMS. The support of optimized media routing via IMS is determined by the originating P-CSCF based on the reception of the DNAI associated with satellite identifier of the satellite serving the UE. The P-CSCF derives the satellite ID from the DNAI.When the P-CSCF activates optimized media routing via IMS, then the P-CSCF selects an IMS-AGW onboard the satellite.

During handover, if the P-CSCF determines, based on the satellite identifiers of the satellite serving the local UE and of the satellite serving the remote UE, that optimized media routing via IMS cannot continue between a satellite that is about to serve the local UE and the satellite serving the remote UE, the P-CSCF activates ground fallback routing by selecting an IMS-AGW on the ground.

NOTE 2:	In this Release of the specification, how the P-CSCF uses the satellite identifier to derive whether optimized media routing over IMS can continue is not covered. The derivation of the satellite ID from the DNAI is based on the operator's policy and implementation.The lack of the DNAI derived from satellite identifier means optimized media routing is no longer possible.

NOTE 3:	The ground fallback routing refers to the case with the media transiting through the ground segment.

* * * Next Change * * * *

Y.2.2.1	Determining the activation of optimized media routing

If the P-CSCF supports optimized media routing via IMS, the P-CSCF determines that the UE is to be in its home country, the UE indicates in the SIP INVITE request message that P-Access-Network-Info header field value is set to indicate an IP-CAN associated with satellite NG-RAN, and the DNAI associated with satellite identifier of the satellite serving the UE is received by the P-CSCF, then the P-CSCF may initiate the procedure for optimized media routing via IMS.

NOTE 1:	In order to determine the UE is in its home country, the P-CSCF checks the MCC fields received in the P-Access-Network-Info header field against the MCC code(s) stored in the P-CSCF.

NOTE 2:	The P-CSCF receives from the PCF, in the Access Network Information, the DNAI associated with satellite identifier of the satellite serving the UE. The PCF receives the information from the SMF.

NOTE 3:	For the purpose of optimized media routing via IMS, in this Release of the specification, in the P-Access-Network-Info header field, the access-class field set to "3GPP-NR-SAT", "3GPP-NR(LEO)", "3GPP-NR(MEO)", or "3GPP-NR(OTHERSAT)" are considered associated with satellite NG-RAN.

NOTE 4:	In this Release of the specification, how the P-CSCF uses the satellite identifier to derive whether optimized media routing over IMS is possible is not specified. The derivation of the satellite ID from the DNAI is based on the operator's policy and implementation.

Upon receiving the SIP INVITE message, the P-CSCF shall allocate an IMS-AGW on ground and send the satellite identifier of the satellite serving the UE received together with an SDP offer to the terminating side to trigger media negotiation. The terminating P-CSCF shall return the 18X response together with the satellite identifier of the satellite serving the remote UE, if any, and the SDP answer. If the satellite identifier of the satellite serving the remote UE is not present, the originating P-CSCF shall not activate optimized media routing via IMS.

If the terminating P-CSCF indicates its activation of the optimized media routing via IMS, the P-CSCF re-selects an IMS-AGW on satellite and releases the IMS-AGW on ground. The P-CSCF shall send the modified SDP offer containing the IMS-AGW transport address to the terminating P-CSCF. Upon receiving the SDP offer, the terminating P-CSCF shall update the IMS-AGW on satellite for the terminating UE with the transport address received in the SDP offer.

The P-CSCF may include a satellite identifier in the P-Access-Network-Info header field within a "satellite-id" parameter in initial requests for a dialog and responses to initial requests for a dialog.

When the P-CSCF receives a DNAI associated with satellite identifier from the PCF for the served UE, it shall include satellite identifier derived from DNAI in SIP messages. When the terminating P-CSCF receives a SIP INVITE request, it shall include the terminating UE's satellite identifier in the P-Access-Network-Info header field of SIP 18X response and SIP 200 (OK) responses. When the originating P-CSCF receives a response containing a "satellite-id" parameter in the P-Access-Network-Info header field, the P-CSCF shall store this information to determine whether optimized media routing can be applied.

For media session modifications, the P-CSCF may include the current satellite identifier in the P-Access-Network-Info header field within the "satellite-id" parameter of any related SIP request or response.

* * * Next Change * * * *

Y.2.2.3	Handling of satellite change

When the P-CSCF receives an Access Network Information notification with a DNAI associated with the target satellite identifier from the PCF, the P-CSCF shall determine whether optimized media routing can continue.

If optimized media routing can continue, the P-CSCF shall select a new IMS-AGW on the target satellite and send the "satellite-id" parameter set to the target satellite identifier in the P-Access-Network-Info header field, with an SDP offer to the terminating side to trigger media negotiation.

If optimized media routing cannot continue, the P-CSCF shall select an IMS-AGW on the ground and activates ground fallback routing.

* * * End of Changes * * * *