3GPP TSG-CT WG1 Meeting #157	C1-256253

# Sophia Antipolis, France, 13-17 October 2025	(was C1-25xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.301 | CR | 4545 | rev | -- | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ESM: Handling IP failures indicated by upper layers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-10-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | When a new PDN session is established upper layers may detect IP layer failures like DNS failures (e.g., no DNS server address is provided during PDU session establishment) or IPv6 address formation failure which make the PDU session unusable as the IP connection is not functional. With the current ESM procedures the UE cannot inform the network about such failures. In consequence, such problems can only be detected indirectly by the network and may thus be hard to be resolved. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to introduce an IP Failure cause in the PCO IE, which the UE can include in the UE requested bearer resource allocation procedure or the UE requested PDN disconnection procedure. Using the PCO IE has the advantage that there is no impact on the AMF/SMF interface and the concept could be used in 4G and 5G. If included in the UE requested bearer resource allocation procedure, the network may try to resolve the problem, or if it cannot be resolved, the SMF can trigger the the PDU session release procedure and may include the Back-off timer value IE. If included in the UE requested PDN disconnection procedure, the network can use the indication for network internal error analysis. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE cannot inform the network about such failures. In consequence, such problems can only be detected indirectly by the network and may thus be hard to be resolved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.3.1, 6.5.3.2, 6.5.3.4.1, 6.5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 6.5.3	UE requested bearer resource allocation procedure

#### 6.5.3.1	General

The purpose of the UE requested bearer resource allocation procedure is for a UE to request an allocation of bearer resources for a traffic flow aggregate. The UE requests a specific QoS demand (QCI) and optionally sends a GBR requirement for a new traffic flow aggregate. If accepted by the network, this procedure invokes a dedicated EPS bearer context activation procedure (see clause 6.4.2) or an EPS bearer context modification procedure (see clause 6.4.3).

The UE may inform the network with the UE requested bearer resource allocation procedure about an IP failure reported by upper layers.

If there is a PDN connection for emergency bearer services established, the UE shall not request additional bearer resources for this PDN connection.

* * * Next Change * * * *

#### 6.5.3.2	UE requested bearer resource allocation procedure initiation

In order to request the allocation of bearer resources for one traffic flow aggregate, the UE shall send a BEARER RESOURCE ALLOCATION REQUEST message to the MME, start timer T3480 and enter the state PROCEDURE TRANSACTION PENDING (see example in figure 6.5.3.2.1).

The UE shall include the EPS bearer identity of the default EPS bearer associated with the requested bearer resource in the Linked EPS bearer identity IE. The UE shall set the TFT operation code in the Traffic flow aggregate IE to "Create new TFT". The packet filters in the Traffic flow aggregate IE shall include at least one packet filter applicable for the uplink direction. In the Required traffic flow QoS IE, the UE shall indicate a QCI and, if the UE also includes a GBR, the additional GBR required for the traffic flow aggregate.

Figure 6.5.3.2.1: UE requested bearer resource allocation procedure

For the NBIFOM procedures as defined in 3GPP TS 24.161 [36], the UE may send a BEARER RESOURCE ALLOCATION REQUEST message to the MME. If the traffic flow aggregate IE is not needed in those procedures, the UE shall set:

-	the length indicator of the Traffic flow aggregate IE to the value 1;

-	the TFT operation code to "000";

-	the E bit to zero; and

-	the number of packet filters to zero;

and in the Required traffic flow QoS IE:

-	the length indicator to the value 1; and

-	the QCI to zero.

If the UE is performing UE requested bearer resource allocation procedure due to one of the following IP failures indicated by the upper layer:

-	DNS failure; or

-	IPv6 address formation failure;

the UE shall include the Protocol configuration options or the Extended protocol configuration options IE in the BEARER RESOURCE ALLOCATION REQUEST message and shall include the corresponding IP failure cause.

* * * Next Change * * * *

#### 6.5.3.4	UE requested bearer resource allocation procedure not accepted by the network

##### 6.5.3.4.1	General

If the bearer resource allocation requested cannot be accepted by the network, the MME shall send a BEARER RESOURCE ALLOCATION REJECT message to the UE. The message shall contain the PTI and an ESM cause value indicating the reason for rejecting the UE requested bearer resource allocation.

The ESM cause value typically indicates one of the following:

#26:	insufficient resources;

#30:	request rejected by Serving GW or PDN GW;

#31:	request rejected, unspecified;

#32:	service option not supported;

#33:	requested service option not subscribed;

#34:	service option temporarily out of order;

#35:	PTI already in use;

#37:	EPS QoS not accepted;

#41:	semantic error in the TFT operation;

#42:	syntactical error in the TFT operation;

#43:	invalid EPS bearer identity;

#44:	semantic error(s) in packet filter(s);

#45:	syntactical error(s) in packet filter(s);

#56:	collision with network initiated request;

#59:	unsupported QCI value;

#60:	bearer handling not supported;

#65:	maximum number of EPS bearers reached; or

#95 – 111:	protocol errors.

If the bearer resource allocation requested is for an established LIPA PDN connection or SIPTO at the local network PDN connection, then the network shall reply with a BEARER RESOURCE ALLOCATION REJECT message with ESM cause #60 "bearer handling not supported".

If the requested new TFT is not available, then the BEARER RESOURCE ALLOCATION REJECT message shall be sent.

The TFT in the request message is checked by the network for different types of TFT IE errors as follows:

a)	Semantic errors in TFT operations:

1)	When the TFT operation is an operation other than "Create a new TFT".

The network shall reject the allocation request with ESM cause #41 "semantic error in the TFT operation".

b)	Syntactical errors in TFT operations:

1)	When the TFT operation = "Create a new TFT" and the packet filter list in the TFT IE is empty.

2)	When there are other types of syntactical errors in the coding of the TFT IE, such as a mismatch between the number of packet filters subfield, and the number of packet filters in the packet filter list.

The network shall reject the allocation request with ESM cause #42 "syntactical error in the TFT operation".

c)	Semantic errors in packet filters:

1)	When a packet filter consists of conflicting packet filter components which would render the packet filter ineffective, i.e. no IP packet will ever fit this packet filter. How the network determines a semantic error in a packet filter is outside the scope of the present document.

2)	When the resulting TFT does not contain any packet filter which applicable for the uplink direction.

The network shall reject the allocation request with ESM cause #44 "semantic errors in packet filter(s)".

d)	Syntactical errors in packet filters:

1)	When the TFT operation = "Create a new TFT" and two or more packet filters in the resultant TFT would have identical packet filter identifiers.

2)	When the TFT operation = "Create a new TFT" and two or more packet filters among all TFTs associated with the PDN connection would have identical packet filter precedence values.

3)	When there are other types of syntactical errors in the coding of packet filters, such as the use of a reserved value for a packet filter component identifier.

In case 2, if the old packet filters do not belong to the default EPS bearer context, the network shall not diagnose an error, shall further process the new request and, if it was processed successfully, shall delete the old packet filters which have identical filter precedence values. Furthermore, the network shall perform an EPS bearer context deactivation request procedure to deactivate the dedicated EPS bearer context(s) for which it has deleted the packet filters.

In case 2, if one or more old packet filters belong to the default EPS bearer context, the network shall release the relevant PDN connection using the EPS bearer context deactivation procedure. If it is the last remaining PDN connection and EMM-REGISTERED without PDN connection is not supported by the UE or the MME, the network shall detach the UE using detach type "re-attach required".

Otherwise the network shall reject the allocation request with ESM cause #45 "syntactical errors in packet filter(s)".

If the Protocol configuration options IE of the BEARER RESOURCE ALLOCATION REQUEST message indicates an IP failure and the network determines that the problem cannot be resolved, the network shall initiate the EPS bearer context deactivation procedure and may include the ESM Back-off timer.

The network may include a Back-off timer value IE in the BEARER RESOURCE ALLOCATION REJECT message.

If the Back-off timer value IE is included and the ESM cause value is different from #26 "insufficient resources" and #65 "maximum number of EPS bearers reached", the network may include the Re-attempt indicator IE to indicate:

-	whether the UE is allowed to attempt a secondary PDP context activation procedure in the PLMN for the same APN in A/Gb or Iu mode or a PDU session modification procedure in the PLMN for the same APN in N1 mode; and

-	whether another attempt in A/Gb and Iu mode, in S1 mode or in N1 mode is allowed in an equivalent PLMN.

Upon receipt of a BEARER RESOURCE ALLOCATION REJECT message, the UE shall stop the timer T3480, release the traffic flow aggregate description associated to the PTI value, and enter the state PROCEDURE TRANSACTION INACTIVE.

* * * Next Change * * * *

#### 6.5.2.2	UE requested PDN disconnection procedure initiation

In order to request PDN disconnection from a PDN, the UE shall send a PDN DISCONNECT REQUEST message to the MME, start the timer T3492 and enter the state PROCEDURE TRANSACTION PENDING (see example in figure 6.5.2.2.1). The PDN DISCONNECT REQUEST message shall include the EPS bearer identity of the default bearer associated with the PDN to disconnect from as the linked EPS bearer identity in the PDN DISCONNECT REQUEST message.

If the UE is requesting the PDN disconnection from a PDN due to one of the following IP failures indicated by upper layer:

-	DNS failure; or

-	IPv6 address formation failure

the UE shall include the Protocol configuration options or the Extended protocol configuration options IE and shall include the corresponding IP failure cause.

Figure 6.5.2.2.1: UE requested PDN disconnection procedure

* * * End of Changes * * * *