3GPP TSG-CT WG1 Meeting #157	C1-256347

# Sophia-Antipolis, France, 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 27.007 | CR | 0910 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to +CCMMD to identify media flows that carry PDU Sets |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | PDU Set is one or more PDUs carrying the payload of one unit of information generated at the application layer. For example, a PDU Set can be a video frame or slice that the application can decode. The +CCMMD AT command does not include information to identify media flows. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update +CCMMD to identify media flows as part of media profile. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be no way to identify media flows in +CCMMD. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

### 13.2.5	Control and modify media description +CCMMD

Table 13.2.5-1: +CCMMD parameter command syntax

| Command | Possible response(s) |
| --- | --- |
| +CCMMD=<ccidx>,<neg_status>,<midx>[,<SDP_md>,<IP_Type>,<local_address>,<remote_address>,<local_port>,<remote_port>] | +CME ERROR: <err> |
| +CCMMD? |  |
| +CCMMD=? | +CCMMD: (list of supported <ccidx>s),(list of supported <midx>s),(list of supported <IP_Type>s),(list of supported <local_address>s),(list of supported <remote_address>s),(list of supported <local_port>s),(list of supported <remote_port>s |

Description

This command allows control of the media used in a multimedia call. The command can be used to initiate modification of the media of an ongoing call, to accept, modify or reject incoming changes in media or to accept, modify or reject the media for an incoming call. Supported media types are typically audio, video and messaging (MSRP).

When <ccidx> matches the index of an ongoing call, the TA/MT will attempt to add or remove media to the call by triggering an SDP renegotiation over the SIP protocol.

When <neg_status>=1, the set command requests an unconditional change the media of the call to that described by <SDP_md>.

When <neg_status>=2, the set command proposes a change of media to that described by <SDP_md>, to which the remote party has to respond before the media of the call is changed. The response from the remote party will be indicated in a +CMCCSI unsolicited result code. If the remote party accepts the change of media, the <neg_status> value in +CMCCSI will be set to 3. If the remote party rejects the change of media, <neg_status> will be set to 4. In both cases the <SDP_md> value in +CMCCSI will describe the currently active media of the call (if any).

If the remote party unconditionally changes the media of the call, this will be indicated in a +CMCCSI unsolicited result code, with <neg_status>=1 and <SDP_md> containing the updated (and now active) media description.

If the remote party proposes to change the media of an ongoing call, this will be indicated in a +CMCCSI unsolicited result code, with <neg_status>=2. The set command is used to respond to the proposal, either by accepting it by setting <neg_status>=3, by rejecting it by setting <neg_status>=4, or indicate that a subset of the incoming proposal is accepted by setting the <neg_status>=3 along with the <SDP_md> containing the subset of the incoming proposal is acceptable.

When the MT receives an incoming call from a remote party, the +CMCCSI unsolicited result code will be sent to the TE with <neg_status>=2. This proposed media for the new call is either accepted, modified or rejected as described above.

The set command allows the MT to identify media flows in the media used in the multimedia call that includes PDU sets.

Defined values

<ccidx>: integer type. Call identification number as defined in the +CMCCS and +CLCCS commands.

<neg_status>: integer type

1	The <SDP_md> parameter describes the desired set of media for the call.

2	The <SDP_md> parameter describes a proposal for a new set of media for the call.

3	Accept the most recently received media proposal. The <SDP_md> parameter describes the accepted media for the call.

4	Reject the most recently received media proposal.

<SDP_md>: string type represented with IRA characters. Media description as per the +CDEFMP command. This parameter shall not be subject to conventional character conversion as per +CSCS.

<midx>: integer type; media index, supported range (0-15).

<IP_Type>: integer type; specifies the IP addressing used.

0	Indicates IPv4 addressing is used.

1	Indicates IPv6 addressing is used.

<local_address>: string type; The string is given as dot-separated numeric (0-255) parameters on the form:
"a1.a2.a3.a4.m1.m2.m3.m4" for IPv4 or
"a1.a2.a3.a4.a5.a6.a7.a8.a9.a10.a11.a12.a13.a14.a15.a16.m1.m2.m3.m4.m5.m6.m7.m8.m9.m10.m11.m12.m13.m14.m15.m16", for IPv6.

<remote_address>: string type; The string is given as dot-separated numeric (0-255) parameters on the form:
"a1.a2.a3.a4.m1.m2.m3.m4" for IPv4 or
"a1.a2.a3.a4.a5.a6.a7.a8.a9.a10.a11.a12.a13.a14.a15.a16.m1.m2.m3.m4.m5.m6.m7.m8.m9.m10.m11.m12.m13.m14.m15.m16", for IPv6.

<local_port>: integer type, with value range from 0 to 65535 which indicates a local UDP port.

<remote_port>: integer type, with value range from 0 to 65535 which indicates a remote UDP port.

Implementation

### Optional.

End of change