3GPP TSG-CT WG1 Meeting #157	C1-256357

Sophia Antipolis, France, 13th – 17th October 2025

Source:	InterDigital

Title:	Updating the ML model retrieval service

Spec:	3GPP TS 24.560 v1.0.0

Agenda item:	19.41

Document for:	Agreement

1. Introduction

<Introduction part (optional)>

2. Reason for Change

The ML model retrieval service was defined in stag-3 based on stage-2 TS 23.482. This pCR adds the missing details related to the data structures for the ML model retrieval services.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.560 v1.0.0.

* * * First Change * * * *

#### 5.14.2.2	Aimles_MLModelRetrieval_Request

##### 5.14.2.2.1	General

This service operation is used by AIMLE client to request for one-time ML model retrieval from the AIMLE server.

##### 5.14.2.2.2	AIML operation for model retrieval

To request one-time ML model retrieval, the AIMLE client shall send an HTTP POST request to AIMLE server as specified in clause 6.13.4. The body of the POST message shall include the MLMdlRetReq data structure as specified in clause 6.13.6.2.5.

Upon receipt of the HTTP POST request from AIMLE client:

a)	the AIMLE server shall verify the identity of the AIMLE client and determine if the AIMLE client is authorized for the request;

b)	if the AIMLE client:

1)	is not authorized to request ML model retrieval, the AIMLE server shall respond to the AIMLE client with an appropriate error status code; or

2)	is authorized to request ML model retrieval then the AIMLE server shall determine if the required ML models are available or not. For the registered AIMLE clients the AIMLE server uses the registered information and for the AIMLE clients that are not registered, the AIMLE server uses the ML model retrieval filters. If the required  ML models are:

i)	available, then in the response the AIMLE server includes an indication that the retrieval was successful and includes the ML models; and

ii)	not available, then in the response the AIMLE server includes an indication that the retrieval failed and includes appropriate error code. The AIMLE server may also perform the ML model information discovery procedure with the ML using clause 5.3.2 of 3GPP TS 29.482.

#### 5.14.2.3	Aimles_MLModelRetrieval_Subscribe

##### 5.14.2.3.1	General

This service operation is used by AIMLE client to subscribe to notifications from the AIMLE server about ML models retrieval.

##### 5.14.2.3.2	AIML operation for model retrieval subscription

To subscribe with the AIML server to be notified of ML model retrieval, the AIMLE client shall send an HTTP POST request to AIMLE server as specified in clause 6.13.3. The body of the POST message shall include the MLMdlRetSub data structure as specified in clause 6.13.6.2.2.

Upon receipt of the HTTP POST request from AIMLE client:

a)	the AIMLE server shall verify the identity of the AIMLE client and determine if the AIMLE client is authorized for the request;

b)	if the AIMLE client:

1)	is not authorized for ML model retrieval subscription, the AIMLE server shall respond to the AIMLE client with an appropriate error status code; or

2)	is authorized to request ML model retrieval then the AIMLE server may create the subscription and store the subscription information. If the required AIML server:

i)	has created the subscription, then in the response the AIMLE server shall include an HTTP "201 Created" status code with the response body including an indication of success, the subscription identity and may include an expiration time to maintain the subscription; or

ii)	has not created the subscription, then in the response the AIMLE server shall include an indication of failure that the subscription is not created and shall include an appropriate error response as specified in clause 6.13.7.

If the AIMLE client determined the received HTTP POST request needs to be redirected, the AIMLE client may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative AIMLE client towards which the HTTP POST request should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5].

#### 5.14.2.4	Aimles_MLModelRetrieval_Notify

##### 5.14.2.4.1	General

This service operation is used by AIMLE server to inform the AIMLE client about the availability of ML model.

##### 5.14.2.4.2	AIML operation for model retrieval notification

To indicate availability of ML models to a subscriber, i.e, AIMLE client, the AIMLE server shall send an HTTP POST request to AIMLE client as specified in clause 6.13.5. The body of the POST message shall include the MLMdlRetNotif data structure as specified in clause 6.13.6.2.4.

Upon receipt of the HTTP POST request:

a)	the AIMLE client shall verify the identity of the AIMLE server and determine if the AIMLE server is authorized to indicate the newly available ML models; and

b)	if the AIMLE server:

1)	is not authorized, the AIMLE client shall respond to the AIMLE server with an appropriate error status code; or

2)	is authorized, the AIMLE client shall respond to the AIMLE server with:

i)	if the HTTP POST request is handled successfully, an HTTP "204 No Content" status code; and

ii)	if the HTTP POST request is not handled successfully, an appropriate error response as specified in clause 6.13.7.

#### 5.14.2.5	Aimles_MLModelRetrieval_UpdateSubscription

##### 5.14.2.5.1	General

This service operation is used by AIMLE client to update subscription with the AIMLE server for ML model retrieval.

##### 5.14.2.5.2	AIML operation for model retrieval subscription update

To update subscription with the AIML server for ML model retrieval, the AIMLE client shall send an HTTP PATCH request (for partial update) as specified in clause 6.13.3.3.3.3 or HTTP PUT request (for full replacement) to AIMLE server as specified in clause 6.13.3.3.3.1. The body of the PATCH message shall include the MLMdlRetSubPatch data structure as specified in clause 6.13.6.2.3 or PUT message shal include the MLMdlRetSub data structure as specified in clause 6.13.6.2.2.

Upon receipt of the HTTP PATCH or PUT request from AIMLE client:

a)	the AIMLE server shall verify the identity of the AIMLE client and determine if the AIMLE client is authorized for the request;

b)	if the AIMLE client:

1)	is not authorized for ML model retrieval subscription update, the AIMLE server shall respond to the AIMLE client with an appropriate error status code; or

2)	is authorized to request ML model retrieval then the AIMLE server may update the subscription information. If the AIML server:

i)	has updated the subscription, then in the response the AIMLE server shall include an indication of success, and may include an expiration time;

ii)	response may include either:

-	an HTTP "204 No Content" status code; or

-	an HTTP "200 OK" status code with the response body including a representation of the updated resource within the MLMdlRetSub data structure as specified in clause 6.13.6.2.2; or

iii)	has not updated the subscription, then in the response the AIMLE server shall include an indication of failure and may include an appropriate error response as specified in clause 6.13.7.

If the AIMLE client determined the received HTTP POST request needs to be redirected, the AIMLE client may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative AIMLE client towards which the HTTP POST request should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5].

#### 5.14.2.6	Aimles_MLModelRetrieval_Unsubscribe

##### 5.14.2.6.1	General

This service operation is used by AIMLE client to unsubscribe with the AIMLE server for ML model retrieval.

##### 5.14.2.6.2	AIML operation to unsubscribe for model retrieval

To unsubscribe with the AIML server for ML model retrieval, the AIMLE client shall send an HTTP DELETE request to AIMLE server as specified in clause 6.13.3.3.3.2.

Upon receipt of the HTTP DELETE request from AIMLE client:

a)	the AIMLE server shall verify the identity of the AIMLE client and determine if the AIMLE client is authorized for the request;

b)	if the AIMLE client:

1)	is not authorized to unsubscribe for ML model retrieval, the AIMLE server shall respond to the AIMLE client with an appropriate error status code; or

2)	is authorized to request ML model retrieval then the AIMLE server may cancel the subscription. If the AIML server:

i)	has cancelled the subscription, then in the response the AIMLE server shall include an indication of success; or

ii)	has not cancelled the subscription, then in the response the AIMLE server shall include an indication of failure and may include an appropriate error response as specified in clause 6.13.7.

If the AIMLE client determined the received HTTP POST request needs to be redirected, the AIMLE client may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative AIMLE client towards which the HTTP POST request should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5].

* * * End of Change * * * *