3GPP TSG-CT WG1 Meeting #157	C1- 256364

Sophia Antipolis, France, 13-17 October 2025

Source:	LG Electronic

Title:	Pseudo-CR on removal of ENs in Inventory procedure initiation

Spec:	3GPP TS 24.369 v1.0.0

Agenda item:	19.70

Document for:	Decision

1. Introduction

This contribution is to resolve two Editor’s Notes in 5.2.2 Inventory procedure initiation.

2. Reason for Change

The proposed changes are:

- to simply remove the following EN as no update is needed:

Editor's note:	The reference to TS 38.391 may be updated according to RAN2 progress.

- to update the message name (“Request AIoT paging” to “INVNETORY REQUEST”) in Figure 5.2.2-1 according to TS 38.413

- to add a note to clarify the message name “AIoT paging” in Figure 5.2.2-1

In TS 38.319, the AIoT paging message is defined as “A-IoT Paging”. In RAN specifications, TS 38.319 and TS 39.413, “A-IoT” is used, not “AIoT” as used in NAS specifications. The new note is to keep the current NAS term “AIoT paging” used in TS 24.369 and clarify the message naming for reference.

- to remove the following EN:

Editor's note:	Figure 5.2.2-1 may further align with RAN2/RAN3 according to RAN2/RAN3 progress.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to update the message name during the inventory procedure and remove Editor’s Notes in 5.2.2.

*******

* * * First Change * * * *

5.2.2	Inventory procedure initiation

To initiate the inventory procedure, the AIOTF sends an inventory request to the selected NG-RAN, as defined in 3GPP TS 38.413 [10]. The selected NG-RAN performs the AIoT paging by broadcast of the paging message that includes the AIoT identification information received in the inventory request (see 3GPP TS 38.300 [7] and 3GPP TS 38.391 [8]).

NOTE 1:	How AIOTF selects NG-RAN is defined in 3GPP TS 23.369 [2].

Figure 5.2.2-1 gives an example of inventory procedure for a AIoT device which sends the INVENTORY REPORT message to AIOTF.

Figure 5.2.2-1: Inventory procedure

NOTE 2:	The AIoT paging message in Figure 5.2.2-1 is defined as the A-IoT Paging message in TS 38.391 [8].

Upon receiving a paging message, the lower layers of the AIoT device determines whether to respond to the paging message as specified in 3GPP TS 38.391 [8]. The evaluation in lower layers includes interaction with upper layers to assess AIoT identification information, if included in the paging message, as specified in clause 5.2. If lower layers determine to respond to the paging message, an indication is provided to upper layers.

* * * End of Changes * * * *