3GPP TSG-CT WG1 Meeting #157	C1-256379

Sophia Antipolis, France, 13-17 October 2025

Source:	Samsung

Title:	Pseudo-CR on AIoT Update in the inclusion of AIoT identifier in Inventory report message

Spec:	3GPP TS 24.369 v1.0.0

Agenda item:	19.70

Document for:	Decision

1. Introduction

The AIoT permanent device identifier should not be mandatory in the inventory response as per SA3 agreement.

2. Reason for Change

SA3 discussed and agreed that when the AIOTF uses privacy protection of AIoT device identifier by using either the filtering information or temporary identifier in the inventory procedure, the D2R message should not have the device identifier. The background behind this being, the RES_AIOT calculated by the device, considers the AIoT device identifier as an input, and for these 2 cases, the device identifier need not be sent as a mandatory information.

As in 33.369,

Sections 5.4.2 and 5.4.3:

“In step 5 and 6, a device identification information is not included in the D2R message and Inventory Report message.”

Section 5.4.1:

“When privacy protection is not used during the inventory procedure, the AIoT device includes its AIoT device permanent identifier as a device identification information in the procedure specified in clause 5.2.2.”

3. Conclusions

Current procedure section for inventory procedure and the encoding scheme in CT1 considers the identifier as a mandatory information, which needs to be changed.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369 v1.0.0.

* * * First Change * * * *

### 5.2.3	Inventory procedure completion

The AIoT device determines to respond to the AIoT paging:

a)	based on the indication from lower layer as specified in 3GPP TS 38.391 [8]; or

b)	based on whether the AIoT identification information received from the lower layer is matched or not.

If the AIoT device determines to respond to the AIoT paging, the AIoT device shall send an INVENTORY REPORT message to the AIOTF.

The AIoT device shall derive a RANDAIOT_d value as specified in 3GPP TS 33.369 [6] and shall include the derived RANDAIOT_d  value in the in the RANDAIOT_d IE in the INVENTORY REPORT message.

The AIoT device shall derive a RESAIOT value, using the long-term configured KAIoT and the value of the RANDAIOT_n included in the paging message provided by lower layers as specified in 3GPP TS 33.369 [6]. The derived RESAIOT value shall be included in the in the RESAIOT IE in the INVENTORY REPORT message.

If the AIoT device identifier was included in the AIoT identification information in the AIoT paging message, and the AIoT device decides to respond to the paging request, the AIoT device shall include the AIoT device identifier in the AIoT device identity IE in the INVENTORY REPORT message.

Editor's note:	Details of the AIoT device identity IE is FFS and depends on SA3 progress.

The AIoT device determines whether the AIoT identification information is matched or not as follows:

Editor's note:	It is FFS how to determine the component of AIoT identification information for AIoT device, and this may need coordination with RAN2/3.

a)	if the AIoT identification information includes an AIoT device identifier:

Editor's note:	Details of AIoT device identifier should be aligned with security consideration from SA3. How to match the AIoT device identifier depends on the AIoT device identifier included.

b)	if the AIoT identification information includes filtering information (see clause 31.3 of 3GPP TS 23.003 [5]):

1)	based on ID type field included in the filtering information, the AIoT device determines the AIoT identification information is matched if:

A)	the PLMN ID field is not included in the filtering information, or the PLMN ID field is included in the filtering information and:

i)	the PLMN ID field is included in the AIoT device permanent identifier; and

ii)	the PLMN ID field in the AIoT device permanent identifier is the same as the PLMN ID field within the filtering information;

B)	the NID field is not included in the filtering information, or the NID field is included in the filtering information and:

i)	the NID field is included in the AIoT device permanent identifier; and

ii)	the NID field in the AIoT device permanent identifier is the same as the NID field within the filtering information;

C)	the third party ID field is not included in the filtering information, or the third party ID field is included in the filtering information and:

i)	the third party ID field is included in the AIoT device permanent identifier; and

ii)	the third party ID field in the AIoT device permanent identifier is the same as the third party ID field within the filtering information; and

D)	the identification information filters field is not included in the filtering information, or the identification information filters field is included in the filtering information and for each identification information filter, the component value within the identification information filter is the same as the corresponding bitstring from the AIoT device permanent identifier based on offset field and length field; or

2)	otherwise, the AIoT device determines the AIoT identification information is not matched.

NOTE 1:	The AIoT identification information corresponds to paging ID, as defined in 3GPP TS 38.391 [8]).

Editor's note:	The reference to filtering information (e.g. IE name) will be updated according to CT4 and RAN2 progress.

Based on whether the AIoT identification information is matched:

a)	if the AIoT identification information is matched, the AIoT device shall indicate to the lower layer that the AIoT identification information is matched, and determine to respond to the AIoT paging; or

b)	otherwise, the AIoT device shall ignore the AIoT identification information, indicate to the lower layer that the filtering information is not matched, and determine not to respond to the AIoT paging.

Upon all the selected NG-RAN indicating the inventory procedure has been completed, the inventory procedure is considered as completed by the AIOTF as specified in 3GPP TS 23.369 [2].

NOTE:	How NG-RAN indicates the inventory procedure has been completed is defined in 3GPP TS 38.300 [7].

Editor's note:	The reference to TS 38.300 may be updated according to RAN2 progress.

Editor's note:	AIoT .device authentication of the network is FFS.

* * * Next Change * * * *

### 7.1.2	Inventory report

##### 7.1.2.1	Message definition

The INVENTORY REPORT message is sent by the AIoT device to the AIOTF to report the AIoT device identity.

See table 7.1.2.1-1.

Message type:	INVENTORY REPORT

Significance:	dual

Direction:	AIoT device to AIOTF

Table 7.1.2.1-1: INVENTORY REPORT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Message type | Message type 7.2.2 | M | V | 1 |
|  | Security parameters | Security parameters 7.2.3 | M | V | n |
|  | RANDAIOT_d | RAND 7.2.10 | M | V | TBD |
|  | RESAIOT | Authentication response parameter 7.2.7 | M | V | TBD |
|  | AIoT device identity | AIoT device identity 7.2.6 | O | V | n |

* * * End of Changes * * * *