3GPP TSG-CT WG1 Meeting #157	C1-256478

# Sophia Antipolis, France 13 – 17 October 2025

Source:	Apple

Title:	Design Considerations for Modular NAS

Agenda item:	20.

Document for:	Discussion

# 1 Introduction

CT WGs are starting discussion on 6G SIDs and this includes discussion on design of 6G NAS. Several companies are in the process of presenting their views (offline and online) on this topic. One topic which has emerged in several such discussions is the concept of Modular NAS.

This contribution analyses the key concept behind modularized NAS and lists some aspects to consider with this concept as way forward for 6G.

# 2 Discussion

# 2.1 Current design of 5G NAS

# The current 5G NAS protocol is based on the framework specified in TS 24.007. The protocol, despite its limitations, has worked quite well over last several years for multiple 3GPP releases, and the framework has done so for more than 30 years and 4 generations of mobile communication systems. The NAS specifications describe the main features of NAS and their implementation based on procedures as building blocks of the protocol, and it defines the related messages exchanged between the UE and the network. The UE and network negotiate the capability on a feature-by-feature basis, if necessary, and as such there is no version control on a release basis.

# 2.2 Key functionality of modularized NAS

At a very high level, the key functionality of modularized NAS is to split the NAS functionality into multiple modules wherein each module represents a feature. Examples of different modules could include UE Capabilities, PLMN Selection, IP Connectivity, Security, Network Slicing, QoS, etc. NAS messages are specified to update parameters within each such module and each module has its own version to indicate supported features. Thus, the modularization is not only affecting the specification, but also the implementation; or rather one could say that a specific sub-division into software modules is the basis for the design of both the specification and the implementation.

# 2.3 Design aspects to consider with the modularized NAS approach

There are few things to consider with the modularized NAS approach.

Intrudes on implementation:

To be useful in practice, the concept of modularized NAS requires specifying details of every module in 3GPP specification, making the specification very complex and rigid. This high (and verbose) level of detail mirrors the internal software architecture and implementation details of NAS, and such a level of detail should not be forced by 3GPP specifications. 3GPP should define the external NAS interface towards the network and its operational behavior, while leaving internal module organization to specific implementation.

Slows down operation:

By splitting NAS into a larger number of modules, the number of message exchanges (round trips) between UE and the network increases significantly during different 3GPP procedures. For example, the registration procedure could involve several additional message exchanges (in total there could be more than 5, maybe even 10 round trips) between UE and network and this will make things slower than what they are today. This also increases the possible points of failure within individual modules (RRC, RLFs), making error handling complex (more code, more communication between modules) and overall operation slower. As a result, the user experience could be that 6G is slower than 5G and 4G.

Impacts to state machine:

State machine handling for supporting these modularized functions needs to be adapted for handling these cases at UE and network and is likely to get more complex. Any new message introduced for the modular NAS function will need changes to state machine.

Module Versioning not very helpful:

Doing versioning across 3GPP releases and then having version negotiation for each feature or module within a release is not very helpful.

E.g., in Rel-18, there were 5 new optional network slicing related features introduced: network slice replacement feature (aka "alternative S-NSSAI"), network slice location validity ("AoS-NS"), network slice time validity, partially allowed/rejected network slices, and network slice usage policy control. So, in total we can have 32 different combinations of supported network slicing (sub-)features.

- If each of these features is represented by a separate module in mobility management, each with own NAS messages, this will result in excessive signaling and strong dependencies between the modules. E.g., to determine whether a PDU session can be established for a certain (DNN, network slice) combination, it is not sufficient that the "basic" network slicing module checks the Allowed NSSAI, the UE will potentially also need to check the Partially allowed NSSAI, and for the slices which have a certain "area of service" (AoS-NS) whether the UE is within or out of service area. If the Partially allowed NSSAI and the S-NSSAIs of slices subject to location validity are each provided by the network with a separate NAS message, this introduces an additional delay and it requires some coordination ("end-marker") between the NAS messages which are initiated by different modules on the network side such that the UE knows when it has received all the relevant information from the network and can start with the evaluation.

- On the other hand, if the features are implemented as part of the "basic" network slicing module, then we will need a negotiation of the support of individual Rel-18 features on top of the "Rel-18" version negotiation, and we are basically back to what we have today.

- In network slicing there is also a strong dependency between modules belonging today to the mobility management and the session management sublayer. E.g., the IP connectivity module needs to know whether a certain S-NSSAI is included in the Allowed NSSAI or Partially allowed NSSAI or NSSAI with location/time validity information.

Versioning will not solve problems of erroneous implementations (either for network or UE), and both vendor information and software version information would be required for maintainability. Further, for most features there are network capabilities already defined. So overall this adds lot more complexity.  The use case wherein the network does not support a new feature which UE supports even when the network and UE support the same version (3GPP release) still needs to be handled.

Longer time for specification development:

The more details and content are to be specified, the longer the debate in 3GPP and the longer it takes to develop 3GPP specifications. With modularized NAS the level of detail increases significantly, and this will lead to further discussions and potentially take much longer to develop 3GPP specifications.

Verticals:

It has been mentioned that the current NAS design in 5GS is a high entry bar for verticals. The size of the NAS protocol and the considerations about the impact on the legacy makes it challenging to design verticals-driven features efficiently and according to their needs. This is not very clear and needs to be better articulated with some examples.

# 2.3 Topics to investigate as part of 6G NAS design

There are some topics which can be investigated as part of 6G NAS design. But in our view, the proposed modularization is rather a specific solution instead of an analysis of the potential shortcomings of 5G NAS. CT1 should focus on identifying these shortcomings before taking the next step.

Lean, Mean Specifications, Unused features:

One principal problem is that there are too many features added which do not have market traction. It would be judicious to analyze which aspects could be decoupled from different NAS procedures, such as the registration procedure. This could be covered with the current protocol design, i.e. this does not require modularized NAS. 3GPP could drop obsolete features, clean-up and retain features that are interesting, and market driven. Unused and unviable features impact reusability of 5G components in 6G (efficiency, cost, time to market, etc.), significantly impacting end-user benefits.

Better readability of long procedures:

Some of the current NAS procedures span more than 40 pages of specification text, making things very difficult to locate and impacting overall readability. A better re-organization of these procedures would definitely help.

Making dependencies more transparent:

An additional issue is that currently in the NAS specification it is not explicitly visible which feature was added in which release, and possibly on which older feature it depends. Due to the long procedure descriptions, it is also cumbersome to determine which features are optional and which are mandatory. A better, more transparent documentation of this would be desirable.

Message encoding and error handling:

There are limitations in current NAS protocol design related to message coding (Type 6 IEs, Type 1 IEs), IE coding, extensibility (reserve vs spare values), error handling, etc. The NAS protocol in 6G should resolve these limitations.

# 3 Conclusion

The key concept behind the modularized NAS approach is to split the NAS functionality into multiple modules wherein each module represents a feature and has its own name, version, procedure, messages, parameters and dependencies. Such an approach intrudes on implementation, slows down operation, impacts state machine, increases the time for specification development and has several other aspects that need to be considered carefully. 3GPP needs to carefully evaluate the pros and cons of this concept before embarking on such an approach for 6G NAS specification development.

Further, the proposed modularization is rather a specific solution instead of an analysis of the potential shortcomings of 5G NAS, and CT1 should focus on identifying these shortcomings before taking the next step.