3GPP TSG-CT WG1 Meeting #159	C1-260052

# Goa, India, 9th – 13th February 2026

| CR-Form-v12.5.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.369 | CR | 0006 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the indication from lower layers when handling of paging request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-CT |  |  |  |  |  | Date: |  |  | 2026-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | "If lower layers determine to respond to the paging message, an indication is provided to upper layers." in clause 5.2.2 is unclear since the statement is lack of explanation that this operation only applies to the case when paging request received in lower layers does not include AIoT identification information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add related condition description "If the AIoT identification information is not included in the AIoT paging message" to the statement mentioned above. Backward compatibility analysis: The change is backward compatible since it just clarifies the related condition description. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear statement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2, 5.4.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

### 5.2.2	Inventory procedure initiation

To initiate the inventory procedure, the AIOTF sends an inventory request to the selected NG-RAN, as defined in 3GPP TS 38.413 [10]. The selected NG-RAN performs the AIoT paging by broadcast of the AIoT paging message that includes the AIoT identification information received in the inventory request (see 3GPP TS 38.300 [7] and 3GPP TS 38.391 [8]).

NOTE:	How AIOTF selects NG-RAN is defined in 3GPP TS 23.369 [2].

Figure 5.2.2-1 gives an example of inventory procedure for an AIoT device which sends the INVENTORY REPORT message to AIOTF.

Figure 5.2.2-1: Inventory procedure

Upon receiving an AIoT paging message, the lower layers of the AIoT device determine whether to respond to the AIoT paging message as specified in 3GPP TS 38.391 [8]. The evaluation in lower layers includes interaction with upper layers to assess AIoT identification information as specified in clause 5.2.3, if the AIoT identification information is included in the AIoT paging message. If the AIoT identification information is not included in the AIoT paging message and lower layers determine to respond to the paging message, an indication is provided to upper layers. In addition to an indication to respond to the AIoT paging message or the AIoT identification information, the lower layers provide the RANDAIOT_n value included in the AIoT paging request message to AIoT NAS layer.

==============Next change==============

#### 5.4.1.1	General

The purpose of the sending of the STATUS message is to report certain error conditions detected in a received AIoT NAS message by the AIoT device (see example in figure 5.4.1.1.1). The STATUS message shall be integrity protected. The same ciphering shall apply to the STATUS message as the ciphering in the received AIoT NAS message that triggers the STATUS message.

AIoT device

AIOTF

STATUS

Figure 5.4.1.1.1: Status procedure

==============End of change==============