3GPP TSG-CT WG1 Meeting #159	C1-260053

# Goa, India, 9th – 13th February 2026

| CR-Form-v12.5.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.369 | CR | 0007 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | The length of AIoT device T-ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-CT |  |  |  |  |  | Date: |  |  | 2026-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | The length of the AIoT device T-ID IE is "TBD". The T‑ID is derived via the KDF procedure defined in Annex B.1 of TS 33.369. The KDF-based derivations (HMAC-SHA-256) produce 256-bit length outputs according to the definition of the KDF in TS 33.220. Therefore, the T‑ID value is a fixed length field of 256 bits (32 octets). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define the length of the AIoT device T-ID IE as 33 octets. Backward compatibility analysis: The change is backward compatible since the length of the IE is not defined. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The definition of AIoT device T-ID IE is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.1, 7.1.6.1, 7.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

#### 7.1.3.1	Message definition

The READ COMMAND message is sent by the AIOTF to the AIoT device to read the AIoT data from the memory.

See table 7.1.3.1-1.

Message type:	READ COMMAND

Significance:	dual

Direction:	AIOTF to AIoT device

Table 7.1.3.1-1: READ COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Security header | Security header 7.2.3 | M | V | 1 |
|  | Message authentication code | Message authentication code 7.2.10 | M | V | 4 |
|  | Read command message identity | Message type 7.2.2 | M | V | 1 |
|  | Read offset | Offset 7.2.11 | M | V | 2 |
|  | Read length | AIoT data length 7.2.5 | M | V | 1 |
| 10 | AIoT device T-ID | AIoT device T-ID 7.2.7 | O | TV | 33 |

==============Next change==============

#### 7.1.6.1	Message definition

The WRITE COMMAND message is sent by the AIOTF to the AIoT device to write the indicated AIoT data to the memory.

See table 7.1.6.1-1.

Message type:	WRITE COMMAND

Significance:	dual

Direction:	AIOTF to AIoT device

Table 7.1.6.1-1: WRITE COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Security header | Security header 7.2.3 | M | V | 1 |
|  | Message authentication code | Message authentication code 7.2.10 | M | V | 4 |
|  | Write command message identity | Message type 7.2.2 | M | V | 1 |
|  | Write offset | Offset 7.2.11 | M | V | 2 |
|  | AIoT data | AIoT data 7.2.4 | M | TBD | TBD |
| 10 | AIoT device T-ID | AIoT device T-ID 7.2.7 | O | TV | 33 |

==============Next change==============

### 7.2.7	AIoT device T-ID

The purpose of the AIoT device T-ID information element is to provide the AIoT device T-ID to the AIoT device.

The AIoT device T-ID information element is coded as shown in figure 7.2.7-1 and table 7.2.7-1.

The AIoT device T-ID informantion element is a type 3 information element with length of 33 octet.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AIoT device T-ID IEI |  |  |  |  |  |  |  | octet 1 |
| AIoT device T-ID |  |  |  |  |  |  |  | octet 2 octet 33 |

Figure 7.2.7-1: AIoT device T-ID information element

Table 7.2.7-1: AIoT device T-ID

AIoT device T-ID (octets 2 to 33)

This value of field is encoded as the generated T-ID as specified in 3GPP TS 33.369 [6].

==============End of change==============