3GPP TSG-CT WG1 Meeting #159	C1-260113

# Goa, India, 9th – 13th February 2026

| CR-Form-v12.5.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.548 | CR | 0106 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolution of editor’s note under clause 7.5.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_App |  |  |  |  |  | Date: |  |  | 2026-01-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | TS 24.548 contains an editor’s note under clause 7.5.7, quote: Editor's note (5GSAT_Ph3_App, CR0097): The details of the <satellite-id> element and <max-store-quota> element are FFS. Additionally, the specification provides the encoding of the <satellite-id> in the XML schema (under clause 7.4.4) as a "string", quote: <xs:element name="satellite-id" type="xs:string"/> However, we need to consider how the SatelliteId is specified by the core specification which is TS 36.331. Quote of clause 6.3.6: #### SatelliteId The IE SatelliteId is used to identify the satellite assistance information of the serving satellite, or neighbour satellites for E-UTRA and/or NR. SatelliteId information element -- ASN1START SatelliteId-r18 ::= INTEGER (0..255) -- ASN1STOP Considering TS 36.331 we can see that declaring "satellite-id" as of type "string" does not guarantee the semantics required by TS 36.331 (i.e., binary coded integer value from 0 to 255). Note that "string" contains any text and it does not express numeric range, i.e., binary encoding as indicated by TS 36.331. It is therefore proposed to use "unsignedByte" (XSD type) which enforces integer semantics, the 0 to 255 range and this is also of one octet size. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Resolution of editor’s note under clause 7.5.7 and update of related XML schema by defining the encoding of the <satellite-id> as of type "unsignedByte" to enforce bynary coded integer semantics and the 0 to 255 range. Backward compatibility analysis: The CR is not backwards compatible as the type of the "satellite-id" is changed. However, this is needed in order to comply with core specification TS 36.331 so that application satellite coverage information provisioning could work properly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Editor’s ntoe remains in frozen release of the specification. Furthermore, current XML definition of "satellite-id" (i.e., type "string") is not aligned with the core specification TS 36.331 so it does not guarantee the semantic binary coded integer value from 0 to 255 required. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7.4.8, 7.5.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[3]	3GPP TS 23.003: "Numbering, addressing and identification".

[4]	3GPP TS 23.203: "Policy and charging control architecture".

[5]	3GPP TS 24.008: "Mobile Radio Interface Layer 3 specification; Core Network Protocols; Stage 3".

[6]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[7]	3GPP TS 24.486: "Vehicle-to-Everything (V2X) Application Enabler (VAE) layer; Protocol aspects; Stage 3".

[8]	3GPP TS 24.545: "Location Management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol specification".

[9]	3GPP TS 24.547: "Identity management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol specification".

[10]	3GPP TS 26.346: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[11]	3GPP TS 29.061: "Interworking between the Public Land Mobile Network (PLMN) supporting packet based services and Packet Data Networks (PDN)".

[12]	3GPP TS 29.214: "Policy and Charging Control over Rx reference point".

[13]	3GPP TS 29.468: "Group Communication System Enablers for LTE (GCSE_LTE); MB2 reference point; Stage 3".

[14]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[15]	Void.

[16]	IETF RFC 3095: "RObust Header Compression (ROHC): Framework and four profiles: RTP, UDP, ESP, and uncompressed".

[17]	IETF RFC 3428: "Session Initiation Protocol (SIP) Extension for Instant Messaging".

[18]	IETF RFC 3841: "Caller Preferences for the Session Initiation Protocol (SIP)".

[19]	IETF RFC 4825: "The Extensible Markup Language (XML) Configuration Access Protocol (XCAP)".

[20]	IETF RFC 5795: "The Robust Header Compression (ROHC) Framework".

[21]	Void

[21A]	IETF RFC 6838: "Media Type Specifications and Registration Procedures".

[22]	IETF RFC 9110: "HTTP Semantics".

[23]	IETF RFC 7252: "The Constrained Application Protocol (CoAP)".

[24]	IETF RFC 7959: "Block-Wise Transfers in the Constrained Application Protocol (CoAP)".

[25]	IETF RFC 7641: "Observing Resources in the Constrained Application Protocol (CoAP)".

[26]	IETF RFC 8323: "CoAP (Constrained Application Protocol) over TCP, TLS, and WebSockets".

[27]	IETF RFC 8949: "Concise Binary Object Representation (CBOR)".

[28]	Void

[29]	IETF RFC 9177: "Constrained Application Protocol (CoAP) Block- Wise Transfer Options Supporting Robust Transmission".

[30]	IETF RFC 8610: "Concise Data Definition Language (CDDL): A Notational Convention to Express Concise Binary Object Representation (CBOR) and JSON Data Structures".

[31]	3GPP TS 24.546: "Configuration management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol specification".

[32]	OMA OMA-TS-XDM_Core-V2_1-20120403-A: "XML Document Management (XDM) Specification".

[33]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[34]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[35]	3GPP TS 26.517: "5G Multicast-Broadcast User Services; Protocols and Formats".

[36]	Open API: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[r36331]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC) protocol specification".

==============Next change==============

### 7.4.8	XML schema for application/vnd.3gpp.seal-store-forward-events-info+xml

Editor's note (5GSAT_Ph3_App, CR0097): 	The format of the  <max-store-quota> element is FFS.

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="urn:3gpp:ns:seal-store-forwardeventsInfo:1.0"

xmlns:sealSFevents="urn:3gpp:ns:seal-store-forwardeventsInfo:1.0"

elementFormDefault="qualified"

attributeFormDefault="unqualified"

xmlns:xenc="http://www.w3.org/2001/04/xmlenc#">

<!-- the root element -->

<xs:element name="seal-store-forward-events-info" id="SFevents">

<xs:complexType>

<xs:choice maxOccurs="unbounded">

<xs:element name="store-forward-subscription" type="sealSFevents:SFSubscriptionType"/>

<xs:element name="store-forward-notification" type="sealSFevents:SFNotificationType"/>

<xs:element name="store-forward-report" type="sealSFevents:SFReportType"/>

<xs:element name="anyExt" type="sealSFevents:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:choice>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

</xs:element>

<xs:complexType name="SFSubscriptionType">

<xs:sequence>

<xs:element name="subscription-id" type="xs:string"/>

<xs:element name="store-forward-events" type="sealSFevents:SFEventsType"/>

<xs:element name="store-forward-trigger" type="xs:string"/>

<xs:element name="anyExt" type="sealSFevents:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="SFEventsType">

<xs:sequence>

<xs:element name="ue-store-forward-mode" type="xs:string"/>

<xs:element name="est-time" type="xs:positiveInteger"/>

<xs:element name="feedlnk-period" type="xs:positiveInteger"/>

<xs:element name="anyExt" type="sealSFevents:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="SFNotificationType">

<xs:sequence>

<xs:element name="store-forward-events" type="sealSFevents:SFEventsType"/>

<xs:element name="store-forward-trigger" type="xs:string"/>

<xs:element name="time-stamp" type="xs:string"/>

<xs:element name="anyExt" type="sealSFevents:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="SFReportType">

<xs:sequence>

<xs:element name="UE-type" type="xs:string" minOccurs="0"/>

<xs:element name="satellite-info" type="sealSFevents:SATInfoType" minOccurs="0"/>

<xs:element name="anyExt" type="sealSFevents:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="SATInfoType">

<xs:sequence>

<xs:element name="satellite-id" type="xs:unsignedByte" minOccurs="0"/>

<xs:annotation>

<xs:documentation>

SatelliteId per 3GPP TS 36.331 clause 6.3.6: unsigned 1 octet integer, range 0..255.

</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="rat-type" type="xs:string" minOccurs="0"/>

<xs:element name="ue-store-forward-mode" type="xs:string" minOccurs="0"/>

<xs:element name="max-store-quota" type="sealSFevents:SFMaxStoreQuotaType" minOccurs="0"/>

<xs:element name="max-reten-time" type="xs:positiveInteger" minOccurs="0"/>

<xs:element name="anyExt" type="sealSFevents:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="anyExtType">

<xs:sequence>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:schema>

==============Next change==============

### 7.5.7	SEALSatelliteS&FEventsInfo document

Editor's note (5GSAT_Ph3_App, CR0093): 	The details of the <est-time> element and <feedlnk-period> element are FFS.

The <seal-store-forward-events-info> element is the root element of the XML document. The <seal-store-forward-events-info> element contains one of following sub-elements:

a)	<store-forward-subscription> elements;

b)	<store-forward-notification> element; or

c)	<store-forward-report> element.

The <store-forward-subscription> elements:

a)	shall include <subscription-id> to identitfy the subscription;

b)	shall include one or more <store-forward-events> elements. The <store-forward-events> element inludes:

1)	a <ue-store-forward-mode>, an optional element indicatings the UE is registered in S&F mode or moving from S&F mode to not registered in S&F mode. Table 7.5.7-1 provides the valid values of <ue-store-forward-mode>.

Table 7.5.7-1: ue-store-forward-mode

| Value | Description |
| --- | --- |
| IN_SF_MODE | Indicates that the UE is registered in Store and Forward mode. |
| OUT_OF_SF_MODE | Indicates that the UE is moving from Store and Forward mode to not registered in Store and Forward mode. |
| Other value shall be ignored. |  |

2)	may contain a <est-time> element indicating the estimated/expected time required to deliver the data to the UE from the time data has been received in the network; and

3)	may contain a <feedlnk-period> element indicating the availability period when the feeder link is available.

c)	shall include one or more <store-forward-trigger> elements to identify the events that triggered NRM client sending the report. Table 7.5.7-2 provides the valid values of <store-forward-trigger>.

Table 7.5.7-2: store-forward-trigger

| Value | Description |
| --- | --- |
| UE_SF_MODE | Indicates that the triggering condition is the UE status in S&F mode. |
| EST_DELIVERY_TIME | Indicates that the triggering condition is the DL S&F estimated delivery time. |
| Other value shall be ignored. |  |

The <store-forward-notification> element:

a)	shall include one or more <store-forward-events> elements. The <store-forward-events> element inludes:

1)	a <ue-store-forward-mode>, a optional element indicatings the UE is registered in S&F mode or moving from S&F mode to not registered in S&F mode. Table 7.5.7-1 provides the valid values of <ue-store-forward-mode>.

2)	may contain a <est-time> element indicating the estimated/expected time required to deliver the data to the UE from the time data has been received in the network; and

3)	may contain a <feedlnk-period> element indicating the availability period when the feeder link is available.

b)	may include one or more <store-forward-trigger> elements to identify the events that triggered NRM client sending the report. Table 7.5.7-2 provides the valid values of <store-forward-trigger>.

c)	may contain a <time-stamp> element. String with format "date-time" as defined in OpenAPI Specification [36].

The <store-forward-report> element:

a)	may contain a <UE-type> element to indicate the type of VAL UE (e.g. IOT). Table 7.5.7-3 provides the valid values of <store-forward-trigger>; and

Table 7.5.7-3: store-forward-trigger

| Value | Description |
| --- | --- |
| Normal UE | Indicates that the UE is not an IOT device. |
| IoT device | Indicates that the UE is an IOT device. |
| Other value shall be ignored. |  |

b)	shall contain a <satellite-info> element to indicate the satellite information of VAL UE, which shall include at least one of the below sub-elements:

1)	a <satellite-id> element to indicate the serving satellite ID. The value part of the satellite-id is an unsigned byte coded value from 0 to 255 (see SatelliteId IE in 3GPP TS 36.331 [r36331] clause 6.3.6)

2)	a <rat-type> element set to the satellite RAT types corresponding to the satellite availability in the indicated geographical area. Table 7.5.7-4 provides the valid values of rat-type;

Table 7.5.7-4: Rat-type

| Value | Description |
| --- | --- |
| "NR_LEO" | NR (LEO) satellite access type |
| "NR_MEO" | NR (MEO) satellite access type |
| "NR_GEO" | NR (GEO) satellite access type |
| "NR_OTHER_SAT" | NR (OTHERSAT) satellite access type |
| Other value shall be ignored. |  |

3)	a <ue-store-forward-mode> element to indicate the UE is registered in S&F mode or moving from S&F mode to not registered in S&F mode as described in table 7.5.7-1;

4)	a <max-store-quota> element to indicate the maximum data storage quota for the VAL UE for all of services on the application layer; and

5)	a <max-retention-time> element to indicate the maximum data retention time for the VAL UE per service on the application layer. The value is given in seconds.

The recipient of the XML document ignores any unknown element and any unknown attribute.

==============End of change==============