3GPP TSG-CT WG1 Meeting #159	C1-260313

# Goa, India, 9th – 13th February 2026

| CR-Form-v12.5.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.369 | CR | 0042 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Indication to inform the device is permanently disabled |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-CT |  |  |  |  |  | Date: |  |  | 2026-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | According to TS 23.369, the permanently disabled AIoT device: - disables transmission capability of RF signals, and - no longer responds to the inventory procedure. To implment the above stage-2 requirements in stage-3, the NAS layer provides an indication to lower layers to inform the device is permanently disabled so that the lower layers no longer deliver a paging indication to the NAS layer. Also, this CR suggests to delete the requirement for the AIoT device to disable the communication capability of the AIoT device because how to implement this in the NAS layer is unclear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The NAS layer informs to lower layers that the device is permanently diabled. Backward compatibility analysis: As Ambient IoT is the new Rel-19 feature, there is no backwards compatibile issues based on the changes of this CR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The lower layers do not know that the AIoT device is permanently disabled, and the lower layers will deliver a paging indication to the NAS layer. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

5.3.4.3	Permanent disable command procedure accepted by AIoT device

Upon receiving the PERMANENT DISABLE COMMAND message, the AIoT device shall send a PERMANENT DISABLE COMPLETE message to the AIOTF and inform to the lower layer that the AIoT device is permanently disabled.

==============End of change==============