3GPP TSG-CT WG1 Meeting #157	C1-256787

Sophia-Antipolis, France 13-17 October 2025	was C1-xxxxxx

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.554 | CR | 0846 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduce the list of layer-2 IDs for U2N relays in the path |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2025-10-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There is an EN: “Editor’s note [5G_ProSe_Ph3, CR#0680]: It is FFS whether the Layer-2 IDs are included in the Multi-hop path info IE, together with the list of the User info IDs (or as a separate IE), due to the following statement from clause 6.3.2.5.3 in 3GPP TS 23.304: "A 5G ProSe Intermediate UE-to-Network Relay sends a Solicitation message, it additionally includes its own User Info ID in the message. i.e. the message contains the path information which is an (ordered) list of User Info ID and Layer-2 ID of Relays in the path that has relayed the Solicitation message".” The list of Layer-2 ID of relays in the path is related to the list of user info ID. According to CR0558, the layer-2 ID is added. So it is proposed to add U2N layer-2 IDs in the Multi-hop path info IE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the list of layer-2 IDs for U2N relays in the path info. Corret the path info IE’s name from “multi-hop path info” to “multi-hop U2N path info”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The list of layer-2 IDs for U2N relays in Solicitation message is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8b.2.1.3.2.3, 8b.2.1.3.3.2, 10.2.1, 10.2.16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8b.2.1.3.2.3	Discoverer intermediate UE procedure for multi-hop UE-to-network relay discovery initiation

This procedure is performed by:

a)	a 5G ProSe multi-hop child UE-to-network relay UE towards a 5G ProSe multi-hop parent UE-to-network relay UE; or

b)	a 5G ProSe intermediate UE-to-network UE towards a 5G ProSe multi-hop UE-to-network relay UE.

The UE is authorised to perform the discoverer intermediate UE procedure for multi-hop UE-to-network relay discovery if:

a)	the UE is authorised to act as a 5G ProSe intermediate UE-to-network UE in the PLMN indicated by the serving cell, and

1)	the UE is served by NG-RAN; or

2)	the UE is not served by NG-RAN and intends to use the provisioned radio resources for UE-to-network relay discovery;

b)	the UE is configured with:

1)	the relay service code parameter identifying the connectivity service to be solicited or responded to as specified in clause 5.2.8 and the indicated security procedure for the relay service code is supported by the UE. For 5G ProSe layer-3 intermediate UE-to-network relay UE:

i)	the S-NSSAI associated with that relay service code shall belong to the allowed NSSAI of the UE; and

ii)	if the UE is camped on a cell whose TAI is in the list of "non-allowed tracking areas" or is camped on a cell whose TAI is not in the list of "allowed tracking areas", then the relay service code shall be associated with high priority access as defined in clause 5.3.5 of 3GPP TS 24.501 [11];

2)	an indication that multi-hop relay is allowed for the relay service code and the maximum number of allowed hops as specified in clause 5.2.8; and

3)	the User info ID for the multi-hop UE-to-network relay discovery parameter, as specified in clause 5.2.8; and

c)	the back-off timer T3346 used for NAS mobility management congestion control as specified in clause 5.3.9 of 3GPP TS 24.501 [11] is not running at the UE;

otherwise, the UE is not authorised to perform the discoverer intermediate UE procedure for multi-hop UE-to-network relay discovery.

Figure 8b.2.1.3.2.3.1 illustrates the interaction of the UEs in the discoverer intermediate UE procedure for multi-hop UE-to-network relay discovery.

Figure 8b.2.1.3.2.3.1: Discoverer intermediate UE procedure for multi-hop UE-to-network Relay discovery

For PROSE PC5 DISCOVERY message signal strength measurement, the UE manages a periodic measurement timer T5126, which is used to trigger the periodic PROSE PC5 DISCOVERY message signal strength measurement between the UE and the next 5G ProSe intermediate UE-to-network relay UE or the 5G ProSe multi-hop UE-to-network relay UE with which the UE has a link established. Timer T5126 is started whenever the UE has established a direct link with a 5G ProSe intermediate UE-to-network relay UE or a 5G ProSe multi-hop UE-to-network relay UE and restarted whenever the UE receives the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response from the 5G ProSe intermediate UE-to-network relay UE or the 5G ProSe multi-hop UE-to-network relay UE with which the UE has a link established.

If the UE is authorised to perform the discoverer intermediate UE procedure for multi-hop UE-to-network relay discovery and when the UE is triggered by the upper layers to solicit proximity of a connectivity service provided by a 5G ProSe intermediate UE-to-network relay UE or a 5G ProSe multi-hop UE-to-network relay UE, or when the periodic measurement timer T5126 expires, or upon receiving and processing a PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation that has a matching RSC and has the User info ID of the receiving UE included in the list of User info IDs in the Intermediate relays IE, if any, then the UE:

a0)	if the hop count parameter received in the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation is smaller than the hop limit parameter in the message, or is smaller than the maximum number of allowed hops for the RSC as specified in clause 5.2.8;

a)	if the UE is served by NG-RAN and the UE in 5GMM-IDLE mode needs to request resources for sending PROSE PC5 DISCOVERY messages for relay discovery as specified in 3GPP TS 38.331 [13], shall perform a service request procedure as specified in 3GPP TS 24.501 [11];

b)	shall obtain a valid UTC time for the discovery transmission from the lower layers and generate the UTC-based counter corresponding to this UTC time;

c)	shall generate a PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation. In the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation, the UE:

1)	shall set the discoverer info parameter to the User info ID obtained from the discoverer info parameter in the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation;

2)	shall set the relay service code parameter to the relay service code parameter identifying the connectivity service to be solicited, as obtained from the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation:

3)	shall include the MIC field computed as described in 3GPP TS 33.503 [34] by using the UTC-based counter and the DUIK contained in the <UNR-discovery-security-parameters-accept> element of the PROSE_SECURITY_PARAM_RESPONSE message, if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used, or the PROSE_SECURITY_MATERIAL_RESPONSE message, if security procedure over control plane for 5G ProSe multi-hop UE-to-network relay is used;

4)	shall set the UTC-based counter LSB parameter to the 4 least significant bits of the UTC-based counter;

5)	shall set the ProSe direct discovery PC5 message type parameter as specified in table 10.2.1.17;

6)	shall include the Multi-hop U2N path info IE and include in it the list of the User info IDs and layer-2 IDs received in the Multi-hop U2N path info IE of the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation, if any, and add its own configured User info ID and layer-2 ID for the multi-hop UE-to-network relay discovery parameter, as specified in clause 5.2.8, at the end of the list;

7)	may include the target discoveree info parameter set to the user info ID of the targeted discoveree user (i.e. the targeted 5G ProSe multi-hop UE-to-network relay UE) if it was included in the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation;

8)	may include the Intermediate relays IE containing the list of User info IDs of the 5G ProSe intermediate UE-to-network relay UEs if it was included in the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation;

9)	shall include the hop count parameter after being incremented by 1;

10)		shall include the hop limit parameter and set it to the value obtained from the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation, if available;

11)	may include the Accumulated QoS obtained from the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation, after updating it with the QoS of the PC5 link between the discoverer UE and its 5G ProSe multi-hop child UE-to-network relay UE or its 5G ProSe multi-hop remote UE; and

12)	shall include the PLMN ID set to the PLMN ID value obtained from the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation, and in cleartext;

NOTE 1:	The confidentiality security mechanism ensures that the PLMN ID IE is not encrypted as specified in 3GPP TS 33.503 [34]. Also the scrambling security mechanism ensures that the PLMN ID IE is not scrambled as specified in 3GPP TS 33.503 [34].

d)	shall apply the DUIK, DUSK, or DUCK with the associated Encrypted Bitmask, along with the UTC-based counter to the PROSE PC5 DISCOVERY message for whichever security mechanism(s) configured to be applied, e.g., integrity protection, message scrambling or confidentiality protection of one or more above parameters, as specified in 3GPP TS 33.503 [34];

e)	shall set the default destination layer-2 ID as specified in clause 5.2.8 to the destination layer-2 ID and self-assign a source layer-2 ID for sending the multi-hop UE-to-network relay discovery solicitation message; and

NOTE 2:	The UE implementation ensures that the value of the self-assigned source layer-2 ID is different from any other self-assigned source layer-2 ID(s) in use for 5G ProSe direct communication as specified in clause 7.2, is different from any other provisioned destination layer-2 ID(s) as specified in clause 5.2 and is different from any other self-assigned source layer-2 ID in use for a simultaneous 5G ProSe direct discovery procedure over PC5 with a different discovery model.

f)	shall pass the resulting PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation along with the source layer-2 ID, destination layer-2 ID and an indication that the message is for 5G ProSe direct discovery to the lower layers for transmission over the PC5 interface.

If the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation is used to solicit proximity of a connectivity service provided by another 5G ProSe intermediate UE-to-network relay UE or a 5G ProSe multi-hop UE-to-network relay UE, the UE shall ensure that it keeps on passing the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation for transmission until the UE is triggered by an upper layer application to stop soliciting proximity of a connectivity service provided by a 5G ProSe intermediate UE-to-network relay UE or a 5G ProSe multi-hop UE-to-network relay UE, or until the UE stops being authorised to perform the discoverer intermediate UE procedure for multi-hop UE-to-network relay discovery. How this is achieved is left up to UE implementation.

NOTE 3:	The discoverer UE can stop discoverer intermediate UE procedure for multi-hop UE-to-network relay discovery for power saving by implementation specific means e.g. an implementation-specific maximum number of 5G ProSe direct links configured in the UE, or an implementation-specific timer expires.

If the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation is used to trigger the PROSE PC5 DISCOVERY message signal strength measurement between the UE and by the next 5G ProSe intermediate UE-to-network relay UE or the 5G ProSe multi-hop UE-to-network relay UE with which the UE has a link established, the UE shall start the retransmission timer T5125. If retransmission timer T5125 expires, the UE shall retransmit the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation and restart timer T5125. If no response is received from the 5G ProSe intermediate UE-to-network relay UE or the 5G ProSe multi-hop UE-to-network relay UE with which the UE has a link established after reaching the maximum number of allowed retransmissions, the UE shall trigger relay reselection procedure.

NOTE 4:	The maximum number of allowed retransmissions is UE implementation specific.

For the discoverer UE (the 5G ProSe intermediate UE-to-network relay UE), upon reception of a PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response along with the destination layer-2 ID which the UE is configured to respond for, if the received PLMN ID in the message is not associated with the target relay service code of the connectivity service which the UE is authorized to monitor, the UE shall ignore the message. If the received PLMN ID in the message is associated with the target relay service code of the connectivity service which the UE is authorized to discover, the UE shall use the associated DUSK, if received from the 5G DDNMF or 5G PKMF (if security procedure over user plane for multi-hop 5G ProSe UE-to-network relay is used) and the UTC-based counter obtained during the reception operation to unscramble the PROSE PC5 DISCOVERY message as described in 3GPP TS 33.503 [34]. Then, if a DUCK is received from the 5G DDNMF or 5G PKMF (if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used), the UE shall use the DUCK and the UTC-based counter to decrypt the configured message-specific confidentiality-protected portion, as described in 3GPP TS 33.503 [34]. Finally, if a DUIK is received from the 5G DDNMF or 5G PKMF (if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used), the UE shall use the DUIK and the UTC-based counter to verify the MIC field in the unscrambled PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response.

NOTE 5:	The UE can determine the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response is for 5G ProSe direct discovery based on an indication from the lower layer.

Then if:

a)	the relay service code parameter of the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response is the same as the relay service code parameter of the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation; and

b)	the User info ID of the Discoverer UE receiving the message is included in the list of User info IDs in the Multi-hop U2N path info IE of the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response;

then the UE shall initiate the discoveree intermediate UE procedure for multi-hop UE-to-network relay discovery by sending PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response towards its 5G ProSe multi-hop child UE-to-network relay UE or 5G ProSe multi-hop remote UE as specified in clause 8b.2.1.3.3.3.

In addition, the UE can measure the signal strength of the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response for relay selection or reselection. If the UE has received the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response from the 5G ProSe intermediate UE-to-network relay UE or the 5G ProSe multi-hop UE-to-network relay UE with which the UE has a link established, the UE shall stop the retransmission timer T5125 and start the periodic measurement timer T5126.

* * * Next Change * * * *

8b.2.1.3.3.2	Discoveree UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery initiation

This procedure is performed by:

a)	a 5G ProSe multi-hop UE-to-network relay UE towards a 5G ProSe intermediate UE-to-network relay UE or a 5G ProSe multi-hop remote UE.

The UE is authorised to perform the discoveree UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery if:

a)	the UE is authorised to act as a 5G ProSe multi-hop UE-to-network relay UE in the PLMN indicated by the serving cell, and

1)	the UE is served by NG-RAN; or

2)	the UE is not served by NG-RAN and intends to use the provisioned radio resources for UE-to-network relay discovery;

b)	the UE is configured with:

1)	the relay service code parameter identifying the connectivity service to be responded to as specified in clause 5.2.8 and the indicated security procedure for the relay service code is supported by the UE. For 5G ProSe multi-hop layer-3 UE-to-network relay UE,

i)	the S-NSSAI associated with that relay service code shall belong to the allowed NSSAI of the UE; and

ii)	if the UE is camped on a cell whose TAI is in the list of "non-allowed tracking areas" or is camped on a cell whose TAI is not in the list of "allowed tracking areas", then the relay service code shall be associated with high priority access as defined in clause 5.3.5 of 3GPP TS 24.501 [11];

2)	an indication that multi-hop relay is allowed for the relay service code and the maximum number of allowed hops as specified in clause 5.2.8; and

3)	the User info ID for the multi-hop UE-to-network relay discovery parameter, as specified in clause 5.2.8; and

c)	the back-off timer T3346 used for NAS mobility management congestion control as specified in clause 5.3.9 of 3GPP TS 24.501 [11] is not running at the UE;

otherwise, the UE is not authorised to perform the discoveree UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery.

Figure 8b.2.1.3.3.2.1 illustrates the interaction of the UEs in the discoveree UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery.

Figure 8b.2.1.3.3.2.1: Discoveree UE procedure for intermediate UE-to-network Relay discovery

When the UE is triggered by an upper layer application to start responding to solicitation on proximity of a connectivity service provided by the 5G ProSe multi-hop UE-to-network relay UE and if the UE is authorised to perform the discoveree UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery, then the UE:

a)	if the UE is served by NG-RAN and the UE in 5GMM-IDLE mode needs to request resources for sending PROSE PC5 DISCOVERY messages as specified in 3GPP TS 38.331 [13], shall perform a service request procedure as specified in 3GPP TS 24.501 [11]; and

b)	shall instruct the lower layers to start monitoring for PROSE PC5 DISCOVERY messages.

Upon reception of a PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation, for the relay service code of the connectivity service which the UE is authorized to respond, if the received PLMN ID in the message is not associated with the target relay service code of the connectivity service which the UE is authorized to respond, the UE shall ignore the message. If the received PLMN ID in the message is associated with the target relay service code of the connectivity service which the UE is authorized to respond, the UE shall use the associated DUSK, if received from the 5G DDNMF or 5G PKMF (if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used) and the UTC-based counter obtained during the reception operation to unscramble the PROSE PC5 DISCOVERY message as described in 3GPP TS 33.503 [34]. Then, if a DUCK is received from the 5G DDNMF or 5G PKMF (if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used), the UE shall use the DUCK and the UTC-based counter to decrypt the configured message-specific confidentiality-protected portion, as described in 3GPP TS 33.503 [34]. Finally, if a DUIK is received from the 5G DDNMF or 5G PKMF (if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used), the UE shall use the DUIK and the UTC-based counter to verify the MIC field in the unscrambled PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation.

NOTE 1:	The UE can determine the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation is for 5G ProSe direct discovery based on an indication from the lower layer.

Then, if:

a)	the relay service code parameter of the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation is the same as the relay service code parameter configured as specified in clause 5.2.8 for the connectivity service;

b)	the target discoveree info parameter of the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation is the same as the user info ID for the multi-hop UE-to-network relay discovery provided by upper layers or same as the configured user info ID for the multi-hop UE-to-network relay discovery as specified in clause 5.2.8, if the target discoveree info parameter is included in the received PROSE PC5 DISCOVERY message; and

c)	if the hop count parameter in the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation is not greater than the maximum number of allowed hops as specified in clause 5.2.8;

then the UE:

a)	shall obtain a valid UTC time for the discovery transmission from the lower layers and generate the UTC-based counter corresponding to this UTC time;

b)	shall generate a PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response. In the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response, the UE:

1)	shall set the Discoveree info parameter to the configured User info ID for the multi-hop UE-to-network Relay discovery parameter, as specified in clause 5.2.8;

2)	shall set the relay service code parameter to the relay service code parameter of the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation. The relay service code parameter is set to relay service code specific for emergency service if the UE is triggered by an upper layer application to announce availability of an emergency connectivity service provided by a UE-to-network relay, and either of the following:

i)	for 5G ProSe multi-hop layer-3 UE-to-network relay UE, the EMC bit in the 5GS network feature support IE of the REGISTRATION ACCEPT message is not set to "Emergency services not supported" as specified in 3GPP TS 24.501 [11] and the UE is neither using its own emergency service nor relaying the emergency services from one 5G ProSe multi-hop layer-3 remote UE;

3)	shall set the Resource Status Indicator bit of the status indicator parameter to indicate whether or not the UE has resources available to provide a connectivity service for additional ProSe-enabled UEs;

4)	shall include the MIC filed computed as described in 3GPP TS 33.503 [34] by using the UTC-based counter and the DUIK contained in the <UNR-discovery-security-parameters-accept> element of the PROSE_SECURITY_PARAM_RESPONSE message, if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used, or the PROSE_SECURITY_MATERIAL_RESPONSE message, if security procedure over control plane for 5G ProSe multi-hop UE-to-network relay is used;

5)	shall set the UTC-based counter LSB parameter to the 4 least significant bits of the UTC-based counter;

6)	shall set the ProSe direct discovery PC5 message type parameter as specified in table 10.2.1.10;

7)	shall include the Multi-hop U2N path info IE and include in it the ordered list of the User info IDs and layer-2 IDs of the selected path, starting from the User info ID and layer-2 ID of the 5G ProSe multi-hop remote UE and ending with the User info ID and layer-2 ID of the last 5G ProSe intermediate UE-to-network relay UE;

8)	may include the hop count parameter to indicate the number of hops between the 5G ProSe multi-hop remote UE and the 5G ProSe multi-hop UE-to-network relay UE on the path selected by the 5G ProSe multi-hop UE-to-network relay UE;

9)	may include the Accumulated QoS IE that includes the QoS supported over all the PC5 links along the path from the 5G ProSe multi-hop remote UE to the 5G ProSe multi-hop UE-to-network relay UE; and

10)	shall include the PLMN ID set to its HPLMN ID and in cleartext;

NOTE 1:	The confidentiality security mechanism ensures that the PLMN ID IE is not encrypted as specified in 3GPP TS 33.503 [34]. Also the scrambling security mechanism ensures that the PLMN ID IE is not scrambled as specified in 3GPP TS 33.503 [34].

c)	shall apply the DUIK, DUSK, or DUCK with the associated Encrypted Bitmask, along with the UTC-based counter to the PROSE PC5 DISCOVERY message for whichever security mechanism(s) configured to be applied, e.g., integrity protection, message scrambling or confidentiality protection of one or more above parameters, as specified in 3GPP TS 33.503 [34];

d)	shall set the destination layer-2 ID to the source layer-2 ID received from the discoverer UE used in the transportation of the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation and self-assign a source layer-2 ID for sending the multi-hop UE-to-network relay discovery response message; and

NOTE 2:	The UE implementation ensures that the value of the self-assigned source layer-2 ID is different from any other self-assigned source layer-2 ID(s) in use for 5G ProSe direct communication as specified in clause 7.2 and is different from any other provisioned destination layer-2 ID(s) as specified in clause 5.2.

e)	shall pass the resulting PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response along with the source layer-2 ID, destination layer-2 ID and an indication that the message is for 5G ProSe direct discovery to the lower layers for transmission over the PC5 interface.

NOTE 3:	It is up to the implementation of the 5G ProSe multi-hop UE-to-network relay UE how it decides when to send the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response, e.g. due to considering the need to wait till it gets multiple PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation from different paths. The 5G ProSe multi-hop UE-to-network relay UE can decide on the selected path based on different factors such as the PC5 signal strength, number of hops to the 5G ProSe multi-hop remote UE, the path information in the Multi-hop U2N path info IE, the Accumulated QoS for PC5 link.

The UE shall maintain the association(s) between the self-assigned source layer-2 ID and relay service code once generating the PROSE PC5 DISCOVERY message. Each self-assigned source layer-2 ID can be associated with only one relay service code.

NOTE 4:	If the UE is processing a PROSE DIRECT LINK ESTABLISHMENT REQUEST message from the same source layer-2 ID of the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation, it depends on UE implementation to avoid the conflict of destination layer-2 ID (e.g. send a PROSE DIRECT LINK ESTABLISHMENT REJECT message containing PC5 signalling protocol cause value #3 "conflict of layer-2 ID for unicast communication is detected", or ignore the PROSE DIRECT DISCOVERY message for multi-hop UE-to-network relay discovery solicitation).

* * * Next Change * * * *

10.2.1	Message definition

This message is sent by the UE over the PC5 interface for open 5G ProSe direct discovery and restricted 5G ProSe direct discovery. See table 10.2.1.1, table 10.2.1.2, table 10.2.1.3, table 10.2.1.4, table 10.2.1.5, table 10.2.1.6, table 10.2.1.7, table 10.2.1.8, table 10.2.1.9, table 10.2.1.10, table 10.2.1.11, table 10.2.1.12, table 10.2.1.13, table 10.2.1.14, table 10.2.1.15, table 10.2.1.16, table 10.2.1.17, table 10.2.1.18 table 10.2.1.19, table 10.2.1.20, table 10.2.1.21, table 10.2.1.22, table 10.2.1.23 and table 10.2.1.24.

Message type:	PROSE PC5 DISCOVERY

Significance:	dual

Direction:	UE to peer UE

Table 10.2.1.1: PROSE PC5 DISCOVERY message content for open 5G ProSe direct discovery announcement

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.14 | M | V | 1 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | ProSe application code | ProSe application code 11.2.2 | M | V | 23 |
| 7A | Metadata | Metadata 11.2.13 | O | TLV-E | 4-8195 |
| NOTE: The discovery type is set to "Open discovery" and the content type is set to "Announcement". |  |  |  |  |  |

Table 10.2.1.2: PROSE PC5 DISCOVERY message content for restricted 5G ProSe direct discovery announcement

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.14 | M | V | 1 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | ProSe restricted code | ProSe restricted code 11.2.3 | M | V | 23 |
| 7A | Metadata | Metadata 11.2.13 | O | TLV-E | 4-8195 |
| NOTE: The discovery type is set to "Restricted discovery" and the content type is set to "Announcement". |  |  |  |  |  |

Table 10.2.1.3: PROSE PC5 DISCOVERY message content for restricted 5G ProSe direct discovery solicitation

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.14 | M | V | 1 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | ProSe query code | ProSe restricted code 11.2.3 | M | V | 23 |
| 2A | Discoveree user info | Application layer ID 11.2.15 | O | TLV | 3-257 |
| NOTE: The discovery type is set to "Restricted discovery" and the content type is set to "Solicitation". |  |  |  |  |  |

Table 10.2.1.4: PROSE PC5 DISCOVERY message content for restricted 5G ProSe direct discovery response

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.14 | M | V | 1 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | ProSe response code | ProSe restricted code 11.2.3 | M | V | 23 |
| 7A | Metadata | Metadata 11.2.13 | O | TLV-E | 4-8195 |
| NOTE: The discovery type is set to "Restricted discovery" and the content type is set to "response". |  |  |  |  |  |

Table 10.2.1.5: PROSE PC5 DISCOVERY message for group member discovery announcement

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.14 | M | V | 1 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Application layer group ID | Application layer group ID 11.2.6 | M | LV | 2-257 |
|  | Announcer info | User info ID 11.2.7 | M | V | 6 |
| 7A | Metadata | Metadata 11.2.13 | O | TLV-E | 4-8195 |
| NOTE: The discovery type is set to "Restricted discovery", the content type is set to "Group member discovery announcement/group member discovery response" and the discovery model is set to "Model A". |  |  |  |  |  |

Table 10.2.1.6: PROSE PC5 DISCOVERY message for group member discovery solicitation

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.14 | M | V | 1 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Application layer group ID | Application layer group ID 11.2.6 | M | LV | 2-256 |
|  | Discoverer info | User info ID 11.2.7 | M | V | 6 |
| 28 | Target user info | User info ID 11.2.7 | O | TV | 7 |
| NOTE: The discovery type is set to "Restricted discovery", the content type is set to "Group member discovery solicitation" and the discovery model is set to "Model B". |  |  |  |  |  |

Table 10.2.1.7: PROSE PC5 DISCOVERY message for group member discovery response

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.14 | M | V | 1 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Application layer group ID | Application layer group ID 11.2.6 | M | LV | 2-256 |
|  | Discoveree info | User info ID 11.2.7 | M | V | 6 |
| 7A | Metadata | Metadata 11.2.13 | O | TLV-E | 4-8195 |
| NOTE: The discovery type is set to "Restricted discovery", the content type is set to "Group member discovery announcement/group member discovery response" and the discovery model is set to "Model B". |  |  |  |  |  |

Table 10.2.1.8: PROSE PC5 DISCOVERY message for UE-to-network relay discovery announcement

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | PLMN ID | PLMN ID 11.2.20 | M | V | 3 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Announcer info | User info ID 11.2.7 | M | V | 6 |
|  | Relay service code (NOTE 2) | Relay service code 11.2.8 | M | V | 3 |
|  | Status indicator | Status indicator 11.2.9 | M | V | 1 |
| 52 | NCGI | NCGI 11.2.12 | O | TV | 9 |
| 63 | RRC container | RRC container 11.2.14 | O | TLV | 3-257 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "UE-to-network relay discovery announcement/UE-to-network relay discovery response" and the discovery model is set to "Model A". NOTE 2: If the announcing UE works as a 5G ProSe Layer-3 UE-to-network relay UE, the S-NSSAI associated with the relay service code belongs to the allowed NSSAI of the UE. |  |  |  |  |  |

Table 10.2.1.9: PROSE PC5 DISCOVERY message for UE-to-network relay discovery solicitation

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Discoverer info | User info ID 11.2.7 | M | V | 6 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
| 29 | Target discoveree info | User info ID 11.2.7 | O | TV | 7 |
| NOTE: The discovery type is set to "Restricted discovery", the content type is set to "UE-to-network relay discovery solicitation" and the discovery model is set to "Model B". |  |  |  |  |  |

Table 10.2.1.10: PROSE PC5 DISCOVERY message for UE-to-network relay discovery response

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | PLMN ID | PLMN ID 11.2.20 | M | V | 3 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Discoveree info | User info ID 11.2.7 | M | V | 6 |
|  | Relay service code (NOTE 2) | Relay service code 11.2.8 | M | V | 3 |
|  | Status indicator | Status indicator 11.2.9 | M | V | 1 |
| 52 | NCGI | NCGI 11.2.12 | O | TV | 9 |
| 63 | RRC container | RRC container 11.2.14 | O | TLV | 3-257 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "UE-to-network relay discovery announcement/UE-to-network relay discovery response" and the discovery model is set to "Model B". NOTE 2: If the discoveree UE works as a 5G ProSe Layer-3 UE-to-network relay UE, the S-NSSAI associated with the relay service code belongs to the allowed NSSAI of the UE. |  |  |  |  |  |

Table 10.2.1.11: PROSE PC5 DISCOVERY message for relay discovery additional information

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | PLMN ID | PLMN ID 11.2.20 | M | V | 3 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
|  | Announcer info | User info ID 11.2.7 | M | V | 6 |
| 52 | NCGI | NCGI 11.2.12 | O | TV | 9 |
| 51 | Relay TAI | TAI 11.2.10 | O | TV | 4 |
| 10 | NID | NID 11.2.23 | O | TLV | 8 |
| NOTE: The discovery type is set to "Restricted discovery", the content type is set to "Relay discovery additional information" and the discovery model is set to "Model A". |  |  |  |  |  |

Table 10.2.1.12: PROSE PC5 DISCOVERY message for UE-to-UE relay discovery announcement

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | PLMN ID | PLMN ID 11.2.20 | M | V | 3 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
|  | Direct discovery set | Direct discovery set 11.2.17 | M | LV-E | 15-8323 |
| 2D | Status indicator | Status indicator 11.2.9 | O | TV | 2 |
| 2C | UE-to-UE relay UE info | User info ID 11.2.7 | O | TV | 7 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "UE-to-UE relay discovery announcement" and the discovery model is set to "Model A". |  |  |  |  |  |

Table 10.2.1.13: PROSE PC5 DISCOVERY message for UE-to-UE relay discovery solicitation

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | PLMN ID | PLMN ID 11.2.20 | M | V | 3 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
|  | Direct discovery set | Direct discovery set 11.2.17 | M | LV-E | 12-520 |
| 2C | UE-to-UE relay UE info | User info ID 11.2.7 | O | TV | 7 |
| 2D | Status indicator | Status indicator 11.2.9 | O | TV | 2 |
| 8- | Announce prohibited indication | Announce prohibited indication 11.2.19 | O | TV | 1 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "UE-to-UE relay discovery solicitation" and the discovery model is set to "Model B". |  |  |  |  |  |

Table 10.2.1.14: PROSE PC5 DISCOVERY message for UE-to-UE relay discovery response

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | PLMN ID | PLMN ID 11.2.20 | M | V | 3 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
|  | Direct discovery set | Direct discovery set 11.2.17 | M | LV-E | 12-520 |
| 2C | UE-to-UE relay UE info | User info ID 11.2.7 | O | TV | 7 |
| 2D | Status indicator | Status indicator 11.2.9 | O | TV | 2 |
| 8- | Announce prohibited indication | Announce prohibited indication 11.2.19 | O | TV | 1 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "UE-to-UE relay discovery response" and the discovery model is set to "Model B". |  |  |  |  |  |

Table 10.2.1.15: PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery announcement

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | ProSe direct discovery PC5 message content type extensions (NOTE 2) | ProSe direct discovery PC5 message content type extensions 11.2.1a | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | PLMN ID | PLMN ID 11.2.20 | M | V | 3 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Announcer info | User info ID 11.2.7 | M | V | 6 |
|  | Relay service code (NOTE 3) | Relay service code 11.2.8 | M | V | 3 |
|  | Status indicator | Status indicator 11.2.9 | M | V | 1 |
|  | Hop count | Hop count 11.2.21 | M | V | 1 |
| 10 | Hop limit | Hop limit 11.2.22 | O | TV | 1 |
| 11 | Root relay info | User info ID 11.2.7 | O | TV | 7 |
| 12 | Accumulated QoS | PC5 QoS flow descriptions 11.3.5 | O | TLV | 6-65538 |
| 52 | NCGI | NCGI 11.2.12 | O | TV | 9 |
| 63 | RRC container | RRC container 11.2.14 | O | TLV | 3-257 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "Extended content type" and the discovery model is set to "Model A". NOTE 2: The extended content type is set to “Multi-hop UE-to-network relay discovery announcement”. NOTE 3: If the announcing UE works as a 5G ProSe multi-hop layer-3 UE-to-network relay UE, the S-NSSAI associated with the relay service code belongs to the allowed NSSAI of the UE. |  |  |  |  |  |

Table 10.2.1.16: PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery additional information

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | ProSe direct discovery PC5 message content type extensions (NOTE 2) | ProSe direct discovery PC5 message content type extensions 11.2.1a | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | PLMN ID | PLMN ID 11.2.20 | M | V | 3 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
|  | Announcer info | User info ID 11.2.7 | M | V | 6 |
|  | Hop count | Hop count 11.2.21 | M | V | 1 |
| 8- | Hop limit | Hop limit 11.2.22 | O | TV | 1 |
| 29 | Root relay info | User info ID 11.2.7 | O | TV | 7 |
| 52 | NCGI of multi-hop UE-to-network relay | NCGI 11.2.12 | O | TV | 9 |
| 10 | NCGI info set of intermediate UE-to-network relay | NCGI info set 11.2.25 | O | TLV | 16 |
| 51 | Relay TAI | TAI 11.2.10 | O | TV | 4 |
| 53 | NID | NID 11.2.23 | O | TLV | 8 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "Extended content type" and the discovery model is set to "Model A". NOTE 2: The extended content type is set to “Multi-hop UE-to-network relay discovery additional information”. |  |  |  |  |  |

Table 10.2.1.17: PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | ProSe direct discovery PC5 message content type extensions (NOTE 2) | ProSe direct discovery PC5 message content type extensions 11.2.1a | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | PLMN ID | PLMN ID 11.2.20 | M | V | 3 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Discoverer info | User info ID 11.2.7 | M | V | 6 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
| 29 | Target discoveree info | User info ID 11.2.7 | O | TV | 7 |
| 20 | Multi-hop U2N path info | List of user info IDs and layer-2 IDs 11.3.48 | O | TLV | 12-257 |
| 21 | Intermediate relays | List of user info ID 11.3.47 | O | TLV | 8-257 |
| 22 | Hop count | Hop count 11.2.21 | O | TV | 1 |
| 23 | Hop limit | Hop limit 11.2.22 | O | TV | 1 |
| 24 | Accumulated QoS | PC5 QoS flow descriptions 11.3.5 | O | TLV | 6-65538 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "Extended content type" and the discovery model is set to "Model B". NOTE 2: The extended content type is set to "Multi-hop UE-to-network relay discovery solicitation". |  |  |  |  |  |

Table 10.2.1.18: PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | ProSe direct discovery PC5 message content type extensions (NOTE 2) | ProSe direct discovery PC5 message content type extensions 11.2.1a | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | PLMN ID | PLMN ID 11.2.20 | M | V | 3 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Discoveree info | User info ID 11.2.7 | M | V | 6 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
|  | Status indicator | Status indicator 11.2.9 | M | V | 1 |
|  | Multi-hop U2N path info | List of user info IDs and layer-2 IDs 11.3.48 | M | LV | 12-257 |
| 10 | Hop count | Hop count 11.2.21 | O | TV | 1 |
| 12 | Accumulated QoS | PC5 QoS flow descriptions 11.3.5 | O | TLV | 6-65538 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "Extended content type" and the discovery model is set to "Model B". NOTE 2: The extended content type is set to "Multi-hop UE-to-network relay discovery response". |  |  |  |  |  |

Table 10.2.1.19: PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery announcement for IP data unit type

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | ProSe direct discovery PC5 message content type extensions (NOTE 2) | ProSe direct discovery PC5 message content type extensions 11.2.1a | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Multi-hop UE-to-UE relay UE info | User info ID 11.2.7 | M | V | 6 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "Extended content type" and the discovery model is set to "Model A". NOTE 2: The extended content type is set to “Multi-hop UE-to-UE relay discovery announcement for IP data unit type”. |  |  |  |  |  |

Table 10.2.1.20: PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery solicitation for IP data unit type

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | ProSe direct discovery PC5 message content type extensions (NOTE 2) | ProSe direct discovery PC5 message content type extensions 11.2.1a | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Discoverer info | User info ID 11.2.7 | M | V | 6 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
| 29 | Multi-hop UE-to-UE relay UE info | User info ID 11.2.7 | O | TV | 7 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "Extended content type" and the discovery model is set to "Model B". NOTE 2: The extended content type is set to “Multi-hop UE-to-UE relay discovery solicitation for IP data unit type”. |  |  |  |  |  |

Table 10.2.1.21: PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery response for IP data unit type

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | ProSe direct discovery PC5 message content type extensions (NOTE 2) | ProSe direct discovery PC5 message content type extensions 11.2.1a | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Multi-hop UE-to-UE relay UE info | User info ID 11.2.7 | M | V | 6 |
|  | Discoverer info | User info ID 11.2.7 | M | V | 6 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "Extended content type" and the discovery model is set to "Model B". NOTE 2: The extended content type is set to “Multi-hop UE-to-UE relay discovery response for IP data unit type”. |  |  |  |  |  |

Table 10.2.1.22: PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery announcement for Ethernet and unstructured data unit type

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | ProSe direct discovery PC5 message content type extensions (NOTE 2) | ProSe direct discovery PC5 message content type extensions 11.2.1a | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Multi-hop UE-to-UE relay UE info | User info ID 11.2.7 | M | V | 6 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
|  | List of multi-hop UE-to-UE relay paths | List of multi-hop UE-to-UE relay paths 11.2.24 | M | LV-E | 2-65537 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "Extended content type" and the discovery model is set to "Model A". NOTE 2: The extended content type is set to “Multi-hop UE-to-UE relay discovery announcement for Ethernet and unstructured unit type”. |  |  |  |  |  |

Table 10.2.1.23: PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery solicitation for Ethernet and unstructured data unit type

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | ProSe direct discovery PC5 message content type extensions (NOTE 2) | ProSe direct discovery PC5 message content type extensions 11.2.1a | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Discoverer info | User info ID 11.2.7 | M | V | 6 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
| 10 | List of multi-hop UE-to-UE relay paths | List of multi-hop UE-to-UE relay paths 11.2.24 | O | TLV-E | 3-65538 |
| 29 | Target discoveree info | User info ID 11.2.7 | O | TV | 7 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "Extended content type" and the discovery model is set to "Model B". NOTE 2: The extended content type is set to “Multi-hop UE-to-UE relay discovery solicitation for Ethernet and unstructured data unit type”. |  |  |  |  |  |

Table 10.2.1.24: PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery response for Ethernet and unstructured data unit type

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | ProSe direct discovery PC5 message content type extensions (NOTE 2) | ProSe direct discovery PC5 message content type extensions 11.2.1a | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Discoveree info | User info ID 11.2.7 | M | V | 6 |
|  | Discoverer info | User info ID 11.2.7 | M | V | 6 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
|  | List of multi-hop UE-to-UE relay paths | List of multi-hop UE-to-UE relay paths 11.2.24 | M | LV-E | 2-65537 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "Extended content type" and the discovery model is set to "Model B". NOTE 2: The extended content type is set to “Multi-hop UE-to-UE relay discovery response for Ethernet and unstructured data unit type”. |  |  |  |  |  |

* * * Next Change * * * *

10.2.16	Multi-hop U2N path info

The Multi-hopU2N path info information element is included in the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation and PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response to provide the ordered list of the user info IDs and layer-2 IDs of the selected path.

* * * End of Changes * * * *