3GPP TSG-CT WG1 Meeting #157	C1-256534

Sophia Antipolis, France, 13-17 October 2025	Revision of C1-256021

Source:	China Mobile, SHARP, CEWiT, InterDigital

Title:	pCR on the timers for the procedures

Spec:	3GPP TS 24.369 v1.0.0

Agenda item:	19.70

Document for:	Agreement

1. Introduction

This p-CR proposes to consider the timers in the AIOTF used in the command procedures in the 3GPP TS 24.369 specification.

This p-CR is related to the subclauses for read/write/disable command procedure initiation/complete and abnormal cases in the AIOTF.

2. Reason for Change

According to TS 23.369:

***

5.2.2.3	Permanent Disable Command

...

The Permanent Disable command is sent to an AIoT Device when an authorized AF uses the Permanent Disable command service operations as described in clause 5.2.2.1, or if the network determines to disable the AIoT Device.

...

***

The inventory procedure and command procedure could be triggered not only by the AF but also by the NW, the AIOTF needs an timer for the command procedure to handle abnormal cases (e.g. no response from the AIoT device) and determines the command is not accepted/fulfilled by the device.

In addition, TS 24.501 defines 5GMM timer not only for retransmission but also for state/context management, e.g.Implicit de-registration timer, Mobile reachable timer, Implementation specific timer for onboarding services.

It is suggested to define the timers for the command procedures for abnormal case handling and determine the result of the procedures for device management by the NW.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369 v1.0.0.

* * * First Change * * * *

5.3	Command procedures

5.3.1	General

This clause describes command procedures between the AIOTF and AIoT device. Command procedures are initiated by the AIOTF to perform read, write, or permanent disable operation on the AIoT device.

* * * Next Change * * * *

5.3.2	Read command procedure

5.3.2.1	General

The purpose of the read command procedure is to retrieve AIoT data from the AIoT device (see example in figure 5.3.2.1.1).

AIoT device

AIOTF

READ COMMAND

READ COMPLETE

OR

READ COMMAND REJECT

AIoT device

AIOTF

READ COMMAND

READ COMPLETE

OR

READ COMMAND REJECT

Start T1

Stop T1

Stop T1

Figure 5.3.2.1.1: Read command procedure

5.3.2.2	Read command procedure initiation

The AIOTF initiates the read command procedure by sending a READ COMMAND message to the AIoT device and starting the timer T1. In the READ COMMAND message, the AIOTF:

a)	shall include the Read offset IE set to the offset from the start of the AIoT device's user memory, indicating where to read the AIoT data; and

b)	shall include the Read length IE set to the length of the AIoT data to be read.

* * * Next Change * * * *

5.3.2.4	Read command procedure completion by the AIOTF

Upon receiving the READ COMPLETE message, the AIOTF shall consider the read command procedure as successfully completed and stop the timer T1.

* * * Next Change * * * *

5.3.3	Write command procedure

5.3.3.1	General

The purpose of the write command procedure is to write the AIoT data to the AIoT device's user memory (see example in figure 5.3.3.1.1).

AIoT device

AIOTF

WRITE COMMAND

WRITE COMPLETE

OR

WRITE COMMAND REJECT

AIoT device

AIOTF

WRITE COMMAND

WRITE COMPLETE

OR

WRITE COMMAND REJECT

Start T2

Stop T2

Stop T2

Figure 5.3.3.1.1: Write command procedure

5.3.3.2	Write command procedure initiation

The AIOTF initiates the write command procedure by sending a WRITE COMMAND message to the AIoT device and starting the timer T2. In the WRITE COMMAND message, the AIOTF:

a)	shall include the Write offset IE set to the offset from the start of the AIoT device's user memory indicating where to write the AIoT data; and

b)	shall include the AIoT data IE set to the AIoT data to be written.

* * * Next Change * * * *

5.3.3.4	Write command procedure completion by the AIOTF

Upon receiving the WRITE COMPLETE message, the AIOTF shall consider the write command procedure as successfully completed and stop the timer T2.

* * * Next Change * * * *

5.3.4	Permanent disable command procedure

5.3.4.1	General

The purpose of the permanent disable command procedure (see example in figure 5.3.4.1.1) is to permanently disable the communication capability of the AIoT device as specified in 3GPP TS 23.369 [2].

AIoT device

AIOTF

PERMANENT DISABLE COMMAND

PERMANENT DISABLE COMPLETE

AIoT device

AIOTF

PERMANENT DISABLE COMMAND

PERMANENT DISABLE COMPLETE

Start T3

Stop T3

Figure 5.3.4.1.1: Permanent disable command procedure

5.3.4.2	Permanent disable command procedure initiation

The AIOTF initiates the permanent disable command procedure by sending a PERMANENT DISABLE COMMAND message to the AIoT device and starting the timer T3.

* * * Next Change * * * *

5.3.4.4	Permanent disable command procedure completion by the AIOTF

Upon receiving the PERMANENT DISABLE COMPLETE message, the AIOTF shall consider the permanent disable command procedure as successfully completed and stop the timer T3.

* * * Next Change * * * *

5.3.5	Abnormal cases in the AIOTF

The following abnormal cases can be identified:

a)	Lower layer failure

In the case the lower layer indication of a command failure received as specified by 3GPP TS 38.413 [10], the AIOTF shall abort the command procedure.

b)	Expiry of the timer for the command procedure

The AIOTF shall abort the command procedure and consider the command procedure is unsuccessful.

8	List of system parameters

8.1	General

This clause describes the definition of the timers for AIoT.

8.x	Timers of command procedure

Timers of command procedure are shown in table 8.x.

Table 8.x: Timers of command procedure – AIOTF side

| TIMER NUM. | TIMER VALUE | DEFINITION | CONDITION OF START | NORMAL STOP | ON EXPIRY |
| --- | --- | --- | --- | --- | --- |
| T1 | NOTE | Timer for the read command procedure | Read command procedure initiated | - Read command procedure completed or rejected. | - Abort the read command procedure and consider the read command procedure is unsuccessful. |
| T2 | NOTE | Timer for the write command procedure | Write command procedure initiated | - Write command procedure completed or rejected. | - Abort the write command procedure and consider the write command procedure is unsuccessful. |
| T3 | NOTE | Timer for the permanent disable command procedure | Permanent disable command procedure initiated | - Permanent disable command procedure completed. | - Abort the permanent disable command procedure and consider the permanent disable command procedure is unsuccessful. |
| NOTE: The value of this timer is network dependent. |  |  |  |  |  |

* * * End of Changes * * * *