3GPP TSG-CT WG1 Meeting #157	C1-256537

Sophia Antipolis, France, 13-17 October 2025

Source:	vivo, Samsung, Huawei, HiSilicon, Ericsson

Title:	Resolve EN on matching procedure

Spec:	3GPP TS 24.369-100

Agenda item:	19.70

Document for:	Decision

1. Introduction

<Introduction part (optional)>

2. Reason for Change

This CR resolves the following EN for matching in inventory procedure based on the work of SA3 and CT4.

Editor's note:	Details of AIoT device identifier should be aligned with security consideration from SA3. How to match the AIoT device identifier depends on the AIoT device identifier included.

According to the clause 5.4.1 and clause 5.4.3 of TS 33.369 (SA3), as quoted below:

When privacy protection is not used during the inventory procedure, the AIoT device includes its AIoT device permanent identifier as a device identification information in the procedure specified in clause 5.2.2.

For the protection of AIoT device permanent identifier during the inventory procedure with AIoT device identifier described in clause 5.2.2, the following changes shall apply: - In step 1, AIOTF shall retrieve a T-ID in addition to the RANDAIOT_n from ADM. The ADM shall, based on T-ID type, either fetch the stored T-ID in the AIoT device profile or generate the T-ID as specified in Annex B.1. - In step 2, 3 and 4, the T-ID shall be used as a device identification information. - In step 2 and 3 the AIOTF includes indication of type of T-ID handling. T-ID can be either concealed type or stored type. The concealed type can be based on either the stored T-ID or the permanent identifier. If needed the handling also indicates whether the stored T-ID type shall be updated with or without a command. NG-RAN includes the T-ID handling in the paging message. - In step 4, the AIoT device, based on the T-ID handling indication in the paging message, generates the T-ID in the same way as the ADM did in step 1. The AIoT device determines it needs to reply to the NG-RAN if the generated T-ID matches with the received T-ID. In case the stored T-ID update shall be done without a command, the AIoT Device generates a new Temp_ID_n+1 as specified in Annex B.1 and stores the new Temp ID_n+1. … NOTE 2: In case of concealed T-ID type, every AIoT devices that receive an Inventory Request with T-ID need to perform a T-ID matching by generating a T-ID based on the KAIoT_root and check if the generated T-ID is matched with the received T-ID. It is assumed that the AIoT device that receive the Inventory Request has enough energy to perform this T-ID matching in addition to the Inventory procedure specified in clause 5.2.2.

If privacy protection is not used in the inventory procedure, the AIoT device identifier included in the AIoT identification information is the permanent identifier; If privacy protection is used in the inventory procedure, the AIoT device identifier included in the AIoT identification information is the temporary ID (T-ID);

For the case of permanent identifier, the AIoT device uses the AIoT device permanent identifier to match. For the case of concealed T-ID, the AIoT device generates new T-ID based on stored T-ID or permanent identifier, and uses the generated T-ID to match. For the case of stored T-ID, the AIoT device uses the stored T-ID to match.

According the agreed C4-253570:

Figure 31.3.1: Structure of Filtering Information Figure 31.3.x: Structure of Filtering Element Figure 31.3.y: Structure of ID Type Filter Figure 31.3.2: Structure of Identification Information Filter

and the clause 5.4.2 of TS 33.369 (SA3):

NOTE 1: The attacker may obtain a AIoT device ID by performing a bitwise enumeration in multiple paging messages. To mitigate the attack, the AIoT device need to be configured with filtering information to match by limiting which bits of AIoT device identifier is allowed for filtering information (guidance would be to limit to the leftmost n bits of the permanent device identifier, e.g., only allow filtering information for the leftmost 64 bits and not respond otherwise).

If filtering information is included in the AIoT identification information, the AIoT identification information is matched only if all the filtering elements in the filtering information are matched. The filtering type in each filtering element indicates which component need to be matched.

Therefore, this EN is resolved by aligning with the privacy consideration of device identifier and structure of filtering information.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369 V1.0.0.

* * * First Change * * * *

5.2.3	Inventory procedure completion

The AIoT device determines to respond to the AIoT paging:

a)	based on the indication from lower layer as specified in 3GPP TS 38.391 [8]; or

b)	based on whether the AIoT identification information received from the lower layer is matched or not.

If the AIoT device determines to respond to the AIoT paging, the AIoT device shall send an INVENTORY REPORT message to the AIOTF.

The AIoT device shall derive a RANDAIOT_d value as specified in 3GPP TS 33.369 [6] and shall include the derived RANDAIOT_d  value in the in the RANDAIOT_d IE in the INVENTORY REPORT message.

The AIoT device shall derive a RESAIOT value, using the long-term configured KAIoT and the value of the RANDAIOT_n included in the paging message provided by lower layers as specified in 3GPP TS 33.369 [6]. The derived RESAIOT value shall be included in the in the RESAIOT IE in the INVENTORY REPORT message.

The AIoT device shall include the AIoT device identity IE in the INVENTORY REPORT message.

Editor's note:	Details of the AIoT device identity IE is FFS and depends on SA3 progress.

The AIoT device determines whether the AIoT identification information is matched or not as follows:

Editor's note:	It is FFS how to determine the component of AIoT identification information for AIoT device, and this may need coordination with RAN2/3.

a)	if the AIoT identification information includes an AIoT device permanent identifier:

1)	the AIoT device determines the AIoT identification information is matched if privacy protection is not used, and the AIoT identification information is the same as the AIoT device permanent identifier of the AIoT device; or

2)	otherwise, the AIoT device determines the AIoT identification information is not matched;

b)	if the AIoT identification information includes a T-ID (see clause 5.4.3 of 3GPP TS 33.369 [6]):

Editor’s note:	Details on how to match T-ID will be specified after T-ID handling indication is defined, and it may need coordination with RAN2/RAN3/CT4.

c)	if the AIoT identification information includes filtering information (see clause 31.3 of 3GPP TS 23.003 [5]):

1)	the AIoT device determines the AIoT identification information is matched if all the filtering elements in the filtering information are matched. A filtering element is considered as matched if:

A)	the filtering element is ID Type, and the component value of the filtering element is the same as the corresponding bitstring from the ID type of the AIoT device permanent identifier based on the offset field and the length field;

B)		the filtering element is PLMN ID and:

i)	the PLMN ID field is included in the AIoT device permanent identifier; and

ii)	the PLMN ID field in the AIoT device permanent identifier is the same as the component value of the filtering element;

C)	the filtering element is NID and:

i)	the NID field is included in the AIoT device permanent identifier; and

ii)	the NID field in the AIoT device permanent identifier is the same as the component value of the filtering element;

D)	the filtering element is third party identifier and:

i)	the third party identifier field is included in the AIoT device permanent identifier; and

ii)	the third party identifier field in the AIoT device permanent identifier is the same as the component value of the filtering element; or

E)	the filtering element is identification information, and the component value of the filtering element is the same as the corresponding bitstring from the AIoT device permanent identifier based on the offset field and the length field; or

2)	otherwise, the AIoT device determines the AIoT identification information is not matched.

NOTE 1:	It is allowed to limit only the leftmost n bits of the AIoT permanent device identifier to be used for matching purpose by local configuration (see clause 5.4.2 of 3GPP TS 33.369 [6]). In this case, any filtering element that requires to match the not allowed bit(s) is considered as not matched.

NOTE 2:	The AIoT identification information corresponds to the paging ID, as defined in 3GPP TS 38.391 [8]).

Based on whether the AIoT identification information is matched:

a)	if the AIoT identification information is matched, the AIoT device shall indicate to the lower layer that the AIoT identification information is matched, and determine to respond to the AIoT paging; or

b)	otherwise, the AIoT device shall ignore the AIoT identification information, indicate to the lower layer that the filtering information is not matched, and determine not to respond to the AIoT paging.

Upon all the selected NG-RAN indicating the inventory procedure has been completed, the inventory procedure is considered as completed by the AIOTF as specified in 3GPP TS 23.369 [2].

NOTE 3:	How NG-RAN indicates the inventory procedure has been completed is defined in 3GPP TS 38.300 [7].

Editor's note:	The reference to TS 38.300 may be updated according to RAN2 progress.

Editor's note:	AIoT .device authentication of the network is FFS.

* * * End of Changes * * * *