3GPP TSG-CT WG1 Meeting #157	C1-256546

Sophia Antipolis, France, 13-17 October 2025                                       (was C1-256366)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.122 | CR | 1363 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Disaster roaming services on CSG cell |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MINT_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the MINT (release-17), the UE obtain the disaster roaming service on CAG cell if the CAG-ID broadcasted is in the "Allowed CAG list". For a PLMN that provides disaster roaming services, if one of the CAG-ID(s) broadcasted by a CAG cell for the PLMN is authorized based on the "Allowed CAG list" included in the entry for the PLMN in the "CAG information list", then the UE may attempt to access the PLMN on the CAG cell for disaster roaming services. In the EPC, similar changes are needed for the CSG cell. UE can access the CSG cell for the disaster roaming service when the CSG-ID broadcasted is allowed CSG or operator CSG Scenario - Suppose, - For UE-1, Network provisioned with Allowed CSG list (e.g CSGID-1) or an Operator CSG list (e.g CSGID-2) for the associated PLMN-1. - For UE-2, Network have not provisioned with Allowed CSG list and an Operator CSG list for the associated PLMN-1. - PLMN-1 is not allowed for normal service (e.g S1 mode is disabled) in EPC - UE faced disaster condition - UE found the CSGID-1/2 cell which can provide the disaster roaming service. Then UE-1, shall be able to attach to disaster roaming services and UE-2 shall not as the CSGID-1/2 is not configured for the UE-2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | UE can obtain the disater roaming service in the CSG cell when the CSG-ID broadcasted is in the allowed CSG or operator CSG |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE may attempt CSG cell which is not allowed for the UE and results in failure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.10.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 3.10.3	MINT for MS with 5G-only national roaming access to obtain service in EPS under disaster condition

The MS with 5G-only national roaming access to a VPLMN, can obtain connectivity service in EPS from that VPLMN for the area where a Disaster Condition applies. The PLMN ID of the VPLMN under disaster condition can be the same or different from the PLMN ID of VPLMN providing connectivity service in EPS.

For a PLMN that provides disaster roaming services, the MS is allowed to access the CSG cell for disaster roaming services with a CSG identity that is in the Allowed CSG list or in the Operator CSG List.

The MS may support MINT-EPS.

If the MS supports MINT-EPS, the MS can be provisioned by the network with following information:

a)	an indication of whether disaster roaming in EPS is enabled in the UE, provided by the HPLMN;

b)	a "list of PLMN(s) to be used under disaster condition" provided by the HPLMN, consisting of zero or more entries, each containing a PLMN ID. The PLMNs are listed in order of decreasing priority, with the first PLMN being the highest priority PLMN;

c)	one or more "list of PLMN(s) to be used under disaster condition", where each VPLMN can provide one "list of PLMN(s) to be used in EPS under disaster condition", consisting of zero or more entries, each containing a PLMN ID. The PLMNs are listed in order of decreasing priority, with the first PLMN being the highest priority PLMN;

d)	a disaster roaming wait range consisting of a minimum wait time and a maximum wait time; and

e)	a disaster return wait range consisting of a minimum wait time and a maximum wait time.

If the MS supports MINT-EPS, the network may provide the UE with above information to support disaster roaming services in EPS, during a registration or a generic UE configuration update procedure as specified in 3GPP TS 24.501 [64] or during the attach or tracking area updating procedure as specified in 3GPP TS 24.301 [23A]. The handling of this information, including storage in non-volatile memory (as specified in 3GPP TS 24.301 [23A] Annex C), updates upon USIM insertion, and deletion, follows the description in subclause 3.10.1.

If the MS does not have an indication of whether disaster roaming is enabled in the EPS in the UE stored in the ME, or the indication of whether disaster roaming is enabled in the EPS in the UE stored in the ME is set to "Disaster roaming is disabled in the UE for EPS", disaster roaming in EPS is disabled at the MS. In this case, the MS shall not perform disaster roaming in EPS.

Upon selecting a PLMN for disaster roaming in EPS, if there is a disaster roaming wait range stored in the ME, the MS shall generate a random number within the disaster roaming wait range and start a timer set to the generated random number. While the timer is running, the MS shall not initiate attach or tracking area updating with the exception of performing an attach for emergency services, in the selected PLMN. If performing an attach for emergency services in the selected PLMN, the MS shall keep the timer running. Upon expiration of the timer, if the MS does not have a PDN connection for emergency bearer services, the MS may initiate attach, if still camped on the selected PLMN. Upon expiration of the timer, if the MS has a PDN connection for emergency bearer services, the MS shall initiate attach after release of the PDN connection for emergency bearer services, if still camped on the selected PLMN.

The UE determines that a disaster condition has ended if:

a)	the UE has successfully registered over non-3GPP access on a PLMN;

b)	the UE has successfully registered with an allowable PLMN except for disaster roaming services;

c)	the UE is not registered for disaster roaming services and a E-UTRAN cell selected for camping of the selected PLMN broadcasts neither the disaster related indication nor a "list of one or more PLMN(s) with disaster condition for which disaster roaming services is offered by the available PLMN" including the MS determined PLMN with disaster condition; or

NOTE 1:	If the UE is registered for disaster roaming services and the serving E-UTRAN cell of the registered PLMN broadcasts neither the disaster related indication nor a "list of one or more PLMN(s) with disaster condition for which disaster roaming services is offered by the available PLMN" including the MS determined PLMN with disaster condition, the UE does not determine that disaster condition has ended.

d)	the UE is attached for disaster roaming services receives cause value #11 "PLMN not allowed" or #13 "Roaming not allowed in this tracking area" during a tracking area updating procedure or a service request procedure or receives cause value #11 "PLMN not allowed" during a network-initiated detach procedure;

NOTE 2:	If the network informs the UE registered for disaster roaming services that it is registered for emergency services, then while the UE is registered for emergency services, the UE does not determine that disaster condition has ended.

e)	the UE performing periodic scan determines that PLMN providing service in 5GS is available.

Upon determining that a disaster condition has ended and selecting the PLMN previously with disaster condition, if a disaster return wait range stored in the ME is provided by:

1)	the PLMN providing disaster roaming services; or

2)	the selected PLMN,

and the MS is not allowed to access the network with access class 11 - 15, the MS shall generate a random number within the disaster return wait range and start a timer set to the generated random number. While the timer is running, the MS shall not initiate registration procedure with the exception of performing an initial egistration for emergency services, in the selected PLMN. If performing an initial registration for emergency services in the selected PLMN, the MS shall keep the timer running. Upon expiration of the timer or in the case the timer is not started, if the MS does not have an emergency PDU session, the MS may initiate registration, if still camped on the selected PLMN. Upon expiration of the timer, if the MS has an emergency PDU session, the MS shall initiate registration after release of the emergency PDU session, if still camped on the selected PLMN.