3GPP TSG-CT WG3 Meeting #143	C3-254109

# Sophia-Antipolis, France, 13 - 17 October 2025						(Revision of C3-254xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.548 | CR | 0062 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Multimodal sync policy threshold handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_App_Ph2 |  |  |  |  |  | Date: |  |  | 6-10-2025 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Sync policy, The first and second multimodaltype can be same. There is no restriction that it should not of the same type. For example, 2 video streams in the same multi modal session can have the sync threshold by which the first video should not be delayed too much compare to the 1st video. As explained in Media Synchronization Requirements - Research Questions Task Force, “studies conducted on speech intelligibility when audio quality is degraded in such a way as to simulate age-related hearing loss have shown that when the audio signal leads the video, intelligibility declines appreciably for even the shortest asynchrony of 40 ms, but when the video signal leads the audio, intelligibility remains relatively stable for onset asynchronies up to 160 - 200 ms (Grant & Greenberg, 2001).” In Sync policy, the multimodal type can have different thresholds between different media handling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Sync policy, the restriction of the same multimodal type is removed. The sync policy positive and negative threshold is provided. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The unnecessary extra restriction in the Sync policy which forbid the policy between the same multimodal type. It is not possible to fully meet the media synchornization requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.6.2.14, A.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces a backward compatible feature to the following APIs: TS29548_SDD_PolicyConfiguration.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First Change * * * *

##### 6.5.6.2.14	Type: SyncPolicy

Table 6.5.6.2.14-1: Definition of type SyncPolicy

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| flowType1 | MultiModalFlowType | M | 1 | Contains the first multi-modal flow type. |  |
| flowType2 | MultiModalFlowType | M | 1 | Contains the second multi-modal flow type. |  |
| syncThresholdPositive | DurationMilliSec | M | 1 | Contains the maximum tolerable time delay, expressed in milliseconds, by which the multi-modal flow type identified by the "flowType1" attribute can be delayed compared to the multi-modal flow type identified by the "flowType2" attribute of the application, such that they can be perceived as being synchronous. |  |
| syncThresholdNegative | DurationMilliSec | O | 1 | Contains the maximum tolerable time delay, expressed in milliseconds, by which the multi-modal flow type identified by the "flowType2" attribute can be delayed compared to the multi-modal flow type identified by the "flowType1" attribute of the application, such that they can be perceived as being synchronous. |  |

# * * * * Next changes * * * *

# A.6	SDD_PolicyConfiguration API

openapi: 3.0.0

info:

title: SEALDD Server Policy Configuration Service

version: 1.1.0-alpha.3

description: |

SEALDD Server Policy Configuration Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.548 V19.4.0; Service Enabler Architecture Layer for Verticals (SEAL);

SEAL Data Delivery (SEALDD) Server Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.548/

servers:

- url: '{apiRoot}/sdd-pc/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

security:

- {}

- oAuth2ClientCredentials: []

paths:

/configurations:

post:

summary: Request the creation of a Policy Configuration.

operationId: CreatePolicyConfig

tags:

- Policy Configurations (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyConfig'

responses:

'201':

description: >

Created. The Policy Configuration is successfully created and a representation of

the created Individual Policy Configuration resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyConfig'

headers:

Location:

description: >

Contains the URI of the created Individual Policy Configuration resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/configurations/{configId}:

parameters:

- name: configId

in: path

description: >

Represents the identifier of the Individual Policy Configuration resource.

required: true

schema:

type: string

get:

summary: Retrieve an existing Individual Policy Configuration resource.

operationId: GetIndPolicyConfig

tags:

- Individual Policy Configuration (Document)

responses:

'200':

description: >

OK. The requested Individual Policy Configuration resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyConfig'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Request the update of an existing Individual Policy Configuration resource.

operationId: UpdateIndPolicyConfig

tags:

- Individual Policy Configuration (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyConfig'

responses:

'200':

description: >

OK. The Individual Policy Configuration resource is successfully updated and a

representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyConfig'

'204':

description: >

No Content. The Individual Policy Configuration resource is successfully updated

and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing Individual Policy Configuration resource.

operationId: ModifyIndPolicyConfig

tags:

- Individual Policy Configuration (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/PolicyConfigPatch'

responses:

'200':

description: >

OK. The Individual Policy Configuration resource is successfully modified and a

representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyConfig'

'204':

description: >

No Content. The Individual Policy Configuration resource is successfully modified and

no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Request the deletion of an existing Individual Policy Configuration resource.

operationId: DeleteIndPolicyConfig

tags:

- Individual Policy Configuration (Document)

responses:

'204':

description: >

No Content. The Individual Policy Configuration resource is successfully deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

PolicyConfig:

description: >

Represents a SEALDD Policy Configuration.

type: object

properties:

appTrafficIds:

type: array

items:

type: string

minItems: 1

valTargetId:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

sliceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

sealddPol:

$ref: '#/components/schemas/SealddPolicy'

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTimeRo'

multimodsddPol:

$ref: '#/components/schemas/MultiModalSealddPolicy'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- appTrafficIds

anyOf:

- required: [sealddPol]

- required: [multimodsddPol]

PolicyConfigPatch:

description: >

Represents the requested modifications to a SEALDD Policy Configuration.

type: object

properties:

sealddPol:

$ref: '#/components/schemas/SealddPolicy'

multimodsddPol:

$ref: '#/components/schemas/MultiModalSealddPolicy'

SealddPolicy:

description: >

Represents a SEALDD Policy.

type: object

properties:

qualGuarPol:

$ref: '#/components/schemas/QualGuarPolicy'

qualOptPol:

$ref: '#/components/schemas/QualGuarPolicy'

bdwCtrlSets:

type: array

items:

$ref: '#/components/schemas/BdwCtrlPolicy'

minItems: 1

geoPol:

type: array

items:

$ref: '#/components/schemas/GeofencingPolicy'

minItems: 1

tempPol:

$ref: '#/components/schemas/TempPolicy'

non3gppAccMeasPol:

$ref: '#/components/schemas/Non3gppAccessMeasPol'

anyOf:

- required: [qualGuarPol]

- required: [qualOptPol]

- required: [bdwCtrlSets]

- required: [geoPol]

- required: [tempPol]

- required: [non3gppAccMeasPol]

QualGuarPolicy:

description: >

Represents the quality guarantee policy.

type: object

properties:

thresholds:

$ref: '#/components/schemas/QualGuarThresh'

required:

- thresholds

QualGuarThresh:

description: >

Represents the quality guarantee related thresholds.

type: object

properties:

measId:

type: array

items:

$ref: 'TS29548_SDD_TransmissionQualityMeasurement.yaml#/components/schemas/MeasurementId'

minItems: 1

measThesh:

$ref: 'TS29548_SDD_TransmissionQualityMeasurement.yaml#/components/schemas/TransQualMeasCriteria'

required:

- measId

- measThesh

GeofencingPolicy:

description: Represents a geofencing policy.

type: object

properties:

valServiceId:

type: string

area:

$ref: '#/components/schemas/GeofencingArea'

action:

$ref: '#/components/schemas/GeoPolAction'

required:

- valServiceId

- area

- action

GeofencingArea:

description: Represents a geofencing area.

type: object

properties:

geoAreas:

type: array

items:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/GeographicArea'

minItems: 1

anyOf:

- required: [geoAreas]

MultiModalSealddPolicy:

description: >

Represents a Multi-modal SEALDD Policy.

type: object

properties:

syncPol:

type: array

items:

$ref: '#/components/schemas/SyncPolicy'

minItems: 1

description: >

Contains the synchronization policy.

The key of the map shall be set to the multi-modal flow type provided within the

"flowType" attribute of the corresponding map value encoded using the SyncPolicy data

structure.

alignmentPol:

$ref: '#/components/schemas/AlignmentPolicy'

ueToUePol:

$ref: '#/components/schemas/UEToUEPolicy'

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTimeRo'

anyOf:

- required: [syncPol]

- required: [alignmentPol]

- required: [ueToUePol]

TempPolicy:

description: >

Represents a temporal policy.

type: object

properties:

schedule:

type: array

items:

$ref: 'TS29122_CpProvisioning.yaml#/components/schemas/ScheduledCommunicationTime'

minItems: 1

timeWindow:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

oneOf:

- required: [schedule]

- required: [timeWindow]

Non3gppAccessMeasPol:

description: >

Represents the non-3GPP access measurement policy.

type: object

properties:

accessIds:

type: array

items:

type: string

minItems: 1

thresholds:

type: array

items:

$ref: '#/components/schemas/SignalStrengthThreshold'

minItems: 1

thrHandling:

$ref: 'TS29549_SS_NetworkResourceMonitoring.yaml#/components/schemas/ThresholdHandlingMode'

timeThr:

$ref: '#/components/schemas/TempPolicy'

anyOf:

- required: [accessIds]

- required: [thresholds]

- required: [timeThr]

SignalStrength:

description: >

Represents the signal strength value.

type: object

properties:

rssi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

anyOf:

- required: [rssi]

SignalStrengthThreshold:

description: >

Represents the signal strength threshold.

type: object

properties:

value:

$ref: '#/components/schemas/SignalStrength'

direction:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/MatchingDirection'

required:

- value

- direction

SyncPolicy:

description: >

Represents the synchronization policy.

type: object

properties:

syncThresholdPositive:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/DurationMilliSec'

syncThresholdNegative:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationMilliSec'

required:

- syncThresholdPositive

AlignmentPolicy:

description: >

Represents the multi-modal flows alignment policy.

type: object

properties:

multiModalId:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/MultiModalId'

maxAlignTime:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/DurationMilliSec'

anyOf:

- required: [multiModalId]

- required: [maxAlignTime]

UEToUEPolicy:

description: >

Represents the multi-modal UE-to-UE policy.

type: object

properties:

proxThresh:

$ref: '#/components/schemas/ProximityThresholds'

qoSThresh:

$ref: '#/components/schemas/QoSThresholds'

qoEThresh:

$ref: '#/components/schemas/QoEThresholds'

anyOf:

- required: [proxThresh]

- required: [qoSThresh]

- required: [qoEThresh]

ProximityThresholds:

description: >

Represents the proximity thresholds for entering/leaving the UE-to-UE direct communication

mode.

type: object

properties:

minUeToUedist:

type: number

format: double

minimum: 0

avgUeToUedist:

type: number

format: double

minimum: 0

maxUeToUedist:

type: number

format: double

minimum: 0

anyOf:

- required: [minUeToUedist]

- required: [avgUeToUedist]

- required: [maxUeToUedist]

QoSThresholds:

description: >

Represents the QoS thresholds for entering/leaving the UE-to-UE direct communication mode.

type: object

properties:

minLatency:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

avgLatency:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

maxLatency:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

minBitRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

avgBitRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

maxBitRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

minPackLossRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketLossRate'

avgPackLossRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketLossRate'

maxPackLossRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketLossRate'

minPackErrRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketErrRate'

avgPackErrRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketErrRate'

maxPackErrRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketErrRate'

minJitter:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

avgJitter:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

maxJitter:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

anyOf:

- required: [minLatency]

- required: [avgLatency]

- required: [maxLatency]

- required: [minBitRate]

- required: [avgBitRate]

- required: [maxBitRate]

- required: [minPackLossRate]

- required: [avgPackLossRate]

- required: [maxPackLossRate]

- required: [minPackErrRate]

- required: [avgPackErrRate]

- required: [maxPackErrRate]

- required: [minJitter]

- required: [avgJitter]

- required: [maxJitter]

QoEThresholds:

description: >

Represents the QoE thresholds for entering/leaving the UE-to-UE direct communication mode.

type: object

properties:

mos:

type: integer

minimum: 1

maximum: 5

anyOf:

- required: [mos]

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

BdwCtrlPolicy:

anyOf:

- type: string

enum:

- REALLOCATE_DL

- REALLOCATE_UL

- NOT_REALLOCATE_DL

- NOT_REALLOCATE_UL

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the bandwidth control policy.

Possible values are:

- REALLOCATE_DL: Indicates that the bandwidth control action is to reallocate the

bandwidth limit between different VAL users for DL traffic.

- REALLOCATE_UL: Indicates that the bandwidth control action is to reallocate the

bandwidth limit between different VAL users for UL traffic.

- NOT_REALLOCATE_DL: Indicates that the bandwidth control action is to not reallocate

the bandwidth limit between different VAL users for DL traffic.

- NOT_REALLOCATE_UL: Indicates that the bandwidth control action is to not reallocate

the bandwidth limit between different VAL users for UL traffic.

GeoPolAction:

anyOf:

- type: string

enum:

- ALLOW

- BLOCK

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the geofencing policy action.

Possible values are:

- ALLOW: Indicates that the geofencing policy is to allow.

- BLOCK: Indicates that the geofencing policy is to block.

# Data types describing alternative data types or combinations of data types:

#

# * * * * End of Changes * * * *