3GPP TSG-CT WG3 Meeting #143	C3-254117

# SophiaAntipolis, FR, 13 - 17 October 2025								(Revision of C3-254xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.522 | CR | 1713 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IMS Session handling editor note removal |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 6-10-2025 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per TS 23.502 clause 5.2.6.40.1: The service provides the capability to create and release an IMS session with standalone data channel media and modify data channel media in a specific IMS session. NOTE: In this Release only session management of IMS data channel is specified. CT3 has to be enhanced to support only IMS data channel in this release. In TS 29.175, ImsSessionInfo is defined as a resource Table 6.3.3.2.3.1-2: Data structures supported by the POST Request Body on this resource Data type; P; Cardinality; Description. ImsSessionInfo; M; 1; Creates a new Individual IMS Session resource. In TS 29.522, ImsSessionInfo datatype is defined within ImsSession data type. This leads to the unncessary one extra level in NEF implementation and also missing the flexibility to update the individua data during the modification. ImsSessionInfo is not fully inline with NEF SA2 requirement in terms of Media type handling. NEF has the requirement to handle Data channel only. Another caveat in the approach is that the duplicate NotifyUri datatype within ImsSessionInfo and ImsSession. ##### In TS 29.175 clause 6.1.6.3.4 Enumeration: MediaType from Rel-18 is as follows: Table 6.1.6.3.4-1: Enumeration MediaType Enumeration value; Description. "DC"; Data channel media type.. "AUDIO"; Audio media type.. "VIDEO"; Video media type.. ##### As TS 29.522, the reuse of MediaType in TS 29.175 is not applicable as per TS 23.502 requirements mentioned above. The below editor note has to be removed in TS 29.522 aligning with the above: Editor’s Note: The data mdodel of ImsSessionManagement is FFS. |
| Summary of change: |  | ImsSession datatype is updated in the simplified way. MediaInfo & MediaType is introduced for IMS data channel only support. The editor’s note is removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The open item in ImsSessionManagement with the editor’s note. The stage 3 is not inline with stage 3 in terms of media type handing. The extra one level of data handling due to ImsSessionInfo usage. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.46.3, 5.42.5.1, 5.42.5.2.2, 5.42.5.2.4, 5.42.5.3.3, A.40 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces a backward compatible feature to the following APIs: TS29522_ImsSessionManagement.yaml TS29175_Nimsas_ImsSessionManagement.yaml TS29571_CommonData.yaml TS29122_CommonData.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First Change * * * *

#### 4.4.46.3	Procedure for IMS session Update

In order to update an existing IMS data channel Session, the AF shall trigger the Nnef_ImsSessionManagement API by sending an HTTP PUT/PATCH request targeting the URI of the corresponding "Individual IMS Session" resource, with the request body including either:

-	the ImsSessiondata structure, in case the HTTP PUT method is used; or

-	the requested modifications to the resource, in case the HTTP PATCH method is used.

Upon reception of the corresponding HTTP PUT/PATCH request:

-	the NEF shall check whether the AF is authorized to perform this operation;

-	if the AF is authorized:

-	if needed, the NEF may interact with the HSS as specified in 3GPP TS 29.562 [80] to retrieve the IMS AS instance serving the target user;

-	otherwise, the NEF shall interact with the serving IMS AS to update the associated IMS Session by invoking the Nimsas_ImsSessionManagement service API of the IMS AS as specified in 3GPP TS 29.175 [78]; and

-	upon reception of a successful response from the IMS AS and successful processing of the request, the NEF shall respond to the AF with either:

-	an HTTP "200 OK" status code with the response body including a representation of the updated "Individual IMS Session" resource within the ImsSession data structure; or

-	an HTTP "204 No Content" status code.

and

-	on failure or if the NEF receives an error response from the HSS or IMS AS, the NEF shall take proper error handling actions, as specified in clause 5.42.7, and respond to the AF with an appropriate error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

# * * * * Next changes * * * *

#### 5.42.5.1	General

This clause specifies the application data model supported by the API.

Table 5.42.5.1-1 specifies the data types defined for the ImsSessionManagement service-based interface protocol.

Table 5.42.5.1-1: ImsSessionManagement specific Data Types

| Data type | Clause defined | Description | Applicability |  |
| --- | --- | --- | --- | --- |
| ImsSession | 5.42.5.2.2 | Represent an IMS Session. |  |  |
| ImsSessionPatch | 5.42.5.2.3 | Represents the requested modifications to an IMS Session. |  |  |
| MediaInfo | 5.42.5.2.4 | Represents the media information used in the IMS session |  |
| MediaType | 5.42.5.3.3 | Represents the IMS media type |  |

Table 5.42.5.1-2 specifies data types re-used by the ImsSessionManagement service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the ImsSessionManagement service based interface.

Table 5.42.5.1-2: ImsSessionManagement re-used Data Types

| Data type | Reference |  | Comments |  | Applicability |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DcMediaPara | 3GPP TS 29.175 [78] |  | Represents the parameters of the IMS DC media. |  |  |  |
| ImsSessionEventNotification | 3GPP TS 29.175 [78] |  | Represents the IMS Session Notification. |  |  |  |  |
| SessionId | 3GPP TS 29.571 [8] |  | Represents IMS Session Identity. |  |  |  |
| SessionInfoSM | 3GPP TS 29.175 [78] |  | Represents the IMS session information to be used. |  |  |  |
| SupportedFeatures | 3GPP TS 29.571 [8] |  | Represents the list of supported feature(s) and is used to negotiate the applicability of the optional features. |  |  |  |  |
| Uri | 3GPP TS 29.122 [4] |  | Represents a URI. |  |  |  |  |

# * * * * Next changes * * * *

##### 5.42.5.2.2	Type: ImsSession

Table 5.42.5.2.2-1: Definition of type ImsSession

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| afId | string | M | 1 | Contains the identifier of the AF that is sending the request. |  |
| mediaInfoSet | array(MediaInfo) | M | 1..N | Media info set includes a list of media related information. In this release, the maximum value of this attribute is 1 as the data channel is the only media type supported. The array is provided for future usage. |  |
| notifCorrelationId | string | C | 0..1 | This IE shall be present together with the notifUri, if available. When present, it shall contain the correlation ID to associate the IMS session event notification to be reported. |  |
| notifUri | Uri | O | 0..1 | Contains the URI via which IMS Session notifications shall be delivered. |  |
| sessionId | SessionId | C | 0..1 | Contains the session ID which is the identity of the IMS session. This IE contains the information in the Call-ID header which is the typical header of SIP message. It shall not be contained when creating of a new IMS session. Otherwise, it shall be contained during the response. |  |
| sessionInfo | SessionInfoSM | M | 1 | The sessionInfo is the IMS session related information. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 5.42.6. This attribute shall be present only when feature negotiation is required. |  |

# * * * * Next changes * * * *

##### 5.42.5.2.4	Type: MediaInfo

Table 5.42.5.2.4-1: Definition of type MediaInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mediaType | MediaType | M | 1 | The media type used in the session. |  |
| mediaCorrelationId | string | M | 1 | The media correlation ID is the identity of the media. |  |
| dcMediaParas | array(DcMediaPara) | C | 1..N | The information of the created data channel media. It shall be contained when the attribute "mediaType" is set to "DC". |  |

# * * * * Next changes * * * *

##### 5.42.5.3.3	Enumeration: MediaType

The enumeration MediaType represents the type of the media for which the notification is generated. It shall comply with the provisions defined in table 5.42.5.3.3-1.

Table 5.42.5.3.3-1: Enumeration MediaType

| Enumeration value | Description |
| --- | --- |
| DC | Indicates that the media type is Data channel. |

# * * * * Next changes * * * *

# A.40	ImsSessionManagement API

openapi: 3.0.0

info:

title: 3gpp-ims-sm

version: 1.0.0-alpha.2

description: |

API for IMS Session Management.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.522 V19.4.0; 5G System; Network Exposure Function Northbound APIs.

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.522/'

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/3gpp-ims-sm/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122.

paths:

/ims-sessions:

get:

summary: Request to retrieve all the active IMS Sessions managed by the NEF.

operationId: GetImsSessions

tags:

- IMS Sessions (Collection)

responses:

'200':

description: >

OK. All the active IMS Sessions resources managed by the NEF are returned.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/ImsSession'

minItems: 0

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

post:

summary: Request to create a new IMS Session resource.

tags:

- IMS Session Creation (Collection)

operationId: CreateImsSession

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ImsSession'

responses:

'201':

description: >

Created. Successful creation of a new Individual IMS Session resource.

content:

application/json:

schema:

$ref: '#/components/schemas/ImsSession'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure

{apiRoot}/3gpp-ims-sm/v1/ims-sessions/{sessionId}

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'409':

$ref: 'TS29122_CommonData.yaml#/components/responses/409'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

ImsSessionNotif:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: 'TS29175_Nimsas_ImsSessionManagement.yaml#/components/schemas/ImsSessionEventNotification'

responses:

'204':

description: >

No Content. The IMS EE notification is successfully received and acknowledged.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/ims-sessions/{sessionId}:

parameters:

- name: sessionId

in: path

description: >

Represents the identifier of the Individual IMS Session resource.

required: true

schema:

type: string

get:

summary: Retrieve an existing Individual IMS Session resource.

operationId: GetIndImsSession

tags:

- Individual IMS Session (Document)

responses:

'200':

description: >

OK. The requested "Individual IMS Session" resource is successfully returned in the

response body.

content:

application/json:

schema:

$ref: '#/components/schemas/ImsSession'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Update an existing Individual IMS Session resource.

operationId: UpdateIndImsSession

tags:

- Individual IMS Session (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ImsSession'

responses:

'200':

description: >

OK. The Individual IMS Session resource is successfully updated and a representation

of the updated resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/ImsSession'

'204':

description: >

No Content. The Individual IMS Session Management resource is

successfully updated and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'409':

$ref: 'TS29122_CommonData.yaml#/components/responses/409'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Delete an existing Individual IMS Session resource.

operationId: DeleteIndImsSession

tags:

- Individual IMS Session (Document)

responses:

'204':

description: >

No Content. The Individual IMS Session resource is successfully deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'409':

$ref: 'TS29122_CommonData.yaml#/components/responses/409'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

ImsSession:

description: Represents the IMS Session Information.

type: object

properties:

afId:

type: string

mediaInfoSet:

description: Includes a list of media related information.

type: array

items:

$ref: '#/components/schemas/MediaInfo'

minItems: 1

notifCorrelationId:

type: string

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

sessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SessionId'

sessionInfo:

$ref: 'TS29175_Nimsas_ImsSessionManagement.yaml#/components/schemas/SessionInfoSM'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- afId

- mediaInfoSet

- sessionInfo

MediaInfo:

description: The media information used during the creation or updated IMS session.

type: object

required:

- mediaType

- mediaCorrectionId

properties:

mediaType:

$ref: '#/components/schemas/MediaType'

mediaCorrectionId:

type: string

dcMediaParas:

description: The information of the created data channel media.

type: array

items:

$ref: 'TS29175_Nimsas_ImsSessionManagement.yaml#/components/schemas/DcMediaPara'

minItems: 1

#

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

MediaType:

anyOf:

- type: string

enum:

- DC

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Represents the Media Type.

Possible values are:

- DC: Indicates that the Media type is Data Channel

# * * * * End of Changes * * * *