3GPP TSG CT WG3 Meeting #143	C3-254160

# Sophia-Antipolis, FR, 13th – 17th October, 2025

Source:	Huawei

Title:	Pseudo-CR on updates and corrections to the AIMLES_ContextTransfer API

Spec:	3GPP TS 29.482 (V1.1.0)

Agenda item:	19.42 (Metaverse_App)

Document for:	Agreement

1. Introduction

The following issues have been identified in the definition of the AIMLES_ContextTransfer API:

-	The name of the custom operation can be simplified to "Transfer" and the corresponding URI path segment name to "/transfer" as "transfer" is already a verb, which thus already covers what is required by the related naming conventions. This will also simplify and shorten this URI path segment.

-	For the percentage of completion of the AIMLE operations, it should not be required to be provide it when the status is set to "paused" or "active". It should be optional so as to allow a level of freedom for various types of implementations, especially that this is not required by the related stage 2 requirements.

-	For the status of AIMLE operations, the value "completed" is currently missing and needs to be added to align with the related stage 2 requirements in Table 8.24.3.1-2 of 3GPP TS 23.482.

-	The attribute names can be simplified and shortened to remove the part "aimle" at the beginning as the data types already include it.

-	The names of some attributes, such as the attribute conveying the identifier of the AIMLE Server currently managing the AIMLE Client and the one conveying the identifiers of the AIMLE Servers that previously managed the AIMLE Client, need to be made clearer with regards to their contents.

-	Various terminology alignments and a few editorial corrections to enhance the quality and readability.

2. Reason for Change

Address the above-detailed issues.

3. Conclusions

N/A

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.482 V 1.1.0.

* * * * Start of Changes * * * *

###### 5.2.1.2.2.1	General

This service operation is used by a service consumer to request AIMLE context transfer to the AIMLE Server.

The following procedures are supported by the "AIMLES_ContextTransfer_Request" service operation:

-	AIMLE Context Transfer Request.

* * * * Next Changes * * * *

###### 5.2.1.2.2.2	AIMLE Context Transfer Request

Figure 5.2.1.2.2.2-1 depicts a scenario where a service consumer sends a request to the AIMLE Server to request AIMLE context information transfer (see also clause 8.24 of 3GPP°TS°23.482°[13]).

Figure 5.2.1.2.2.2-1: Procedure for AIMLE Context Transfer Request

1.	In order to request AIMLE context information transfer, the service consumer shall send an HTTP POST request to the AIMLE Server targeting the URI of the corresponding custom operation (i.e., "Transfer"), with the request body including the TransReq data structure.

2a.	Upon success, the AIMLE Server shall respond with an HTTP "204 No Content" status code to indicate that the AIMLE context transfer request is successfully received and processed.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.1.1.7.

* * * * Next Changes * * * *

##### 6.1.1.4.1	Overview

The structure of the custom operation URIs of the AIMLES_ContextTransfer API is shown in Figure 6.1.1.4.1-1.

Figure 6.1.1.4.1-1: Custom operation URI structure of the AIMLES_ContextTrasnfer API

Table 6.1.1.4.1-1 provides an overview of the custom operations and applicable HTTP methods defined for the AIMLES_ContextTransfer API.

Table 6.1.1.4.1-1: Custom operations without associated resources

| Custom Operation Name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| Transfer | /transfer | POST | Enables a service consumer to request AIMLE context information transfer to the AIMLE Server. |

The custom operations shall support the URI variables defined in table 6.1.1.4.1-2.

Table 6.1.1.4.1-2: URI variables for this custom operation

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1.1. |

* * * * Next Changes * * * *

##### 6.1.1.4.2	Operation: Transfer

* * * * Next Changes * * * *

###### 6.1.1.4.2.2	Operation Definition

This operation shall support the request data structures and the response data structures and response codes specified in tables 6.1.1.4.2.2-1 and 6.1.1.4.2.2-2.

Table 6.1.1.4.2.2-1: Data structures supported by the POST Request Body on this custom operation

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TransReq | M | 1 | Contains the parameters to request AIMLE context information transfer. |

Table 6.1.1.4.2.2-2: Data structures supported by the POST Response Body on this custom operation

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The AIMLE Context Transfer request is successfully received and processed. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.1.4.2.2-3: Headers supported by the 307 Response Code on this custom operation

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative AIMLE Server. |

Table 6.1.1.4.2.2-4: Headers supported by the 308 Response Code on this custom operation

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative AIMLE Server. |

* * * * Next Changes * * * *

##### 6.1.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.1.6.1-1 specifies the data types defined for the AIMLES_ContextTransfer API.

Table 6.1.1.6.1-1: AIMLES_ContextTransfer API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AimleServOpStatus | 6.1.1.6.3.3 | Represents the AIMLE service operations status. |  |
| AimleServStatus | 6.1.1.6.2.3 | Represents the AIMLE service operations status related information. |  |
| TransReq | 6.1.1.6.2.2 | Represents the AIMLE Context Transfer request. |  |

Table 6.1.1.6.1-2 specifies data types re-used by the AIMLES_ContextTransfer API from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the AIMLES_ContextTransfer API.

Table 6.1.1.6.1-2: AIMLES_ContextTransfer API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |

* * * * Next Changes * * * *

###### 6.1.1.6.2.2	Type: TransReq

Table 6.1.1.6.2.2-1: Definition of type TransReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| clientId | AimleClientId | M | 1 | Contains the identifier of the AIMLE Client to which the transferred context is associated. |  |
| currServerId | string | O | 0..1 | Contains the identifier of the AIMLE Server currently managing the AIMLE Client, i.e., the AIMLE Server associated with service area that the AIMLE Client (to which the transferred context is associated) is currently in. |  |
| prevServerIds | array(string) | O | 1..N | Contains the list of the identifier(s) of the AIMLE Server(s) that have previously managed the AIMLE Client (to which the transferred context is associated). This list is maintained by the AIMLE Client. |  |
| servStatus | AimleServStatus | O | 0..1 | Contains the status of the AIML operations at the AIMLE Client. |  |
| servRes | FFS | O | 0..1 | Contains the result of the AIML operations performed by the AIMLE Client. |  |
| servApp | FFS | O | 0..1 | Contains the applicability information of the AIML operations performed by the AIMLE Client. |  |

Editor's Note:	The content of the servRes and the servApp attribute is FFS.

* * * * Next Changes * * * *

###### 6.1.1.6.2.3	Type: AimleServStatus

Table 6.1.1.6.2.2-1: Definition of type AimleServStatus

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| servOpStatus | AimleServOpStatus | M | 1 | Contains the status of the AIML operations at the AIMLE Client. |  |
| servOpCompPer | integer | O | 0..1 | Contains the percentage of completion of the AIML operations at the AIMLE Client. Minimum = 0. Maximum = 100. This attribute may be present only when the value of the "AimleServOpStatus" attribute is either "ACTIVE" or "PAUSED". |  |

* * * * Next Changes * * * *

###### 6.1.1.6.3.2	Simple data types

The simple data types defined in table 6.1.1.6.3.2-1 shall be supported.

Table 6.1.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| AimleClientId | string | Represents the unique identifier of the AIMLE Client. |  |

* * * * Next Changes * * * *

###### 6.1.1.6.3.3	Enumeration: AimleServOpStatus

The enumeration AimleServOpStatus represents the AIMLE service operation status information. It shall comply with the provisions defined in table 6.1.1.6.3.3-1.

Table 6.1.1.6.3.3-1: Enumeration AimleServOpStatus

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| ACTIVE | Indicates that the status of the AIML operations at the AIMLE Client is active. |  |
| PAUSED | Indicates that the status of the AIML operations at the AIMLE Client is paused. |  |
| COMPLETED | Indicates that the status of the AIML operations at the AIMLE Client is completed. |  |

* * * * Next Changes * * * *

# A.2	AIMLES_ContextTransfer API

openapi: 3.0.0

info:

title: AIMLE Context Transfer Service

version: 1.0.0-alpha.3

description: |

AIMLE Context Trnaser Information Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.482 v1.0.0; Service Enabler Architecture Layer for Verticals (SEAL); Artificial

Intelligence Machine Learning Enablement (AIMLE) Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.482/

servers:

- url: '{apiRoot}/aimles-ct/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

security:

- {}

- oAuth2ClientCredentials: []

paths:

/request-trans:

post:

summary: Request AIMLE context information transfer.

operationId: Transfer

tags:

- AIMLE Context Transfer Request

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/TransReq'

responses:

'204':

description: >

No Content. The AIMLE Context Transfer request is successfully received and

processed.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

TransReq:

description: >

Represents the AIMLE Context Transfer request.

type: object

properties:

clientId:

$ref: '#/components/schemas/AimleClientId'

currServerId:

type: string

prevServerIds:

type: array

items:

type: string

minItems: 1

servStatus:

$ref: '#/components/schemas/AimleServStatus'

required:

- clientId

AimleServStatus:

description: >

Represents the AIMLE service operations status related information.

type: object

properties:

servOpStatus:

$ref: '#/components/schemas/AimleServOpStatus'

servOpCompPer:

type: integer

minimum: 0

maximum: 100

required:

- servOpStatus

# SIMPLE DATA TYPES

#

AimleClientId:

description: Represents the unique identifier of a AIMLE Client.

type: string

#

# ENUMERATIONS

#

AimleServOpStatus:

anyOf:

- type: string

enum:

- ACTIVE

- PAUSED

- COMPLETED

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the AIMLE service operations status.

Possible values are:

- ACTIVE: Indicates that the status of the AIML operations at the AIMLE Client is active.

- PAUSED: Indicates that the status of the AIML operations at the AIMLE Client is paused.

- COMPLETED: Indicates that the status of the AIML operations at the AIMLE Client is

completed.

* * * * End of Changes * * * *