3GPP TSG-CT WG3 Meeting #143	C3-254184

Sophia-Antipolis, France, 13 - 17 October, 2025

Source:	Samsung

Title:	Pseudo-CR on defining the Open API description of SS_SAnUsage API

Spec:	3GPP TS 29.437 (V1.1.0)

Agenda item:	19.42

Document for:	Approval

1. Introduction

The stage 2 requirements for the new SS_SAnUsage API have been defined in clauses 8.5.4 of TS 23.437. The API data model and the related descriptions of the service operations of the API are specified in TS 29.437.

2. Reason for Change

With stable API data model, the OpenAPI descriptions needs to specified in TS 29.437.

3. Conclusions

N/A

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.437 V 1.1.0.

* * * First Change * * * *

## 5.1	Introduction

Table 5.1-1 summarizes the corresponding APIs defined for this specification.

Table 5.1-1: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | API Name | Annex |
| --- | --- | --- | --- | --- | --- |
| <service name> | <ref clause> | <short description as included in the OpenAPI file> | <file name> | <apiName in the URI> | <ref Annex> |
| SS_SAnUsage | 6.1.3 | Spatial Anchors Usage | TS29437_SS_SAnUsage.yaml | ssan-usg | A.X |
| SS_SmSmasRegistration | 6.2.5 | SMAS Registration Management | TS29437_SS_SmSmasRegistration.yaml | ssm-smasr | A.8 |

NOTE:	When 3GPP TS 29.122 [2] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 5, the service producer (i.e., SAn Server, SM Server) takes the role of the SCEF and the service consumer takes the role of the SCS/AS.

* * * Next Change * * * *

# A.X	SS_SAnUsage API

openapi: 3.0.0

info:

title: SAn Spatial Anchors Usage

version: 1.0.0-alpha.1

description: |

API for Spatial Anchors Usage.

©2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.437 V1.2.0; Service Enabler Architecture Layer for Verticals (SEAL);

Metaverse Enablement Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.437/

servers:

- url: "{apiRoot}/ssan-usg/v1"

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549.

security:

- {}

- oAuth2ClientCredentials: []

paths:

/subscriptions:

post:

summary: Create a new Spatial Anchors Usage Information Subscription.

operationId: CreateSpatialAnchorsUsageInfoSub

tags:

- Spatial Anchors Usage Information Subscriptions

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialAnchorsUsageInfoSub'

responses:

'201':

description: >

Created. The Spatial Anchors Usage Information Subscription is successfully created and a representation of the created Individual Spatial Anchors Usage Information Subscription resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialAnchorsUsageInfoSub'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'# STRUCTURED DATA TYPES

#

/subscriptions/{subscriptionId}:

parameters:

- name: subscriptionId

in: path

description: Represents the identifier of the Individual Spatial Anchors Usage Information Subscription resource.

required: true

schema:

type: string

get:

summary: Retrieve an existing Individual Spatial Anchors Usage Information Subscription resource.

operationId: GetIndSpatialAnchorsUsageInfoSub

tags:

- Individual Spatial Anchors Usage Information Subscription

responses:

'200':

description: >

OK. The requested Individual Spatial Anchors Usage Information Subscription resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialAnchorsUsageInfoSub'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Update an existing Individual Spatial Anchors Usage Information Subscription resource.

operationId: UpdateIndSpatialAnchorsUsageInfoSub

tags:

- Individual Spatial Anchors Usage Information Subscription

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialAnchorsUsageInfoSub'

responses:

'200':

description: >

OK. The Individual Spatial Anchors Usage Information Subscription resource is successfully updated and a representation of the updated resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialAnchorsUsageInfoSub'

'204':

description: >

No Content. The Individual Spatial Anchors Usage Information Subscription resource is successfully updated and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Modify an existing Individual Spatial Anchors Usage Information Subscription resource.

operationId: ModifyIndSpatialAnchorsUsageInfoSub

tags:

- Individual Spatial Anchors Usage Information Subscription

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialAnchorsUsageInfoSubPatch'

responses:

'200':

description: >

OK. The Individual Spatial Anchors Usage Information Subscription resource is successfully modified and a representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialAnchorsUsageInfoSub'

'204':

description: >

No Content. The Individual Spatial Anchors Usage Information Subscription resource is successfully modified and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Delete an existing Individual Spatial Anchors Usage Information Subscription resource.

operationId: DeleteIndSpatialAnchorsUsageInfoSub

tags:

- Individual Spatial Anchors Usage Information Subscription

responses:

'204':

description: >

No Content. The Individual Spatial Anchors Usage Information Subscription resource is successfully deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: "{tokenUrl}"

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

SpatialAnchorsUsageInfoSub:

description: >

Represents the spatial anchors usage information subscription.

type: object

properties:

eventSubs:

type: array

items:

$ref: '#/components/schemas/AnchorsUsageSub'

minitems: 1

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

report:

$ref: '#/components/SpatialAnchorsUsageInfoNotif'

reportReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/ReportingInfomation'

expTime:

$ref: 'TS29122_CommonData.yaml#/components/DateTime'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

-	eventSubs

-	notifUri

SpatialAnchorsUsageInfoSubPatch:

description: >

Represents the spatial anchors usage information subscription patch.

type: object

properties:

eventSubs:

type: array

items:

$ref: '#/components/schemas/AnchorsUsageSub'

minItems: 1

notifUri:

$ref: 'TS29437_SS_SmManagement.yaml#/components/schemas/Uri'

reportReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/ReportingInfomation'

expTime:

$ref: 'TS29122_CommonData.yaml#/components/DateTime'

AnchorsUsageSub:

description: >

Represent the subscription details related to spatial anchor usage.

type: object

properties:

event:

$ref: '#/components/schemas/AnchorUsageEvent'

evntFltr:

$ref: '#/components/schemas/AnchorsUsageEventFilter'

required:

-	event

-	evntFltr

AnchorsUsageEventFilter:

description: >

Represents the filter information for anchors usage event.

type: object

properties:

anchAnlFltr:

$ref: '#/components/schemas/AnchorsUsageFilter'

AnchorsUsageFilter:

description: >

Represents the spatial anchors usage filter information.

type: object

properties:

anchIds:

type: array

items:

$ref: 'TS29437_SS_SAnManagement.yaml#/components/schemas/SpatialAnchorId'

minItems: 1

anchLoc:

$ref: 'TS29122_MonitoringEvent.yaml#/components/schemas/LocationInfo'

area:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ServiceArea'

appId:

type: string

sampInt:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

anyOf:

- required: [anchIds]

- required: [anchLoc]

- required: [area]

- required: [appId]

- required: [sampInt]

SpatialAnchorsUsageInfoNotif:

description: >

Represents the spatial maps discovery response.

type: object

properties:

subId:

type: string

reports:

type: array

items:

$ref: '#/components/schemas/SpatialAnchorsUsageReport'

minItems: 1

required:

-	subId

-	reports

SpatialAnchorsUsageReport:

description: >

Represents the spatial anchors usage information notification.

type: object

properties:

event:

$ref: '#/components/schemas/AnchorUsageEvent'

anchUsgInf:

type: array

items:

$ref: '#/components/schemas/AnchorsUsageInfo'

minItems: 1

required:

-	event

AnchorsUsageInfo:

description: >

Represents the anchor usage details.

type: object

properties:

anchIds:

type: array

items:

$ref: 'TS29437_SS_SAnManagement.yaml#/components/schemas/SpatialAnchorId'

minItems: 1

discAcc:

type: Uinteger

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

serAcc:

type: Uinteger

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

anchDensty:

type: Uinteger

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

usrDensity:

type: Uinteger

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

required:

-	anchIds

-	discAcc

-	serAcc

-	anchDensity

-	usrDensity

SpatialAnchorUsageReportReq:

description: >

Represents the request for spatial anchor usage report.

type: object

properties:

anchUsg:

type: array

items:

$ref: '#/components/schemas/AnchorUsage'

minItems: 1

required:

-	anchUsg

AnchorUsage:

description: >

Represents the anchor usage information reported.

type: object

properties:

anchId:

$ref: 'TS29437_SS_SAnManagement.yaml#/components/schemas/SpatialAnchorId'

access:

type: Uinteger

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

repPer:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

required:

-	anchId

-	access

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

AnchorUsageEvent:

anyOf:

- type: string

enum:

- SPATIAL_ANCHOR_USAGE_INFORMATION

- type: string

description: >

This string provides forward-compatibility with future extensions to the

enumeration but is not used to encode content defined in the present version of

this API.

description: |

Represents the type of entity.

Possible values are:

- SPATIAL_ANCHOR_USAGE_INFORMATION: Indicates that the event is related to spatial anchors usage.

# Data types describing alternative data types or combinations of data types:

#

* * * * End of Changes * * * *