3GPP TSG-CT WG3 Meeting #143	C3-254188

# Sophia-Antipolis, FR, 13th – 17th October, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.522 | CR | 1724 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | More Application errors for AIoT API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-CT |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | AIOT_TARGETS_ERROR and INVALID_AGGR_TIME_INVERTAVAL application errors are defined for the Naiotf_AIoT API in TS 29.569, but not defined for NEF AIOT API which means the NEF cannot relay these error response to the AF if received from the AIOTF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add AIOT_TARGETS_ERROR and INVALID_AGGR_TIME_INVERTAVAL application errors to Application errors definition Table 5.45.7.3-1, and update the procedure description accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NEF cannot relay AIOT_TARGETS_ERROR and INVALID_AGGR_TIME_INVERTAVAL application errors to the AF if received from the AIOTF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.49.2, 4.4.49.3, 5.45.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not have any impact in the OpenAPI specification. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** 1st Change ***

4.4.49.2	Procedures for AIoT Inventory Management

This procedure is used by an AF to request to perform an AIoT Inventory operation at the NEF (see also clause 6.2.2 of 3GPP TS 23.369 [81]).

In order to perform an AIoT Inventory operation, the AF shall send an HTTP POST request to the NEF targeting the URI of the corresponsing custom operation (i.e., "InventoryRequest"), with the request body including the InventoryReq data structure.

Upon reception of this HTTP POST request message, the NEF shall:

-	check whether the AF is authorized perform this operation based on operator policies, local configuration information (e.g., SLA with the AF) and/or by interacting with the ADM as defined in clause 5.6 of 3GPP TS 23.369 [81];

-	if the AF is not authorized, reject the request with an HTTP "403 Forbidden" status code with the response body including the ProblemDetails data structure containing the "cause" attribute set to the "AF_NOT_AUTHORIZED" application error; and

-	if the AF is authorized:

-	determine the Target AIoT Area information based on the received External Target AIoT Area information;

-	follow the procedures defined in clause 6.2.2 of 3GPP TS 23.369 [81] by interacting with the selected AIOTF(s);

-	if the NEF receives from the AIOTF an error response including a "ProblemDetails" data structure with the "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable; and

-	if the NEF fails to process the AIoT Inventory request, reject the request with an HTTP "500 Internal Server Error" status code with the response body including the ProblemDetails data structure containing the "cause" attribute set to the "UNSPECIFIED_FAILURE" application error.

Upon successful response from the AIOTF(s) and successful processing of the request, the NEF shall return an HTTP "200 OK" status code to the AF, with the response body including AIoT Inventory related information within the InventoryResp data structure.

On failure or if the NEF receives an error code from the ADM and/or AIOTF(s), the NEF shall take proper error handling actions, as specified in clause 5.45.7, and respond to the AF with an appropriate error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

# *** 2nd Change ***

4.4.49.3	Procedures for AIoT Command Management

This procedure is used by an AF to request to perform an AIoT Command operation to the NEF (see also clause 6.2.3 of 3GPP TS 23.369 [81]).

In order to perform an AIoT Command operation, the AF shall send an HTTP POST request to the NEF targeting the URI of the corresponsing custom operation (i.e., "CommandRequest"), with the request body including the CommandReq data structure.

Upon reception of this HTTP POST request message, the NEF shall:

-	check whether the AF is authorized perform this operation based on operator policies, local configuration information (e.g., SLA with the AF) and/or by interacting with the ADM as defined in clause 5.6 of 3GPP TS 23.369 [81];

-	if the AF is not authorized, reject the request with an HTTP "403 Forbidden" status code with the response body including the ProblemDetails data structure containing the "cause" attribute set to the "AF_NOT_AUTHORIZED" application error; and

-	if the AF is authorized:

-	determine the Target AIoT Area information based on the received External Target AIoT Area information;

-	follow the procedures defined in clause 6.2.3 of 3GPP TS 23.369 [81] by interacting with the selected AIOTF(s);

-	if the NEF receives from the AIOTF an error response including a "ProblemDetails" data structure with the "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable; and

-	if the NEF fails to process the AIoT Command request, reject the request with an HTTP "500 Internal Server Error" status code with the response body including the ProblemDetails data structure containing the "cause" attribute set to the "UNSPECIFIED_FAILURE" application error.

Upon successful response from the AIOT and successful processing of the request, the NEF shall return an HTTP "200 OK" status code to the AF, with the response body including AIoT Command related information within the CommandResp data structure.

On failure or if the NEF receives an error code from the ADM and/or AIOTF(s), the NEF shall take proper error handling actions, as specified in clause 5.45.7, and respond to the AF with an appropriate error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

# *** 3rd Change ***

5.45.7.3	Application Errors

The application errors defined for the AIoT API are listed in table 5.45.7.3-1.

Table 5.45.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |
| AF_NOT_AUTHORIZED | 403 Forbidden | The request for AIoT services is rejected because the AF is not authorized for the requested AIoT Services. |  |
| AIOT_TARGETS_ERROR | 403 Forbidden | The request for AIoT services is rejected because the target(s) of the AIoT request (e.g., target AIoT device(s), filtering information) is/are not supported and/or not allowed. |  |
| INVALID_AGGR_TIME_INVERTAVAL | 403 Forbidden | The AIoT Inventory request is rejected because the provided aggregation time interval is invalid. |  |
| UNSPECIFIED_FAILURE | 500 Internal Server Error | The request for AIoT services is rejected because the NEF failed to process it. |  |

*** End of Changes ***