3GPP TSG-CT WG3 Meeting #143	C3-254226

Sophia-Antipolis, France, 13 – 17 October 2025	(Revision of C3-25xxxx))

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.522 | CR | 1731 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to iteration number handling in Nnef_VFLTraining |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-09-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 23.288, clause 6.2H.3 specifies the contents of ML Model VFL Training services for Vertical Federated Learning sent from the consumers of the ML Model training services as input parameters in Nnef_VFLTraining service. As part of this input information it is indicated that in training phase, VFL training iteration number must be sent from the consumer, however, in current Nnef_VFLTraining service the VFL iteration number is not included as information sent from the consumer, but only on the VflTrainingReport data type reused from Nnwdaf_VFLTraining_Subscribe in 29.520, that is, only available when sent by the VFL client in responses and notifications, but not from the service consumer. Hence it is needed to add the iteration number as part of the information sent by the consumers in the Nnef_VFLTraining_Subscribe. Clause numbers for 5.46.5.3 and 5.46.5.4 are not correct, they shall be 5.46.5.2 and 5.46.5.3 respectively. |  |  |  |  |  |  |  |  |
| Summary of change: |  | "iterationNum" attribute is added on VflTrainingSubs and VflTrainingSubsPatch data types. "iterationNum" attribute is added on VflTrainingSubsPatch data type. Clause and subclause numbers for 5.46.5.3 are replaced by 5.46.5.2 and Clause and subclause numbers for 5.46.5.4 are replaced by 5.46.5.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not aligned with stage 2 requirements on supporting the iteration number as part of the information received from the customer in Nnef_VFLTraining_Subscribe requests. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.50.2, 5.46.5.1, 5.46.5.2 (and subclauses), 5.46.5.3 (and subclauses), A.44 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards compatible correction in the OpenAPI file of TS29522_Nnef_VFLTraining.yaml API |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** 1st Change ***

#### 4.4.50.2	Creation of new VFL Training subscription

In order to create a new "Individual VFL Training Subscription" resource, the AF shall initiate an HTTP POST request to the NEF for the "VFL Training Subscriptions" resource. The HTTP POST request message body shall include the VflTrainingSubs data structure with contents as described in clause 5.46.5.3.2.

Upon receipt of the corresponding HTTP POST message, the NEF authorizes the request and upon successful authorization, the NEF shall determine the internal identifier of the targeted NWDAF based on the provided external NWDAF identifier and interact with the NWDAF by using the Nnwdaf_VFLTraining_Subscribe service operation as defined in 3GPP TS 29.520 [27]. If the request is accepted by the NWDAF, the NEF shall create a new "Individual VFL Training Subscription" resource and send an HTTP "201 Created" response with the VflTrainingSubs data structure including the contents of the created "Individual VFL Training Subscription" resource in the response body and a Location header field containing the URI of the created "Individual VFL Training Subscription" resource. If the NEF receives an error response from the NWDAF, the NEF shall not create the resource and shall respond to the AF with a proper error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, if applicable.

# *** Next Change ***

#### 5.46.5.1	General

This clause specifies the application data model supported by the VFLTraining API. Table 5.46.5.1-1 specifies the data types defined for the VFLTraining API.

Table 5.46.5.1-1: VFLTraining API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| VflTrainingSubs | 5.46.5.3.2 | Represents VFL training subscription information. |  |
| VflTrainingSubsPatch | 5.46.5.3.3 | Represents modifications to VFL training subscription information. |  |

Table 5.46.5.1-2 specifies data types re-used by the VFLTraining API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the VFLTraining API.

Table 5.46.5.1-2: VFLTraining API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ReportingInformation | 3GPP TS 29.523 [22] | Describes reporting requirements. |  |
| SupportedFeatures | 3GPP TS 29.571 [8] | Represents the list of supported feature(s) and is used to negotiate the applicability of the optional features. |  |
| Uri | 3GPP TS 29.571 [8] | Represents a URI. |  |
| VflTrainingNotify | 3GPP TS 29.520 [27] | Represents VFL training notification information. |  |
| VflTrainingSub | 3GPP TS 29.520 [27] | Represents VFL training subscription information for a specific analytics ID. |  |
| Uinteger | 3GPP TS 29.571 [8] | Unsigned Integer, i.e. only value 0 and integers above 0 are permissible. |  |

# *** Next Change ***

#### 5.46.5.2	Structured data types

# *** Next Change ***

##### 5.46.5.2.1	Introduction

This clause defines the structured data types to be used in resource representations.

# *** Next Change ***

##### 5.46.5.2.2	Type: VflTrainingSubs

Table 5.46.5.2.2-1: Definition of type VflTrainingSubs

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifUri | Uri | M | 1 | Notification target address. |  |
| notifCorrId | string | M | 1 | Notification correlation identifier. |  |
| vflTrainSubs | array(VflTrainingSub) | M | 1..N | Each entry contains VFL training subscription request for a specific analytics ID. |  |
| reportingReqs | ReportingInformation | O | 0..1 | Reporting requirement information of the subscription. If omitted, the default values within the ReportingInformation data type apply. |  |
| iterationNum | Uinteger | O | 0..1 | Indicates the iteration number of the training in a multi-round training process. |  |
| extNwdafId | string | M | 1 | Contains the external identifier of the NWDAF which is being requested to act as a VFL client. |  |
| trainReports | array(VflTrainingNotify) | O | 1..N | Contains the VFL Training related even(s) report(s). This attribute may be present only if immediate reporting was requested via the "reportingReqs" attribute. |  |
| suppFeat | SupportedFeatures | C | 0..1 | List of supported features used as described in clause 5.46.6. This attribute shall be included when feature negotiation needs to take place. |  |

# *** Next Change ***

##### 5.46.5.2.3	Type: VflTrainingSubsPatch

Table 5.46.5.2.3-1: Definition of type VflTrainingSubsPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| vflTrainSubs | array(VflTrainingSub) | O | 1..N | Contains modified information related to VFL training subscription request for a specific analytics ID. |  |
| notifUri | Uri | O | 0..1 | Contains the updated URI via which VFL Training related notifications shall be delivered. |  |
| notifCorrId | String | O | 0..1 | Contains the updated Notification Correlation Identifier. |  |
| iterationNum | Uinteger | O | 0..1 | Indicates the iteration number of the training in a multi-round training process. |  |
| reportingReqs | ReportingInformation | O | 0..1 | Contains the reporting requirements applicable for VFL Training related reporting. |  |

# *** Next Change ***

#### 5.46.5.3	Simple data types and enumerations

# *** Next Change ***

##### 5.46.5.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

# *** Next Change ***

##### 5.46.5.3.2	Simple data types

The simple data types defined in table 5.46.5.3.2-1 shall be supported.

Table 5.46.5.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

# *** Next Change ***

# A.44	VFLTraining API

openapi: 3.0.0

info:

title: 3gpp-vfl-training

version: 1.0.0-alpha.1

description: |

API for VFL Training.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.522 V19.4.0; 5G System; Network Exposure Function Northbound APIs.

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.522/'

servers:

- url: '{apiRoot}/3gpp-vfl-training/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122.

security:

- {}

- oAuth2ClientCredentials: []

paths:

/{afId}/subscriptions:

parameters:

- name: afId

in: path

description: Represents the identifier of the AF

required: true

schema:

type: string

get:

summary: Retrieve all the active VFL Training Subscriptions.

operationId: GetVFLTrainingSubscriptions

tags:

- VFL Training Subscriptions (Collection)

responses:

'200':

description: >

OK. All the Individual VFL Training Subscription resource(s) managed

by the NEF are returned.

If there are no active Individual VFL Training Subscription resources

at the NEF, an empty array is returned.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/VflTrainingSubs'

minItems: 0

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

post:

summary: Create a new Individual VFL Training Subscription.

operationId: CreateVFLTrainingSubscription

tags:

- VFL Training Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/VflTrainingSubs'

responses:

'201':

description: >

Created. A representation of the created Individual VFL Training Subscription

resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/VflTrainingSubs'

headers:

Location:

description: >

Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

VFLTrainingNotification:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: 'TS29520_Nnwdaf_VFLTraining.yaml#/components/schemas/VflTrainingNotify'

responses:

'204':

description: >

No Content. The notification is successfully received and acknowledged.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/{afId}/subscriptions/{subId}:

parameters:

- name: afId

in: path

description: Represents the identifier of the AF.

required: true

schema:

type: string

- name: subId

in: path

description: Represents the identifier of the Individual VFL Training Subscription resource.

required: true

schema:

type: string

get:

summary: Retrieve an existing Individual VFL Training Subscription resource.

operationId: GetVFLTrainingSubscription

tags:

- Individual VFL Training Subscription (Document)

responses:

'200':

description: >

OK. The requested Individual VFL Training Subscription resource is

successfully returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/VflTrainingSubs'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Request the update of an existing Individual VFL Training Subscription resource.

operationId: UpdateVFLTrainingSubscription

tags:

- Individual VFL Training Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/VflTrainingSubs'

responses:

'200':

description: >

OK. The Individual VFL Training Subscription resource is successfully updated

and a representation of the updated resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/VflTrainingSubs'

'204':

description: >

No Content. The Individual VFL Training Subscription resource is successfully

updated and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Request modification of an existing Individual VFL Training Subscription resource.

operationId: ModifyVFLTrainingSubscription

tags:

- Individual VFL Training Subscription (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/VflTrainingSubsPatch'

responses:

'200':

description: >

OK. The Individual VFL Training Subscription resource is successfully modified

and a representation of the updated resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/VflTrainingSubs'

'204':

description: >

No Content. The Individual VFL Training Subscription resource is successfully

modified and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Request deletion of an existing Individual VFL Training Subscription resource.

operationId: DeleteVFLTrainingSubscription

tags:

- Individual VFL Training Subscription (Document)

responses:

'204':

description: >

No Content. The Individual VFL Training Subscription resource is

successfully deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

VflTrainingSubs:

description: Represents VFL Training subscription.

type: object

properties:

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

notifCorrId:

type: string

description: Notification correlation identifier.

vflTrainSubs:

type: array

items:

$ref: 'TS29520_Nnwdaf_VFLTraining.yaml#/components/schemas/VflTrainingSub'

minItems: 1

extNwdafId:

type: string

description: >

Contains the external identifier of the NWDAF that is being requested to act

as VFL client.

reportingReqs:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

iterationNum:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- notifUri

- notifCorrId

- vflTrainSubs

- extNwdafId

VflTrainingSubsPatch:

description: Represents contents for modifying a VFL Training subscription.

type: object

properties:

vflTrainSubs:

type: array

items:

$ref: 'TS29520_Nnwdaf_VFLTraining.yaml#/components/schemas/VflTrainingSub'

minItems: 1

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

notifCorrId:

type: string

description: Notification correlation identifier.

iterationNum:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

reportingReqs:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

# *** End of Changes ***