3GPP TSG-CT WG3 Meeting #143	C3-254238

# Sophia-Antipolis, France, 13 - 17 October 2025

Source:	Ericsson

Title:	Pseudo-CR on corrections of Data Model in AIMLES_AIMLEServiceOperationsManagement API

Spec:	3GPP TS 29.482 V1.1.0

Agenda item:	19.41

Document for:	Approval

1. Introduction

TS 29.482 AIMLES_AIMLEServiceOperationsManagement API data model is incomplete, has some incorrections, and in the request is missing AIML service operation mode status reporting.

2. Reason for Change

The current document has some incorrections and an incomplete data model for AIMLES_AIMLEServiceOperationsManagement API.

AIMLES_AIMLEServiceOperationsManagement API also misses the AIML service operation mode status reporting in the AimlServOperReq data type, which is present in stage 2 document (TS 23.482).

The addition of service operation mode status according to its description in stage 2: “Indicates the reporting configuration of the AIML service operation status like periodic (e.g. time interval) or event based (e.g., transition of AIML service operation from stop to start)”, also requires adding a Service Operation Event data type.

3. Conclusions

This pCR proposes changes to complete and correct the data model of AIMLES_AIMLEServiceOperationsManagement API.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.482 V1.1.0.

# *** First Change ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 29.122: "T8 reference point for Northbound Application Programming Interfaces (APIs)".

[3]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[4]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[5]	3GPP TR 21.900: "Technical Specification Group working methods".

[6]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs; Stage 2".

[7]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs; Stage 3".

[8]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs".

[9]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[10]	IETF RFC 9113: "HTTP/2".

[11]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[12]	IETF RFC 9457: "Problem Details for HTTP APIs".

[13]	3GPP TS 23.482: "Functional architecture and information flows for AIML Enablement Service".

[14]	3GPP TS 29.549: "Service Enabler Architecture Layer for Verticals (SEAL); Application Programming Interface (API) specification; Stage 3".

[15]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[16]	3GPP TS 24.560: "Artificial Intelligence Machine Learning (AIML) Services - Service Enabler Architecture Layer for Verticals (SEAL); Protocol Specification; Stage 3".

[17]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[18]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3".

[19]	3GPP TS 29.558: "Enabling Edge Applications; Application Programming Interface (API) specification; Stage 3".

[A]	3GPP TS29.520: "5G System; Network Data Analytics Services; Stage 3".

# *** Next Change ***

##### 6.1.4.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.4.6.1-1 specifies the data types defined for the AIMLES_AIMLEServiceOperationsManagement API.

Table 6.1.4.6.1-1: AIMLES_AIMLEServiceOperationsManagement API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AimleConfigMode | Clause 6.1.4.6.2.7 | Represent the AIMLE service operation configuration mode information. |  |
| AimleOperAssist | Clause 6.1.4.6.2.6 | Represent the AIMLE service operation assistance information. |  |
| AimleOperId | Clause 6.1.4.6.2.4 | Represents the AIMLE service operation identifier. |  |
| AimleOperInfo | Clause 6.1.4.6.2.5 | Represents the AIMLE service operation information. |  |
| AimleOperMode | Clause 6.1.4.6.3.3 | Represents the AIMLE service operation mode. |  |
| AimlServOperReq | Clause 6.1.4.6.2.2 | Represents the AIMLE service operations control and management request. |  |
| AimlServOperResp | Clause 6.1.4.6.2.3 | Represents the AIMLE service operations control and management response. |  |
| MeasurementData | Clause 6.1.4.6.2.8 | Represent the measurement data for the AIMLE service operation. |  |
| ServiceOperEvent | Clause 6.1.4.6.3.4 | Represents the event that triggers a AIML service status report. |  |
| StatReport | Clause 6.1.4.6.2.9 | Represents the status report mode for the AIML service operation. |  |

Table 6.1.4.6.1-2 specifies data types re-used by the AIMLES_AIMLEServiceOperationsManagement API from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the AIMLES_AIMLEServiceOperationsManagement API.

Table 6.1.4.6.1-2: AIMLES_AIMLEServiceOperationsManagement API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AimleServStatus | Clause 6.1.1.6.2.3 | Represents the AIMLE service status information. |  |
| DateTime | 3GPP TS 29.122 [2] | Used to indicate a timestamp. |  |
| DurationSec | 3GPP TS 29.122 [2] | Unsigned integer identifying a period of time in units of seconds. |  |
| MatchingDirection | 3GPP TS 29.520 [A] | Defines the matching direction when crossing a threshold. |  |
| SupportedFeatures | 3GPP TS 29.571 [15] | Represents the supported features, used to negotiate the supported optional features of the API. |  |
| Uinteger | 3GPP TS 29.571 [15] | Represents an unsigned Integer. |  |
| Uri | 3GPP TS 29.122 [2] | Used to indicate the notification URI. |  |

# *** Next Change ***

6.1.4.6.2.2	Type: AimlServOperReq

Table 6.1.4.6.2.2-1: Definition of type AimlServOperReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valSvcId | string | O | 0..1 | Contains the VAL service identifier. |  |
| clients | array(string) | C | 0..N | Contains the list of AIMLE Client IDs. (NOTE) |  |
| setId | string | C | 0..1 | Contains the AIMLE Client set ID. (NOTE) |  |
| operId | AimleOperId | O | 0..1 | Contains AIMLE operation ID. |  |
| operInfo | AimleOperInfo | O | 0..1 | Contains AIMLE operation information. |  |
| operMode | AimleOperMode | M | 1 | Contains AIMLE operation mode, |  |
| confModes | array(AimleConfigMode) | O | 1..N | Contains AIMLE operation configuration modes. |  |
| operModeStatReport | StatReport | O | 0..1 | Contains AIML operation status reporting modes. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Represents the supported features. This attribute shall be provided when feature negotiation needs to take place. |  |
| NOTE: Only one of these attributes shall be present. |  |  |  |  |  |

# *** Next Change ***

6.1.4.6.2.3	Type: AimlServOperResp

Table 6.1.4.6.2.3-1: Definition of type AimlServOperResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valSvcId | string | O | 0..1 | Contains the VAL service identifier. |  |
| operId | AimleOperId | M | 1 | Contains AIMLE operation ID. |  |
| operMode | AimleOperMode | M | 1 | Contains AIMLE operation status. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Represents the supported features. This attribute shall be provided when feature negotiation needs to take place. |  |

# *** Next Change ***

6.1.4.6.2.9	Type: StatReport

Table 6.1.4.6.2.9-1: Definition of type StatReport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| period | DurationSec | C | 0..1 | Contains the time interval between each AIMLE status report. (NOTE) |  |
| servOperEvent | ServiceOperEvent | C | 0..1 | Contains the event that triggers a AIMLE status report. (NOTE) |  |
| NOTE: At least one of these attributes shall be included. |  |  |  |  |  |

# *** Next Change ***

6.1.4.6.3.4	Enumeration: ServiceOperEvent

Table 6.1.4.6.3.4-1: Enumeration ServiceOperEvent

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| OPERATION_TRANSITION | Indicates the event is an AIMLE service operation transition. |  |

# *** Next Change ***

# A.8	AIMLES_AIMLEServiceOperationsManagement API

openapi: 3.0.0

info:

title: AIMLE Service Operations Control and Management Service

version: 1.0.0-alpha.1

description: |

AIMLE Service Operations Management Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.482 v1.1.0; Artificial Intelligence Machine Learning Enablement

(AIMLE) Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.482/

servers:

- url: '{apiRoot}/aimles-ct/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122

security:

- {}

- oAuth2ClientCredentials: []

paths:

/request:

post:

summary: Request AIMLE service operations control and management.

operationId: Request

tags:

- AIMLE Service Operation Request

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AimlServOperReq'

responses:

'200':

description: >

The AIMLE service operations control and management request is successfully

received and processed.

content:

application/json:

schema:

$ref: '#/components/schemas/AimlServOperResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

AimlServOperReq:

description: >

Represents the AIMLE service operations control and management request.

type: object

properties:

valSvcId:

type: string

clients:

type: array

items:

type: string

minItems: 1

setId:

type: string

operId:

$ref: '#/components/schemas/AimleOperId'

operInfo:

$ref: '#/components/schemas/AimleOperInfo'

operMode:

$ref: '#/components/schemas/AimleOperMode'

confModes:

type: array

items:

$ref: '#/components/schemas/AimleConfigMode'

minItems: 1

operModeStatReport:

$ref: '#/components/schemas/StatReport'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- operMode

AimlServOperResp:

description: >

Represents the AIMLE service operations control and management response.

type: object

properties:

valSvcId:

type: string

operId:

$ref: '#/components/schemas/AimleOperId'

operMode:

$ref: '#/components/schemas/AimleOperMode'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- operId

- operMode

AimleOperId:

description: >

Represents the AIMLE service operation identifier.

type: object

properties:

trainingId:

type: string

taskId:

type: string

oneOf:

- required: [trainingId]

- required: [taskId]

AimleOperInfo:

description: >

Represents the AIMLE service operation information.

type: object

properties:

container:

type: string

fetchUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

agrUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

assitInfo:

$ref: '#/components/schemas/AimleOperAssist'

required:

- container

- fetchUri

- agrUri

- assitInfo

AimleOperAssist:

description: >

Represent the AIMLE service operation assistance information.

type: object

properties:

maxConvTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

required:

- maxConvTime

AimleConfigMode:

description: >

Represent the AIMLE service operation configuration mode information.

type: object

properties:

mode:

$ref: '#/components/schemas/AimleOperMode'

measThrValues:

$ref: '#/components/schemas/MeasurementData'

thrDirection:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/MatchingDirection'

required:

- mode

- measThrValues

- thrDirection

MeasurementData:

description: Represent the measurement data for the AIMLE service operation.

type: object

properties:

latency:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

time:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

accuracy:

type: integer

minimum: 0

maximum: 100

StatReport:

description: Contains AIML operation status reporting modes.

type: object

properties:

period:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

servOperEvent:

$ref: '#/components/schemas/ServiceOperEvent'

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

AimleOperMode:

anyOf:

- type: string

enum:

- START

- STOP

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the AIMLE service operation mode.

Possible values are:

- START: Indicates the AIMLE service operation mode is start.

- STOP: Indicates the AIMLE service operation mode is stop.

ServiceOperEvent:

anyOf:

- type: string

enum:

- OPERATION_TRANSITION

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents AIMLE event types.

Possible values are:

- OPERATION_TRANSITION: Indicates the event is an AIMLE service operation transition.

# *** End of Changes ***