3GPP TSG-CT WG3 Meeting #143	C3-254305

# Sophia-Antipolis, FR, 13 - 17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.222 | CR | 0444 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Group information in CAPIF_Security_API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 06-10-2025 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | When API invoker at UE needs to access the service API for other UE's resources hosted in the network (e.g., location), it needs to obtain API authorization. The API authorization request needs to include the group identifier whose resources are to be accessed. The group identifier has not been added in CAPIF security API as Group Resource Owner (GRO) authorization was not defined in SA3. Based on the SID proposal (S3-252966) by SA3, it has been decided to implement Group ID authentication as part of the release 20. Therefore, SA6 has added the following Note. “NOTE: The details of the procedure to obtain resource owner’s authorization information from GRO are out the scope of this release of the specification.” Therefore, group id and other related information needs to be added in the AccessTokenReq of CAPIF_Security_API. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the CAPIF_Security_API with group information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 and stage 3 are not aligned. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.4.2.6, 8.5.6, A.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible changes to the following OpenAPI. - TS29222_CAPIF_Security_API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

##### 8.5.4.2.6	Type: AccessTokenReq

Table 8.5.4.2.6-1: Definition of type AccessTokenReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| grant_type | string | M | 1 | This attribute shall contain the grant type as "client_credentials", or when the "RNAA" feature is supported, either "client_credentials" or "authorization_code". (NOTE 3, NOTE 4) |  |
| client_id | string | M | 1 | This attribute shall contain the API invoker Identifier. (NOTE 3) |  |
| resOwnerId | ResOwnerId | O | 0..1 | Contains the identifier of the resource owner. This attribute shall be present only when the access token request is used for RNAA. | RNAA |
| client_secret | string | O | 0..1 | This attribute when present shall contain the onboarding secret which is got during API invoker onboarding. (NOTE 3) |  |
| groupId | string | O | 0..1 | Group identifier information. | CAPIF_Ext2 |
| scope | string | O | 0..1 | Contains the requested OAuth2 scope.. When neither the "CAPIF_Ext1" feature nor the "CAPIF_Ext2" feature are supported, it takes the following format: 3gpp#aefId1:apiName1,apiName2,…apiNameX;aefId2:apiName1,apiName2,…apiNameY;…aefIdN:apiName1,apiName2,…apiNameZ Using delimiter "#" after the discriminator "3gpp", ":" after AEF identifier, "," between API names and ";" between the last API name of the previous AEF identifier and the next AEF identifier. (NOTE 2) Example: '3gpp#aef-jiangsu-nanjing:3gpp-monitoring-event,3gpp-as-session-with-qos;aef-zhejiang-hangzhou:3gpp-cp-parameter-provisioning,3gpp-pfd-management' When the "CAPIF_Ext1" feature is supported and the "CAPIF_Ext2" feature is not supported, it takes the following format: '3gpp#aefId1:apiName1:scopeLevelType.scopeLevelValue:scopeLevelType.scopeLevelValue:…:scopeLevelType.scopeLevelValue,apiName2:scopeLevelType.scopeLevelValue:scopeLevelType.scopeLevelValue:…:scopeLevelType.scopeLevelValue,…apiNameX:scopeLevelType.scopeLevelValue:scopeLevelType.scopeLevelValue:…:scopeLevelType.scopeLevelValue;…aefIdN:apiName1:scopeLevelType.scopeLevelValue:scopeLevelType.scopeLevelValue:…:scopeLevelType.scopeLevelValue,apiName2:scopeLevelType.scopeLevelValue:scopeLevelType.scopeLevelValue:…:scopeLevelType.scopeLevelValue,…apiNameZ:scopeLevelType.scopeLevelValue:scopeLevelType.scopeLevelValue:…:scopeLevelType.scopeLevelValue' Example 1: '3gpp#aef1:3gpp-monitoring-event:res.subscriptions,3gpp-as-session-with-qos :res.subscriptions:op.create;aef-zhejiang-hangzhou:3gpp-cp-parameter-provisioning,3gpp-pfd-management:res.transactions:op.read' Example 2: '3gpp#aef1: 3gpp-time-sync:res.subscriptions:res.configurations:op.update,3gpp-mbs-session:res.mbs-sessions:res.subscriptions:op.create' With the following definitions: - The delimiter "#" shall be used only after the discriminator "3gpp" at the beginning of the scope field. - The delimiter ":" shall be used after the AEF identifier, after the API name and after each scope level. - The delimiter "," shall be used between API names of a certain AEF. - The delimiter ";" shall be used between the last API name of the one AEF section and the next AEF section. - Within the "aefId" field, the "apiName" field, the "scopeLevelType" field and "scopeLevelValue" field, the above defined delimiters are prohibited. - The "scopeLevelType" field shall be set to either "res" (for resource-level access control) or "op" (for operation-level access control). When the "CAPIF_Ext2" feature is supported and the "CAPIF_Ext1" feature is not supported, the content of this attribute shall be set to a list of space-delimited strings, as defined in clause 3.3 of IETF RFC 6749 [23]. |  |
| authCode | string | C | 0..1 | Contains the authorization code. This attribute shall be included only when the access token request is used for RNAA and the OAuth "authorization code" grant type is used. | RNAA |
| redirect_uri | string | O | 0..1 | Contains the redirection URI that was used to obtain the authorization code provided within the "authCode" attribute. This attribute may be included only when the access token request is used for RNAA and the OAuth "authorization code" grant type is used. (NOTE 3) | RNAA |
| NOTE 1: This data structure shall not be treated as a JSON object. It shall be treated as a key, value pair data structure to be encoded using x-www-urlencoded format as specified in clause 17.13.4.1 of W3C HTML 4.01 Specification [22]. NOTE 2: The scope may contain more space-delimited strings which further add additional access ranges to the scope, the definition of those additional strings is out of the scope of the present document. NOTE 3: The "grant_type", "client_id", "client_secret" and "redirect_uri" attributes do not follow the related naming convention defined in clause 7.2.1. These attributes are however kept as currently defined in this specification in order to keep them aligned with corresponding claims defined in IETF RFC 6749 [23] and for backward compatibility considerations. NOTE 4: The enumeration value "client_credentials" or "authorization_code" of the "grant_type" attribute does not follow the related naming convention defined in clause 7.2.1. This enumeration is however kept as currently defined in this specification for backward compatibility considerations. |  |  |  |  |  |

* * * Second Change * * *

### 8.5.6	Feature negotiation

General feature negotiation procedures are defined in clause 7.8. Table 8.5.6-1 lists the supported features for CAPIF_Security_API.

Table 8.5.6-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | Notification_test_event | Testing of notification connection is supported according to clause 7.6. |
| 2 | Notification_websocket | The delivery of notifications over Websocket is supported according to clause 7.6. This feature requires that the Notification_test_event feature is also supported. |
| 3 | SecurityInfoPerAPI | Indicates the support of negotiating and obtaining service API security method information per API. |
| 4 | RNAA | Indicates the support of the RNAA functionality. This feature enables the following functionalities: - Support the OAuth grant types for RNAA. - Support to convey the authorization code in access token requests to support the "authorization code" grant type for RNAA. - Support to communicate the resource owner ID for RNAA access token requests/responses. - Support to communicate the new cause codes for AEF authorization revocation. |
| 5 | CAPIF_Ext1 | Indicates the support of the enhancements for CAPIF functionality. Within this feature, the following enhancements are covered: - Support to communicate the access token during revoking resource owner authorization. - Support finer-granularity API access control. |
| 6 | CAPIF_Ext2 | Indicates the first set of enhancements to support the Rel-19 enhancements to the CAPIF functionalities. This feature enables the following functionalities: - Support generic string-based OAuth scope encoding based on clause 3.3 of IETF RFC 6749 [23]. - Support for UE-deployed API invoker accessing other UEs’ resources of a group |

* * * Third Change * * *

# A.6	CAPIF_Security_API

openapi: 3.0.0

info:

title: CAPIF_Security_API

description: |

API for CAPIF security management.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.4.0-alpha.3"

externalDocs:

description: 3GPP TS 29.222 V19.4.0 Common API Framework for 3GPP Northbound APIs

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.222/

servers:

- url: '{apiRoot}/capif-security/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 7.5 of 3GPP TS 29.222.

paths:

/trustedInvokers/{apiInvokerId}:

get:

parameters:

- name: apiInvokerId

in: path

description: Identifier of an individual API invoker

required: true

schema:

type: string

- name: authenticationInfo

in: query

description: >

When set to 'true', it indicates the CAPIF core function to send the

authentication information of the API invoker. Set to false or omitted otherwise.

schema:

type: boolean

- name: authorizationInfo

in: query

description: >

When set to 'true', it indicates the CAPIF core function to send the

authorization information of the API invoker. Set to false or omitted otherwise.

schema:

type: boolean

responses:

'200':

description: >

The security related information of the API Invoker based on the request

from the API exposing function.

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceSecurity'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'414':

$ref: 'TS29122_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

parameters:

- name: apiInvokerId

in: path

description: Identifier of an individual API invoker

required: true

schema:

type: string

requestBody:

description: create a security context for an API invoker

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceSecurity'

callbacks:

notificationDestination:

'{$request.body#/notificationDestination}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SecurityNotification'

responses:

'204':

description: No Content (successful notification)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

responses:

'201':

description: Successful created.

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceSecurity'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure

{apiRoot}/capif-security/v1/trustedInvokers/{apiInvokerId}

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'414':

$ref: 'TS29122_CommonData.yaml#/components/responses/414'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

parameters:

- name: apiInvokerId

in: path

description: Identifier of an individual API invoker

required: true

schema:

type: string

responses:

'204':

description: No Content (Successful deletion of the existing subscription)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/trustedInvokers/{apiInvokerId}/update:

post:

parameters:

- name: apiInvokerId

in: path

description: Identifier of an individual API invoker

required: true

schema:

type: string

requestBody:

description: Update the security context (e.g. re-negotiate the security methods).

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceSecurity'

responses:

'200':

description: Successful updated.

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceSecurity'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/trustedInvokers/{apiInvokerId}/delete:

post:

parameters:

- name: apiInvokerId

in: path

description: Identifier of an individual API invoker

required: true

schema:

type: string

requestBody:

description: Revoke the authorization of the API invoker for APIs.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SecurityNotification'

responses:

'204':

description: Successful revoked.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/securities/{securityId}/token:

post:

parameters:

- name: securityId

in: path

description: Identifier of an individual API invoker

required: true

schema:

type: string

requestBody:

required: true

content:

application/x-www-form-urlencoded:

schema:

$ref: '#/components/schemas/AccessTokenReq'

responses:

'200':

description: Successful Access Token Request

content:

application/json:

schema:

$ref: '#/components/schemas/AccessTokenRsp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

description: Error in the Access Token Request

content:

application/json:

schema:

$ref: '#/components/schemas/AccessTokenErr'

'401':

description: Unauthorized

content:

application/json:

schema:

$ref: '#/components/schemas/AccessTokenErr'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

schemas:

ServiceSecurity:

type: object

description: >

Represents the details of the security method for each service API interface.

When included by the API invoker, it indicates the preferred method of security.

When included by the CAPIF core function, it indicates the security method to be

used for the service API interface.

properties:

securityInfo:

type: array

items:

$ref: '#/components/schemas/SecurityInformation'

minimum: 1

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

requestTestNotification:

type: boolean

description: >

Set to true by API invoker to request the CAPIF core function to send a

test notification as defined in in clause 7.6. Set to false or omitted otherwise.

websockNotifConfig:

$ref: 'TS29122_CommonData.yaml#/components/schemas/WebsockNotifConfig'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- securityInfo

- notificationDestination

SecurityInformation:

type: object

description: Represents the interface details and the security method.

properties:

interfaceDetails:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/InterfaceDescription'

aefId:

type: string

description: Identifier of the API exposing function

apiId:

type: string

description: API identifier

prefSecurityMethods:

type: array

items:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/SecurityMethod'

minItems: 1

description: Security methods preferred by the API invoker for the API interface.

selSecurityMethod:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/SecurityMethod'

authenticationInfo:

type: string

description: Authentication related information

authorizationInfo:

type: string

description: Authorization related information

grantType:

type: array

items:

$ref: '#/components/schemas/OAuthGrantType'

minItems: 1

required:

- prefSecurityMethods

oneOf:

- required: [interfaceDetails]

- required: [aefId]

SecurityNotification:

type: object

description: Represents the revoked authorization notification details.

properties:

apiInvokerId:

type: string

description: String identifying the API invoker assigned by the CAPIF core function.

aefId:

type: string

description: String identifying the AEF.

apiIds:

type: array

items:

type: string

minItems: 1

description: Identifier of the service API

access_token:

type: string

description: >

JWS Compact Serialized representation of JWS signed JSON object (AccessTokenClaims).

cause:

$ref: '#/components/schemas/Cause'

required:

- apiInvokerId

- apiIds

- cause

AccessTokenReq:

format: x-www-form-urlencoded

description: Represents the access token request information.

properties:

grant_type:

type: string

enum:

- client_credentials

- authorization_code

client_id:

type: string

groupId:

type: string

resOwnerId:

$ref: '#/components/schemas/ResOwnerId'

client_secret:

type: string

scope:

type: string

authCode:

type: string

redirect_uri:

type: string

required:

- grant_type

- client_id

AccessTokenRsp:

type: object

description: Represents the access token response information.

properties:

access_token:

type: string

description: >

JWS Compact Serialized representation of JWS signed JSON object (AccessTokenClaims)

token_type:

type: string

enum:

- Bearer

expires_in:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

scope:

type: string

required:

- access_token

- token_type

- expires_in

AccessTokenClaims:

type: object

description: Represents the claims data structure for the access token.

properties:

iss:

type: string

scope:

type: string

exp:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

resOwnerId:

$ref: '#/components/schemas/ResOwnerId'

required:

- iss

- scope

- exp

ResOwnerId:

type: object

description: >

Represents the identifier of the resource owner.

properties:

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

anyOf:

- required: [gpsi]

AccessTokenErr:

type: object

description: Represents an error in the access token request.

properties:

error:

type: string

enum:

- invalid_request

- invalid_client

- invalid_grant

- unauthorized_client

- unsupported_grant_type

- invalid_scope

error_description:

type: string

error_uri:

type: string

required:

- error

Cause:

anyOf:

- type: string

enum:

- OVERLIMIT_USAGE

- UNEXPECTED_REASON

- AUTHORIZATION_ISSUE

- OTHER_REASON

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Indicates the cause for revoking the API invoker's authorization to the service API.

Possible values are:

- OVERLIMIT_USAGE:

The revocation of the authorization of the API invoker is due to the overlimit

usage of the service API

- UNEXPECTED_REASON:

The revocation of the authorization of the API invoker is due to unexpected reason.

- AUTHORIZATION_ISSUE:

The revocation of the authorization of the API invoker is due to API Invoker

not being authorized anymore by the API Provider.

- OTHER_REASON:

The revocation of the authorization of the API invoker is due to other reason.

OAuthGrantType:

anyOf:

- type: string

enum:

- CLIENT_CREDENTIALS

- AUTHORIZATION_CODE

- AUTHORIZATION_CODE_WITH_PKCE

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Indicates the supported authorization flow (e.g. client credentials flow, authorization code         flow, etc.) to the API invoker.

Possible values are:

- CLIENT_CREDENTIALS: Indicate that the grant type is is client credentials flow.

- AUTHORIZATION_CODE: Indicate that the grant type is authorization code.

- AUTHORIZATION_CODE_WITH_PKCE: Indicate that the grant type is authorization code with PKCE.

* * * End of Changes * * *