3GPP TSG-CT WG3 Meeting #143	C3-254312

# Sophia-Antipolis, FR, 13 - 17 October 2025

Source:	Samsung

Title:	Pseudo-CR on OpenAPI for AIMLES_SplitOpEvent API

Spec:	3GPP TS 29.482 v1.1.0

Agenda item:	19.41

Document for:	Decision

1. Introduction

This pseudo CR implements the OpenAPI for AIMLES_SplitOpEvent API.

2. Reason for Change

As per TS 23.548 v19.2.0, the AIMLES_SplitOpEvent API has been agreed and needs to be implemented in TS 29.548. The API definition has been agreed in CT3#142, hence this CR implements the OpenAPI for the agreed API definition.

3. Conclusions

N/A

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.482 v1.1.0.

* * * First Change * * * *

# A.11	Aimles_SplitOpEvent API

openapi: 3.0.0

info:

title: Aimles_SplitOpEvent

description: |

API for AIMLE Split Operation Event Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.0.0-alpha.1"

externalDocs:

description: >

3GPP TS 29.482 v1.1.0; Artificial Intelligence Machine Learning Enablement

(AIMLE) Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

servers:

- url: '{apiRoot}/aimles-splitopevent/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122

security:

- {}

- oAuth2ClientCredentials: []

paths:

/subscriptions:

post:

summary: Request the creation of an Individual AIMLE Split Operation Event Subscription.

operationId: CreateAimleSplitOpEventSubscription

tags:

- AIMLE Split Operation Event Subscription

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SplitOpEventSub'

responses:

'201':

description: >

The requested Individual AIMLE Split Operation Event Subscription is

successfully created and a representation of the created resource is

returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/SplitOpEventSub'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

myNotification:

'{$request.body#/notifUri}':

post:

summary: Notify on the requested data.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SplitOpEventNotif'

responses:

'204':

description: The notification is successfully received.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

parameters:

- name: subscriptionId

in: path

description: >

Represents the Individual AIMLE Split Operation Event Subscription resource.

required: true

schema:

type: string

get:

summary: Retrieve an existing the Individual AIMLE Split Operation Event Subscription resource.

operationId: GetIndAimleSplitOpEventSub

tags:

- Individual AIMLE Split Operation Event Subscription (Document)

responses:

'200':

description: >

OK. The requested Individual AIMLE Split Operation Event Subscription

resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/SplitOpEventSub'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Request the update of an existing Individual AIMLE Split Operation Event

Subscription resource.

operationId: UpdateIndAimleSplitOpEventSub

tags:

- Individual AIMLE Split Operation Event Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SplitOpEventSub'

responses:

'200':

description: >

OK. The Individual AIMLE Split Operation Event Subscription resource is

successfully updated and a representation of the updated resource shall be

returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/SplitOpEventSub'

'204':

description: >

No Content. The Individual AIMLE Split Operation Event Subscription

resource is successfully updated and no content is returned in the

response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing AIMLE Split Operation Event Subscription resource.

operationId: ModifyIndAimleSplitOpEventSub

tags:

- Individual AIMLE Split Operation Event Subscription (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/SplitOpEventSubPatch'

responses:

'200':

description: >

OK. The Individual AIMLE Split Operation Event Subscription resource is

successfully modified and a representation of the updated resource shall

be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/SplitOpEventSub'

'204':

description: >

No Content. The Individual AIMLE Split Operation Event Subscription

resource is successfully modified and no content is returned in the

response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Remove the Individual AIMLE Split Operation Event Subscription .

operationId: UnsubscribeAimleSplitOpEventSub

tags:

- Individual AIMLE Split Operation Event Subscription (Document)

responses:

'204':

description: >

The individual AIMLE Split Operation Event Subscription resource

matching the subscriptionId is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

SplitOpEventSub:

description: Represents the AIMLE Split Operation Event Subscription information.

type: object

properties:

splitOpPipelineId:

type: string

reportReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

splitOpEventId:

$ref: '#/components/schemas/SplitOpEventId'

discFilters:

$ref: '#/components/schemas/DiscFilters'

assistInfo:

$ref: '#/components/schemas/AssistanceInfo'

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- splitOpPipelineId

- notifUri

- splitOpEventId

SplitOpEventSubPatch:

description: >

Represents the requested modifications to the AIMLE Split Operation Event subscription.

information.

type: object

properties:

reportReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

splitOpEventId:

$ref: '#/components/schemas/SplitOpEventId'

discFilters:

$ref: '#/components/schemas/DiscFilters'

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

SplitOpEventNotif:

description: >

Represents the AIMLE Split Operation Event notification.

type: object

properties:

splitOpEventId:

$ref: '#/components/schemas/SplitOpEventId'

availabilityInfo:

$ref: '#/components/schemas/AvailabilityInfo'

splitOpPipelineInfo:

$ref: '#/components/schemas/SplitOpPipelineInfo'

assistanceInfo:

$ref: '#/components/schemas/AssistanceInfo'

DiscFilters:

description: >

Represents the set of characteristics to determine matching split operation profiles

or nodes.

type: object

properties:

stageInfo:

type: array

items:

$ref: '#/components/schemas/StageInfo'

usageInfo:

$ref: 'TS29482_Aimles_SplitOpNodeRegistration.yaml#/components/schemas/UsageInformation'

minNodes:

type: string

required:

- stageInfo

AssistanceInfo:

description: Represents the assistance information for subscription.

type: object

properties:

deliveryTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

achievableQoS:

type: string

qosSuggestion:

type: string

SplitOpPipelineInfo:

description: Represents split operation pipeline information.

type: object

properties:

splitOpProfile:

$ref: '#/components/schemas/SplitOpProfile'

subEventId:

$ref: '#/components/schemas/SubEventId'

AvailabilityInfo:

description: Represents the availability information of split operation

pipeline.

type: object

properties:

splitOpProfile:

type: array

items:

$ref: '#/components/schemas/SplitOpProfile'

availableNodes:

type: string

SplitOpProfile:

description: Represents the split operation profile that service consumer

participates to.

type: object

properties:

splitOpPipelineId:

type: string

stageInfo:

type: array

items:

$ref: '#/components/schemas/StageInfo'

usageInfo:

$ref: 'TS29482_Aimles_SplitOpNodeRegistration.yaml#/components/schemas/UsageInformation'

headEp:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

tailEp:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

required:

- splitOpPipelineId

- headEp

- tailEp

- stageInfo

StageInfo:

description: Represents the information about split operation stages.

type: object

properties:

stageId:

type: string

numNodes:

type: string

headEp:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

tailEp:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

nodeOrder:

type: array

items:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

mlModelId:

type: string

required:

- stageId

- headEp

- tailEp

- nodeOrder

- mlModelId

#

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

SplitOpEventId:

anyOf:

- type: string

enum:

- SPLIT_OP_AVAILABILITY

- SPLIT_OP_PIPELINE_INFO

- SPLIT_OP_ASSISTANCE_INFO

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and is not used to encode content defined in the present version of this API.

description: |

Represents the split operation events.

Possible values are:

- SPLIT_OP_AVAILABILITY: Indicates split operation availability event.

- SPLIT_OP_PIPELINE_INFO: Indicates split operation pipeline information event.

- SPLIT_OP_ASSISTANCE_INFO: Indicates split operation assistance information event.

SubEventId:

anyOf:

- type: string

enum:

- CREATED

- UPDATED

- DELETED

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and is not used to encode content defined in the present version of this API.

description: |

Represents the sub event of split operation pipeline.

Possible values are:

- CREATED: Indicates that a new split operation profile is created.

- UPDATED: Indicates that an existing split operation profile is updated.

- DELETED: Indicates that an existing split operation profile is deleted.

#

* * * End of Changes * * * *