3GPP TSG CT WG3 Meeting #143	C3-254320

Sophia-Antipolis, France, 13 – 17 October 2025	(Revision of C3-25xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.523 | CR | 0128 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Supported Features |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBIProtoc19 |  |  |  |  |  | Date: |  |  | 2025-09-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In clause 5.6.2.2, the table description on the presence condition for the "suppFeat" attribute is not so correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected table descriptions for the "suppFeat" attribute with correct condition. Also added some editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not correct descriptions for supported features in the impacted clause will arouse wrong implementation issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact the OpenAPI file. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** 1st Change ***

#### 5.6.2.2	Type PcEventExposureSubsc

Table 5.6.2.2-1: Definition of type PcEventExposureSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eventSubs | array(PcEvent) | M | 1..N | Subscribed Policy Control events. |  |
| eventsRepInfo | ReportingInformation | O | 0..1 | Represents the reporting requirements of the subscription. |  |
| groupId | GroupId | C | 0..1 | Represents an internal group identifier and identifies a group of UEs. It shall be present when the subscription is targeting a Group of UE(s). |  |
| filterDnns | array(Dnn) | O | 1..N | Represents the DNNs for which the policy event report shall apply. Each DNN is a full DNN with both the Network Identifier and Operator Identifier, or a DNN with the Network Identifier only. If omitted it represents any DNN. |  |
| filterSnssais | array(Snssai) | O | 1..N | Represents the S-NSSAIs for which the policy event report shall apply. If omitted it represents any S-NSSAI. |  |
| snssaiDnns | array(SnssaiDnnCombination) | C | 1..N | Represents the combination list of S-NSSAI and DNN for which the policy event report shall apply. If omitted, it represents any combination. When the feature "AppDetection" is supported, one combination of S-NSSAI and DNN shall be provided for event "APPLICATION_START" or "APPLICATION_STOP". | EneNA, AppDetection |
| filterServices | array(ServiceIdentification) | O | 1..N | Represents the services for which the policy event report shall apply. If omitted, the policy event report shall apply for all the active services. | ExtendedSessionInformation |
| appIds | array(ApplicationId) | C | 1..N | Represents the applications for which the policy event report shall apply. It shall be provided for event "APPLICATION_START" or "APPLICATION_STOP". | AppDetection |
| tws | array(TimeWindow) | O | 1..N | Contains the time windows for which the NF service consumer is requesting to receive signalling information. It may only be provided if the SIGNALLING_INFO event is subscribed. If omitted, the PCF determines the time windows for which the signalling information is reported based on local configuration. | SignallingInfo |
| notifUri | Uri | M | 1 | Notification URI for Policy Control event reporting. |  |
| notifId | string | M | 1 | Notification Correlation ID assigned by the NF service consumer. |  |
| eventNotifs | array(PcEventNotification) | C | 1..N | Represents the Policy Control Events to be reported in the Npcf_EvenExposure_Subscribe response. It shall be present in the resource creation/update response when the "eventsRepInfo" attribute includes the "immRep" attribute set to true. Otherwise, it shall be omitted. | ERIR |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of Supported features among the ones defined in clause 5.8. Shall be present only when feature negotiation needs to take place. (NOTE) |  |
| NOTE: In the HTTP request, it represents the set of features supported by the NF service consumer. In the HTTP response, it represents the set of features supported by both the NF service consumer and the PCF. |  |  |  |  |  |

*** End of Changes ***