3GPP TSG CT WG3 Meeting #143	C3-254325

Sophia-Antipolis, France, 13 – 17 October 2025	(Revision of C3-25xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.255 | CR | 0043 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to Supported Features |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NBI19 |  |  |  |  |  | Date: |  |  | 2025-09-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In clauses 5.1.6.2.2 and 5.1.6.2.4, the table descriptions for the "suppFeat" attribute are not complete and not always correct. E.g., if the provided attributes do not require feature support, then the feature negotiation is not needed and the "suppFeat" attribute shall not be provided. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected table descriptions for the "suppFeat" attribute with complete description and correct condition in the affected clauses. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not correct table descriptions for supported features in the impacted clauses will arouse wrong implementation issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.6.2.2, 5.1.6.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact the OpenAPI file. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** 1st Change ***

##### 5.1.6.2.2	Type: UAVAuthInfo

Table 5.1.6.2.2-1: Definition of type UAVAuthInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| gpsi | Gpsi | M | 1 | GPSI of the UAV |  |
| serviceLevelId | string | M | 1 | Service Level Device Identity of the UAV |  |
| ipAddr | IpAddr | O | 0..1 | When present, this attribute indicates the IP address associated with the PDU session. |  |
| authMsg | string | O | 0..1 | Contains the authentication message or authorization data (which is not present in the initial request) used in the subsequent request messages during multiple round trip message exchanges. This attribute is deprecated; the attribute "authContainer" should be used instead. |  |
| authContainer | array(AuthContainer) | O | 1..N | Contains the AA related data without the "authResult" attribute. This attribute deprecates "authMsg" attribute. |  |
| pei | Pei | O | 0..1 | PEI associated with the UAV. |  |
| notifyUri | Uri | C | 0..1 | This attribute shall be present in the initial authentication message. It carries the notification URI to receive reauthentication, reauthorization or revocation related notifications |  |
| notifyCorrId | string | C | 0..1 | Notification correlation ID assigned by the NF service consumer. Shall be present when the "notifyUri" attribute is provided. |  |
| uavLocInfo | LocationArea5G | O | 0..1 | This attribute shall contain the UE location information if it is available. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported feature(s) among the ones defined in clause 5.1.8. This attribute shall be present only when feature negotiation needs to take place. |  |

# *** 2nd Change ***

##### 5.1.6.2.4	Type: UAVAuthResponse

Table 5.1.6.2.4-1: Definition of type UAVAuthResponse

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| gpsi | Gpsi | C | 0..1 | GPSI of the UAV. Shall be present except during PDU Session Modification for C2 Communication procedure. |  |
| authResult | AuthResult | C | 0..1 | Conveys the UAV authentication result (success) . Shall be present if there is no intermediate round-trip messages. This attribute is deprecated; the attribute "authContainer" should be used instead. |  |
| authMsg | string | C | 0..1 | Contains the authentication message or authorization data. Shall be present if the USS triggers intermediate round-trip messages. This attribute is deprecated; the attribute "authContainer" should be used instead. |  |
| authContainer | array(AuthContainer) | C | 1..N | Contains the AA related data. |  |
| serviceLevelId | string | O | 0..1 | Contains a new Service Level Device Identity of the UAV |  |
| authSessAmbr | BitRate | O | 0..1 | The DN Authorized Session-AMBR. |  |
| authProfIndex | string | O | 0..1 | DN authorization profile index. |  |
| ussInfo | array(UssInfo) | O | 1..N | Contains the USS addresses and corresponding geographical areas. | MultiUSS |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported feature(s) among the ones defined in clause 5.1.8. This attribute shall be present only when feature negotiation needs to take place. |  |

*** End of Changes ***