3GPP TSG-CT WG3 Meeting #143	C3-254346

# Sophia-Antipolis, France, 13 - 17 October 2025

Source:	Ericsson

Title:	Discussion on LMF based AIML Positioning model provisioning indicator

Document for:	Discussion & Agreement

# Agenda Item:	19.39

# Work Item / Release:	AIML_CN/Rel-19

Abstract of the contribution:

# 1	Decision/action requested

Discuss the proposal on LMF based AIML Positioning model provisioning indicator

# 2	References

[1]	3GPP TS 23.273 "5G System (5GS) Location Services (LCS); Stage 2" v19.4.0

[2]	3GPP TS 23.288 " Architecture enhancements for 5G System (5GS) to support network data analytics services;" v19.4.0

# 3	Stage 2 requirements

1)	TS 23.273 defined that LMF calculate UE positioning and LMF can directly support model training, optionally NWDAF can support model provision/training LMF-based AI/ML positioning, while NWDAF does not analytics LMF-based AI/ML positioning.

- TS 23.273 clause 5.18.0, LMF provides following input parameters in Nnwdaf_MLModelProvision_Subscribe:

- LMF-based AI/ML positioning indication

- Optionally, Vendor ID, ML Model Filter Information (e.g. Area of Interest), Positioning case information, Target of ML Model Reporting, ML Model Target Period, Time when model is needed, Inference Input Data information and ML Model Monitoring Information.

- If vendor specific information is required, then the ML Model Interoperability Information is included.

- If the LMF supports multiple AI/ML Models, indication of support for multiple ML Models, optionally with Number of ML Models and Accuracy level(s) of Interest.

- TS 23.288 clauses 5.2, 6.2A.1 and 7.5.2 defined:

- LMF includes an indication that a model for LMF-based AI/ML Positioning is requested(conditional, required if the ML model is requested for UE location calculation), which is NOT requiring an analytics ID.

NOTE:	How to implement the indication of requesting a model for LMF-based AI/ML Positioning in a backward compatible and extensible manner is up to stage 3.

# 4	Ericsson CR effective & future proof considerations

1)	 ModelEvent with feature control

a.	when “ModelEvent” feature is not supported, existing NwdafEvent and EventFilter are used for model with NWDAF Analytics event;

b.	when “ModelEvent” feature is supported, ModelEvent and ModelEventFilter are used for model without  NWDAF analytics;

c.	starting from “LMF_AIML_POSITIONING” enumeration value for LMF based AIML Positioning, effective extendable for future case(s) of model provisioning/training without NWDAF analytics.

d.	supporting all the stage 2 requirement for LMF-based AI/ML Positioning model provisioning requirements

2)	Effectively NOT impacts existing NwdafEvent which is used for Analytics Event with Analytics related Event Filters.

3)	Future Proof NOT to impacts NwdafEvent which is widely reused in other APIs/Data model for Analytics events. Also avoided mixture of data schema on Analytics parameters and avoided the related NOT needed OpenAPI files impacts.

4)	Future Proof for effective extension of Model Event without analytics by adding new enumeration value(s).

5)	ModelEventFilter with feature control

a.	Extendable for model event filter(s)

b.	Not impacts existing event filter which is used for analytics event(s).

# 5	C3-253118 issues observation

1)	Mixture of new event purely for model provisioning/training, into existing Analytics events in NwdafEvent data type by adding new value in clause 5.1.6.3.4	Enumeration: NwdafEvent, arouse not clear data model schema and not clear service consumer and provider use case,

2)	Not clear and bother extra unnecessary OpenAPI file impacts. Only with table note that UE_POSITIONING event is applicable only for the Nnwdaf_MLModelProvision API under control of feature "AIMLPositioning" and Nnwdaf_MLModelTraining API under control of feature "AIMLPositioning" is not clear for designer to understand from the OpenAPI file, also arouse extra unnecessary OpenAPI file impacts.

3)	Not effective API Design and Extra cost of implementations upon Mixture of event filter only applicable to analytics not applicable to model provisioning, with the ML Model event filter applicable to model provisioning not applicable to analytics. E.g., LMF based AI/ML Positioning filtering are different from analytics filtering in NWDAF.

4)	Not future proof upon mixing model training/provisioning with analytics for the stage 2 clearly defined model provisioning/training only event.

5)	Questionable sustainability of NwdafEvent & EventFilter and complicated future service consumers visiting the same NnwdafEvent & EventFilter which does not request analytics from NWDAF.

# 6	Proposal

Propose to discuss and agree Ericsson submitted TS 29.520 CR 0989 as the direction aligned with stage 2 requirement with effective & future proof considerations.