3GPP TSG-CT WG3 Meeting #145	C3-260045

# Goa, India, 09 - 13 February 2026

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.561 | CR | 0210 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Synchronization of 64-bit counter |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2026-01-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Discussion paper in C3-260044 describes that bad connection to the extent of losing at least 65,536 transformed QUIC packets, should be terminated to be reconnected when QoS is improved. Therefore, no synchronization for 64-bit counter is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removing the related text in clause 22.3.4. Interoperability analysis The change is backward compatible. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Because the required loss of connection must be substantial enough to cause extremely poor data transfer, this behavior is unnecessary, adding complexity without providing any benefits. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 33.501 ... CR #2204 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************************************** First Change ***************************************

### 22.3.4	Sending Media Related Information using QUIC-aware proxying using HTTP

If the media traffic is transmitted over QUIC between the AS and the UE, the UPF (acting as client) and the AS (acting as UDP proxy) may negotiate to use the dedicated QUIC Connection IDs and packet transform in Forwarded Mode to avoid re-encapsulation and re-encryption as defined in draft-ietf-masque-quic-proxy [67]. In case of multiple UEs using the same QUIC connection between the UPF and the AS, for each UE the UPF shall initiate a new HTTP CONNECT request to associate each UE with a separate Connection ID for mapping to a Virtual Connection ID as described in in draft-ietf-masque-quic-proxy [67].

As described in draft-ietf-masque-quic-proxy [67], the AS may still send some packets using the tunnelled mode even when the UPF and the AS have negotiated to use Forwarded Mode (e.g. during the initial media exchange with media related information from the AS to the UPF until it is possible to start using Forwarded Mode). Packets sent in tunnelled mode shall be encoded in HTTP Datagrams as defined in clause 22.3.2. Packets sent using Forwarded Mode shall be encoded as defined in this clause.

During the HTTP CONNECT method as described in draft-ietf-masque-quic-proxy [67], in addition to what is required according to clause 22.3.2:

a)	the UPF shall request use of Forwarded Mode, by setting:

1)	:protocol header field to connect-udp; and

2)	the accept-transform parameter of proxy-quic-forwarding header field to "3GPP_MRI_AESCCM_8",

in the HTTP CONNECT request;

b)	the AS shall respond by setting the transform parameter of proxy-quic-forwarding header field to "3GPP_MRI_AESCCM_8" in the HTTP CONNECT successful response.

The UPF and AS may exchange packets in Forwarded Mode after Connection IDs have been registered as per draft-ietf-masque-quic-proxy [67]. The AS may then transmit the Forwarded Mode packets, i.e., 3GPP_MRI_AESCCM_8 packet transform.

The 3GPP_MRI_AESCCM_8 packet transform is applied to QUIC short header packets, defined in IETF RFC 9000 [68]and shall be encoded according to Figure 22.3.4-1 (using the notational conventions defined in section 1.3 of IETF RFC 9000 [68]).

Transformed QUIC Packet {

Header Form (1) = 0,

Fixed Bit (1) = 1,

Spin Bit (1),

Reserved Bits (2),

Key Phase (1),

Packet Number Length (2),

Destination Connection ID (0..160),

Length of Protected Media Related Information(8),

[Nonce Counter(16)],

[Protected Media Related Information (..)],

Packet Number (8..32),

Packet Payload (8..),

}

Figure 22.3.4-1: Transformed QUIC short header packet defining 3GPP_MRI_AESCCM_8 packet transform

In Figure 22.3.4-1:

a)	Header Form, Fixed Bit, Spin Bit, Reserved Bits, Key Phase, and Packet Number Length shall be set according to section 17.3.1 of IETF RFC 9000 [68].

b)	Destination Connection ID shall be set to the mapping Virtual Connection ID.

c)	Length of Protected Media Related Information shall be encoded in the number of bytes as defined in clause 18.2.3 of 3GPP TS 33.501 [59].

d)	Nonce Counter shall be set as defined in clause 18.2.3 of 3GPP TS 33.501 [59]. This field shall be omitted if Length of Protected Media Related Information is zero.

e)	Protected Media Related Information shall be the output of the security algorithm defined in clause 18.2.4 of 3GPP TS 33.501 [59] which takes as an input the Media Related Information container, defined in clause 22.2. This field shall be omitted if Length of Protected Media Related Information is zero.

f)	Packet Number shall be set to the number of the transformed QUIC short header packet.

g)	Packet Payload shall be set to the end-to-end protected data.

If the media related information is missing:

-	the length of Protected Media Related Information shall be set to zero; and

-	the Protected Media Related Information field and the Nonce Counter field shall be omitted.

*************************************** End of Change ***************************************