3GPP TSG-CT WG3 Meeting #145	C3-260107

# Goa, India, 09 - 13 February, 2026	                                                (Revision of C3-26xxxx)

| CR-Form-v12.5.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.591 | CR | 0276 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Missing Application Errors |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2026-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | For each selected untrusted AF VFL clients, the VFL server NWDAF sends a Nnef_VFLTraining_Subscribe to the NEF handling that AF, and the NEF sends a Naf_VFLTraining_Subscribe to that AF. If the NEF receives an error response from the AF, the NEF shall not create the resource and shall respond to the AF with a proper error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the VFL server with a corresponding application error, if applicable. However, not all the application errors received from the AF can be forwarded to the VLF server as only "OVERLOAD" application error is defined in Nnef_VFLTraining API and 3 other application errors (i.e., "UE_LEFT_AREA", "VFL_REQS_NOT_MET" and "NOT_AVAILABLE_FOR_VFL_PROCESS") supported in Naf_VFLTraining API are missing in Nnef_VFLTraining API. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Complete the Application errors definition in Table 5.9.7.3-1 to align with Naf_VFLTraining API defined in 29.530. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NEF is not able to relay all the application errors received from the AF to the VLF server. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.46.2.2.3.2, 5.46.2.3.3.2, 5.46.2.3.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not have any impact in the OpenAPI specification. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** 1st Change ***

5.9.7.3	Application Errors

The application errors defined for the Nnef_VFLTraining service are listed in Table 5.9.7.3-1.

Table 5.9.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| OVERLOAD | 403 Forbidden | Indicates the NF is overloaded. |
| UE_LEFT_AREA | 403 Forbidden | Indicates the UE has moved out of the serving area. |
| NOT_AVAILABLE_FOR_VFL_PROCESS | 403 Forbidden | Indicates the NF is not available for the VFL process anymore. |
| VFL_REQS_NOT_MET | 403 Forbidden | Indicates the VFL training requirements are not met. |
| NOTE: Including a "ProblemDetails" data structure with the "cause" attribute in the HTTP response is optional unless explicitly mandated in the service operation clauses. |  |  |

*** End of Changes ***