3GPP TSG-CT WG3 Meeting #145	C3-260108

# Goa, India, 09 - 13 February, 2026	                                                (Revision of C3-26xxxx)

| CR-Form-v12.5.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.574 | CR | 0140 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Incorrect attribute name |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2026-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | In openAPI file, "adrfSetId" attribute is wrongly written as "ardfSetId" attribute, but it’s proposed to align the main body with the openAPI file to avoid NBC change. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace "adrfSetId" with "ardfSetId" in the main body. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between the main body and openAPI file. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.6.2.2, 5.1.6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not have any impact in the OpenAPI specification. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** 1st Change ***

5.1.6.2.2	Type NdccfAnalyticsSubscription

Table 5.1.6.2.2-1: Definition of type NdccfAnalyticsSubscription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| anaSub | NnwdafEventsSubscription | M | 1 | Subscribed analytics events. (NOTE 1) |  |
| anaNotifUri | Uri | M | 1 | Notification target address. |  |
| anaNotifCorrId | string | M | 1 | Notification correlation identifier. |  |
| formatInstruct | FormattingInstruction | O | 0..1 | Formatting instructions to be used for sending event notifications. |  |
| procInstructs | array(ProcessingInstruction) | O | 1..N | Processing instructions to be used for sending event notifications. |  |
| targetNfId | NfInstanceId | O | 0..1 | NF instance identifier to which the DCCF shall create the requested subscription. (NOTE 3) |  |
| targetNfSetId | NfSetId | O | 0..1 | NF set identifier to which the DCCF shall create the requested subscription. (NOTE 3) |  |
| adrfId | NfInstanceId | O | 0..1 | Identifier of the ADRF to be used by the DCCF. If the subscription is for runtime analytics (i.e. the "timePeriod" attribute is either absent or contains a time window in the future) then the DCCF shall store the notifications in this ADRF. If the subscription is for historical analytics (i.e. the "timePeriod" attribute contains a time window in the past) then the DCCF shall retrieve the data from this ADRF. (NOTE 3) |  |
| ardfSetId | NfSetId | O | 0..1 | Identifier of the ADRF Set to be used by the DCCF. If the subscription is for runtime analytics (i.e. the "timePeriod" attribute is either absent or contains a time window in the future) then the DCCF shall store the notifications in this ADRF Set. If the subscription is for historical analytics (i.e. the "timePeriod" attribute contains a time window in the past) then the DCCF shall retrieve the data from this ADRF Set. (NOTE 3) |  |
| suppFeat | SupportedFeatures | C | 0..1 | This IE represents a list of Supported features as described in clause 5.1.8. It shall be present if at least one feature defined in clause 5.1.8 is supported. |  |
| timePeriod | TimeWindow | O | 0..1 | Represents a start time and a stop time during which the analytics was performed or is requested to be performed. If this attribute is provided, then the attribute "monDur" within the reporting information of the attribute "anaSub" shall not be provided. (NOTE 2) |  |
| dataCollectPurposes | array(DataCollectionPurpose) | O | 1..N | The purposes of data collection. This attribute may only be provided if user consent is required depending on local policy and regulations and the consumer has not checked user consent. |  |
| NOTE 1: The "notificationURI" and "notifCorreId" attributes of NnwdafEventsSubscription data type shall be ignored by the DCCF. The DCCF will provide the notification target address and the notification correlation ID of the DCCF itself in the NnwdafEventsSubscription data type during the event subscription request to the NWDAF. NOTE 2: It includes the time period either in the past or in the future (i.e., start time as past time and stop time as future time is not allowed). NOTE 3: "targetNfId" and "targetNfSetId" are mutually exclusive. "adrfId" and "ardfSetId" are also mutually exclusive. |  |  |  |  |  |

# *** 2nd Change ***

5.1.6.2.3	Type NdccfDataSubscription

Table 5.1.6.2.3-1: Definition of type NdccfDataSubscription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dataSub | DataSubscription | M | 1 | Represents the requested events subscription. (NOTE 1) |  |
| dataNotifUri | Uri | M | 1 | Notification target address. |  |
| dataNotifCorrId | string | M | 1 | Notification correlation identifier. |  |
| formatInstruct | FormattingInstruction | O | 0..1 | Formatting instructions to be used for sending event notifications. |  |
| procInstructs | array(ProcessingInstruction) | O | 1..N | Processing instructions to be used for sending event notifications. |  |
| targetNfId | NfInstanceId | O | 0..1 | Data Producer NF instance identifier to which the DCCF shall create the requested subscription. (NOTE 3) |  |
| targetNfSetId | NfSetId | O | 0..1 | Data Producer NF set identifier to which the DCCF shall create the requested subscription. (NOTE 3) |  |
| adrfId | NfInstanceId | O | 0..1 | Identifier of the ADRF to be used by the DCCF. If the subscription is for runtime data (i.e. the "timePeriod" attribute is either absent or contains a time window in the future) then the DCCF shall store the notifications in this ADRF. If the subscription is for historical data (i.e. the "timePeriod" attribute contains a time window in the past) then the DCCF shall retrieve the data from this ADRF. (NOTE 3) |  |
| ardfSetId | NfSetId | O | 0..1 | Identifier of the ADRF Set to be used by the DCCF. If the subscription is for runtime data (i.e. the "timePeriod" attribute is either absent or contains a time window in the future) then the DCCF shall store the notifications in this ADRF Set. If the subscription is for historical data (i.e. the "timePeriod" attribute contains a time window in the past) then the DCCF shall retrieve the data from this ADRF Set. (NOTE 3) |  |
| suppFeat | SupportedFeatures | C | 0..1 | This IE represents a list of Supported features as described in clause 5.1.8. It shall be present if at least one feature defined in clause 5.1.8 is supported. |  |
| timePeriod | TimeWindow | O | 0..1 | Represents a start time and a stop time during which data was collected or is requested to be collected. If this attribute is included, then the internal attributes of the data subscription that indicate a subscription duration (e.g. the "targetPeriod" attribute of an "eventSubs" attribute of an "smfDataSub" attribute, or the "monDur" attribute of the ReportingInformation data type) shall not be provided. (NOTE 2) |  |
| dataCollectPurposes | array(DataCollectionPurpose) | O | 1..N | The purposes of data collection. This attribute may only be provided if user consent is required depending on local policy and regulations and the consumer has not checked user consent. |  |
| NOTE 1: The notification target address contained in the subscription attribute that is provided within the "dataSub" attribute (i.e. "eventNotifyUri" of "amfDataSub", "notifUri" of "smfDataSub", "callbackReference" of "udmDataSub", "notifUri" of "nefDataSub", "notifUri" of "afDataSub", "nfStatusNotificationUri" of "nrfDataSub", or "eventNotifyUri" of "nsacfDataSub") and the notification correlation ID contained in the subscription attribute that is provided within the "dataSub" attribute (i.e. the "notifyCorrelationId" of "amfDataSub", "notifId" of "smfDataSub", "notifyCorrelationId" of "udmDataSub", "notifId" of "nefDataSub", "notifId" of "afDataSub", or "notifyCorrelationId" of "nsacfDataSub") shall be ignored by the DCCF. The DCCF will provide the notification target address and the notification correlation ID of the DCCF itself in the data type during the event subscription request to each NF. NOTE 2: It includes the time period either in the past or in the future (i.e., start time as past time and stop time as future time is not allowed). NOTE 3: "targetNfId" and "targetNfSetId" are mutually exclusive. "adrfId" and "ardfSetId" are also mutually exclusive. |  |  |  |  |  |

*** End of Changes ***