3GPP TSG-CT WG3 Meeting #145	C3-260200

Goa, India, 9th – 13th February 2026 					revision of C3-260096

Source:	LG Electronics

Title:	Discussion on 3GPP PS data off exemptions handling for disaster roaming

Document for:	DISCUSSION

Agenda item:	20.2

1. Introduction

3GPP PS data off restricts user plane traffic over 3GPP access while allowing traffic required by operator-configured 3GPP PS data off exempt services. In disaster roaming scenarios, the policy objective may differ from normal roaming, but current specifications do not explicitly address 3GPP PS data off exempt services handling under disaster roaming. This paper discusses the gap and proposes normative work of 3GPP PS data off exemptions under disaster roaming.

2. Background

3GPP PS data Off is specified to restrict user plane traffic when activated, while allowing operator-configured 3GPP PS data off exempt services to remain available. 3GPP PS data off exempt services are typically configured by operators and may be applied differently depending on network context (i.e. home vs roaming).

Disaster roaming mechanisms are being studied/introduced to support service continuity in disaster conditions when the UE cannot access any available network. During disaster conditions, operators may require stricter traffic control to mitigate congestion and ensure that only essential services are available. Therefore, specification support is needed so that 3GPP PS data off exemptions can reflect operator policy intent when disaster roaming applies which means VPLMN may prefer less or limited 3GPP PS data off exempt services for inbound roamers due to disaster roaming than normal inbound roamers.

3. Discussion

3-1.	Scope of the work

The work aims to specify normative support for applying 3GPP PS data off exempt services when disaster roaming applies, so that operator policy intent under disaster conditions can be achieved. The scope focuses on defining necessary UE behaviour and related procedures in EPS and 5GS.

3-2.	 Disaster roaming applicability and policy intent

Disaster roaming may require a different exemption policy intent compared to normal roaming, e.g. allowing only essential services and mitigating unnecessary traffic during disaster conditions. The specifications should support that 3GPP PS data Off exemptions may be applied differently depending on whether disaster roaming applies, and provide sufficient clarity to avoid ambiguous interpretation across implementations.

3-3. Alternative options and selected option

- Option A: Reuse existing roaming exemption configuration without a disaster roaming applicability context.
Pros: minimal specification change (clarification only)
Cons/Risks: may not meet operator intent under disaster conditions (e.g. too permissive or too restrictive), since exemptions optimized for normal roaming may be not necessarily suitable for disaster roaming, and also may limit operators’ ability to differentiate policy intent between normal roaming and disaster roaming.

- Option B (selected): Support operator-configured exemptions applicable when disaster roaming applies.
Pros: enables a disaster-roaming-specific policy intent.
Considerations: requires normative support to make such operator configuration available and applicable to the UE (and, where relevant, to network-side policy/enforcement).

- Option C: Deployment-specific solutions without normative support.
Pros: no standard changes.
Cons/Risks: may lead to divergent outcomes across deployments and unclear operational expectations (e.g. different operator/implementation behaviour under disaster roaming).

Rationale for selecting Option B:

Option B is selected because it directly addresses the identified gap: it allows operators to apply a 3GPP PS data off exemption policy intent that is appropriate for disaster roaming (e.g. essential services only / congestion mitigation), rather than reusing exemptions optimized for normal roaming.

3-4. Why specific TS impacts are needed

- CT1 : 
When 3GPP PS data off is active and disaster roaming applies, normative support is needed to ensure the UE applies the operator-intended 3GPP PS data off exempt services.

  - TS 24.501: to specify UE behaviour for applying 3GPP PS data off exemptions when disaster roaming applies (5GS).

  - TS 24.301: to specify UE behaviour for applying 3GPP PS data off exemptions when disaster roaming applies (EPS).

  - TS 24.368 / TS 24.167: may be impacted to support provisioning/update aspects via Management Objects (e.g., NAS configuration MO and IMS MO as applicable) used to store/provide configuration information referenced by UE behaviour.

- CT3:

  - TS 29.512: Where PCF-based policy is used to apply 3GPP PS data off exemptions, the policy framework need to support policy selection/derivation considering disaster roaming. This requires impacts on Npcf_SMPolicyControl service to provide disaster roaming indication to the PCF.

- CT4

  - TS 29.502: Where pre-configured PCC rule in the SMF or PCF-based policy is used to apply 3GPP PS data off exemptions, the SMF need to aware of the disaster roaming to select/apply the appropriate handling for the session or provide disaster roaming indication to the PCF. Therefore, the AMF need to notify disaster roaming to the SMF.

- CT6

  - TS 31.102: For deployments relying on UICC/USIM-based storage/provisioning, TS 31.102 may require support to store/provide operator configuration information that the UE uses when applying 3GPP PS data off exempt services under disaster roaming (e.g., identifying information, versioning/reference data, and access conditions as applicable).

4. Conclusion and Proposal

Current specifications do not support 3GPP PS data Off exemptions handling when disaster roaming applies, which may prevent operators from applying the intended 3GPP PS data off exemption policy under disaster conditions. It is proposed to progress the corresponding WID.