3GPP TSG CT WG3 Meeting #145	C3-260212

# Goa, India, 9 - 13 February, 2026

| CR-Form-v12.5.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.437 | CR | 0014 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to the data model of the SS_SAnUsage API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_App |  |  |  |  |  | Date: |  |  | 2026-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | The following issues have been identified in the definition of the data model of the SS_SAnUsage API: - In the definition of the Spatial anchor usage information subscription request in Table 8.4.2.3.1-1 of TS 23.437, only the Spatial anchor identifiers, Spatial anchor location information, Area information and Application service information are part of the Spatial anchor filters IE. The Sampling interval is a separate standalone IE that is outside of the Spatial anchor filters IE. - Therefore, the stage 3 definition of it is incorrect, as the "sampInt" is defined as part of the Spatial anchor filters. Even worse than this, the OpenAPI description specifies the "sampInt" as part of the "anyOf" condition. - It is needed to correct this issue by defining the "sampInt" attribute at the SpatialAnchorsUsageInfoSub data type level to align with the corresponding stage 2 definition. - The attribute conveying the list of supported features is missing in the definition of the SpatialAnchorUsageReportReq data type, which represents the request body of the SS_SAnUsage_Request service operation. - The cardinality and presence conditions of the first two attribute of the AnchorUsage data type are misaligned with the OpenAPI description. They are defined as optional in the main body, while they are optional in the OpenAPI description. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR proposes to: - Correct the above-detailed issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The above-detailed issues remain in the specification and the corresponding misalignment with stage 2 remains as well. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.6.2.2, 6.1.3.6.2.3, 6.1.3.6.2.6, 6.1.3.6.2.10, 6.1.3.6.2.11, A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards compatible corrections to the OpenAPI descriptions of the following APIs: - TS29437_SS_SAnUsage.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

6.1.3.6.2.2	Type: SpatialAnchorsUsageInfoSub

Table 6.1.3.6.2.2-1: Definition of type SpatialAnchorsUsageInfoSub

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eventSubs | array(AnchorsUsageSub) | M | 1..N | Contains the spatial anchors usage information subscription details. |  |
| notifUri | Uri | M | 1 | Contains the URI via which notifications shall be delivered. |  |
| report | SpatialAnchorsUsageInfoNotif | C | 0..1 | Contains the Spatial Anchors Usage Information related report(s). This attribute shall be present only if the immediate and one time reporting was requested and the outputs are available. |  |
| sampInt | DurationSec | O | 0..1 | Indicates the interval to sample the spatial anchor(s) related information. |  |
| reportReq | ReportingInfomation | O | 0..1 | Contains the reporting requirements of the subscription. |  |
| expTime | DateTime | O | 0..1 | Contains the absolute time at which the subscription is to expire. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains supported features information, used to negotiate the applicability of optional features. This attribute shall be present only if feature negotiation needs to take place. |  |

==============Next change==============

6.1.3.6.2.3	Type: SpatialAnchorsUsageInfoSubPatch

Table 6.1.3.6.2.3-1: Definition of type SpatialAnchorsUsageInfoSubPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eventSubs | array(AnchorsUsageSub) | O | 1..N | Contains the spatial anchors usage information subscription details. |  |
| notifUri | Uri | O | 0..1 | Contains the URI via which notifications shall be delivered. |  |
| sampInt | DurationSec | O | 0..1 | Indicates the interval to sample the spatial anchor(s) related information. |  |
| reportReq | ReportingInfomation | O | 0..1 | Contains the reporting requirements of the subscription. |  |
| expTime | DateTime | O | 0..1 | Contains the absolute time at which the subscription is to expire. |  |

==============Next change==============

6.1.3.6.2.6	Type: AnchorsUsageFilter

Table 6.1.3.6.2.6-1: Definition of type AnchorsUsageFilter

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| anchIds | array(SpatialAnchorId) | C | 1..N | Contains the identities of the spatial anchors. (NOTE) |  |
| anchLoc | LocationInfo | C | 0..1 | Contains the spatial anchor location information. (NOTE) |  |
| area | ServiceArea | C | 0..1 | Contains the area information. (NOTE) |  |
| appId | string | C | 0..1 | Contains the application service identifier. (NOTE) |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

==============Next change==============

6.1.3.6.2.10	Type: SpatialAnchorUsageReportReq

Table 6.1.3.6.2.10-1: Definition of type SpatialAnchorUsageReportReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| anchUsg | array(AnchorUsage) | M | 1..N | Contains the list of usage information of the spatial anchors. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 6.1.3.8. This attribute shall be present only when feature negotiation is required. |  |

==============Next change==============

6.1.3.6.2.11	Type: AnchorUsage

Table 6.1.3.6.2.11-1: Definition of type AnchorUsage

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| anchId | SpatialAnchorId | O | 0..1 | Contains the identifier of the spatial anchor. This attribute shall be present when available. |  |
| access | Uinteger | O | 0..1 | Contains the number of times the spatial anchor was accessed. This attribute shall be present when available. |  |
| repPer | TimeWindow | O | 0..1 | Contains the time period for which the value of the "access" attribute applies. If omitted, then the value of the "access" attribute corresponds to the number of times the spatial anchor was accessed since the last spatial anchor usage report. |  |

==============Next change==============

# A.4	SS_SAnUsage API

openapi: 3.0.0

info:

title: SAn Server Spatial Anchor Usage Service

version: 1.0.0

description: |

API for Spatial Anchor Usage Service.

©2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.437 V19.0.0; Service Enabler Architecture Layer for Verticals (SEAL);

Metaverse Enablement Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.437/

servers:

- url: "{apiRoot}/ssan-usg/v1"

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549.

security:

- {}

- oAuth2ClientCredentials: []

paths:

/subscriptions:

post:

summary: Create a new Spatial Anchors Usage Information Subscription.

operationId: CreateSpatialAnchorsUsageSub

tags:

- Spatial Anchors Usage Information Subscription (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialAnchorsUsageInfoSub'

responses:

'201':

description: >

Created. The Spatial Anchors Usage Information Subscription is successfully created and

a representation of the created Individual Spatial Anchors Usage Information

Subscription resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialAnchorsUsageInfoSub'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

parameters:

- name: subscriptionId

in: path

description: >

Represents the identifier of the Individual Spatial Anchors Usage Information Subscription

resource.

required: true

schema:

type: string

get:

summary: Retrieve an existing Individual Spatial Anchors Usage Information Subscription resource.

operationId: GetIndSpatialAnchorsUsageInfoSub

tags:

- Individual Spatial Anchors Usage Information Subscription (Document)

responses:

'200':

description: >

OK. The requested Individual Spatial Anchors Usage Information Subscription is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialAnchorsUsageInfoSub'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Update an existing Individual Spatial Anchors Usage Information Subscription resource.

operationId: UpdateIndSpatialAnchorsUsageInfoSub

tags:

- Individual Spatial Anchors Usage Information Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialAnchorsUsageInfoSub'

responses:

'200':

description: >

OK. The Individual Spatial Anchors Usage Information Subscription resource is

successfully updated and a representation of the updated resource is returned in the

response body.

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialAnchorsUsageInfoSub'

'204':

description: >

No Content. The Individual Spatial Anchors Usage Information Subscription resource is

successfully updated and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Modify an existing Individual Spatial Anchors Usage Information Subscription resource.

operationId: ModifyIndSpatialAnchorsUsageInfoSub

tags:

- Individual Spatial Anchors Usage Information Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialAnchorsUsageInfoSubPatch'

responses:

'200':

description: >

OK. The Individual Spatial Anchors Usage Information Subscription resource is

successfully modified and a representation of the updated resource shall be returned in

the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialAnchorsUsageInfoSub'

'204':

description: >

No Content. The Individual Spatial Anchors Usage Information Subscription resource is

successfully modified and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Delete an existing Individual Spatial Anchors Usage Information Subscription resource.

operationId: DeleteIndSpatialAnchorsUsageInfoSub

tags:

- Individual Spatial Anchors Usage Information Subscription (Document)

responses:

'204':

description: >

No Content. The Individual Spatial Anchors Usage Information Subscription resource is

successfully deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/report:

post:

summary: Enables a service consumer to report the spatial anchor usage information.

operationId: RequestSpatialAnchorsUsageInfo

tags:

- Report Spatial Anchor Usage Information

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialAnchorUsageReportReq'

responses:

'204':

description: >

Created. The Spatial Anchor Usage Information is successfully received and

processed.

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialAnchorsUsageInfoSub'

headers:

Location:

description: >

Contains an alternative target URI located in an alternative San Server.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

SpatialAnchorsUsageInfoNotif:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

responses:

'204':

description: >

No Content. The Spatial Anchors Usage Information Notification Successfully

received and acknowledged.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: "{tokenUrl}"

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

SpatialAnchorsUsageInfoSub:

description: >

Represents the spatial anchors usage information subscription.

type: object

properties:

eventSubs:

type: array

items:

$ref: '#/components/schemas/AnchorsUsageSub'

minItems: 1

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

report:

$ref: '#/components/schemas/SpatialAnchorsUsageInfoNotif'

sampInt:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

reportReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- eventSubs

- notifUri

SpatialAnchorsUsageInfoSubPatch:

description: >

Represents the spatial anchors usage information subscription.

type: object

properties:

eventSubs:

type: array

items:

$ref: '#/components/schemas/AnchorsUsageSub'

minItems: 1

notiUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

sampInt:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

reportReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

AnchorsUsageSub:

description: >

Represent the subscription details related to spatial anchor usage.

type: object

properties:

event:

$ref: '#/components/schemas/AnchorUsageEvent'

evntFltr:

$ref: '#/components/schemas/AnchorsUsageEventFilter'

required:

- event

- evntFltr

AnchorsUsageEventFilter:

description: >

Represents the filter information for anchors usage event.

type: object

properties:

anchAnlFltr:

$ref: '#/components/schemas/AnchorsUsageFilter'

AnchorsUsageFilter:

description: >

Represents the spatial anchors usage filter information.

type: object

properties:

anchIds:

type: array

items:

$ref: 'TS29437_SS_SAnManagement.yaml#/components/schemas/SpatialAnchorId'

minItems: 1

anchLoc:

$ref: 'TS29122_MonitoringEvent.yaml#/components/schemas/LocationInfo'

area:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ServiceArea'

appId:

type: string

anyOf:

- required: [anchIds]

- required: [anchLoc]

- required: [area]

- required: [appId]

SpatialAnchorsUsageInfoNotif:

description: >

Represents the spatial anchors usage information notification.

type: object

properties:

subId:

type: string

reports:

type: array

items:

$ref: '#/components/schemas/SpatialAnchorsUsageReport'

minItems: 1

required:

- subId

- reports

SpatialAnchorsUsageReport:

description: >

Represents the usage report in the notification.

type: object

properties:

event:

$ref: '#/components/schemas/AnchorUsageEvent'

anchUsgInf:

type: array

items:

$ref: '#/components/schemas/AnchorsUsageInfo'

minItems: 1

required:

- event

AnchorsUsageInfo:

description: >

Represents the anchor usage details.

type: object

properties:

anchIds:

type: array

items:

$ref: 'TS29437_SS_SAnManagement.yaml#/components/schemas/SpatialAnchorId'

minItems: 1

discAcc:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

serAcc:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

anchDensity:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

usrDensity:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

required:

- anchIds

- discAcc

- serAcc

- anchDensity

- usrDensity

SpatialAnchorUsageReportReq:

description: >

Represents the request for spatial anchor usage report.

type: object

properties:

anchUsg:

type: array

items:

$ref: '#/components/schemas/AnchorUsage'

minItems: 1

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- anchUsg

AnchorUsage:

description: >

Represents the anchor usage information reported.

type: object

properties:

anchId:

$ref: 'TS29437_SS_SAnManagement.yaml#/components/schemas/SpatialAnchorId'

access:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

repPer:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

AnchorUsageEvent:

anyOf:

- type: string

enum:

- SPATIAL_ANCHOR_USAGE_INFORMATION

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration but

is not used to encode content defined in the present version of this API.

description: |

Represents the type of entity.

Possible values are:

- SPATIAL_ANCHOR_USAGE_INFORMATION: Indicates that the event is related to spatial

anchors usage.

# Data types describing alternative data types or combinations of data types:

#

==============End of change==============