3GPP TSG CT WG3 Meeting #145	C3-260236

# Goa, India, 9 - 13 February, 2026

| CR-Form-v12.5.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.561 | CR | 0212 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to obsoleted IETF RFC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2026-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | IETF RFC 8415 was obsoleted by RFC 9915. The changes between the obsoleted RFC 8415 and the new RFC 9915 are mainly to obsolete the IA_TA option, UNICAST option and UseMulticast status code mechanisms. Those changes do not impact the current text in this specification. Thus, RFC 8415 should be obsoleted by RFC 9915 accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR proposes to: - Correct the above-detailed issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The above-detailed issues remain in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 10.1, 10.2.3, 10.2.4, 10.2.5.1, 10.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.281: "General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)".

[5]	3GPP TS 29.061: "Interworking between the Public Land Mobile Network (PLMN) supporting packet based services and Packet Data Networks (PDN)".

[6]	IETF RFC 3748: "Extensible Authentication Protocol (EAP)".

[7]	IETF RFC 3579: "RADIUS (Remote Authentication Dial In User Service) Support For Extensible Authentication Protocol (EAP)".

[8]	IETF RFC 2865: "Remote Authentication Dial In User Service (RADIUS)".

[9]	IETF RFC 3162: "RADIUS and IPv6".

[10]	IETF RFC 4818: "RADIUS Delegated-IPv6-Prefix Attribute".

[11]	IETF RFC 5216: "The EAP-TLS Authentication Protocol".

[12]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[13]	3GPP TS 24.229: "IP Multimedia Call Control Protocol based on SIP and SDP; Stage 3".

[14]	IETF RFC 2132: "DHCP Options and BOOTP Vendor Extensions".

[15]	IETF RFC 3361: "Dynamic Host Configuration Protocol (DHCP-for-IPv4) Option for Session Initiation Protocol (SIP) Servers".

[16]	IETF RFC 3646: "DNS Configuration options for Dynamic Host Configuration Protocol for IPv6 (DHCPv6)".

[17]	IETF RFC 3319: "Dynamic Host Configuration Protocol (DHCPv6) Options for Session Initiation Protocol (SIP) Servers".

[18]	IETF RFC 2131: "Dynamic Host Configuration Protocol".

[19]	IETF RFC 1542: "Clarification and Extensions for the Bootstrap Protocol".

[20]	IETF RFC 4039: "Rapid Commit Option for the Dynamic Host Configuration Protocol version 4 (DHCPv4)".

[21]	IETF RFC 9915: "Dynamic Host Configuration Protocol for IPv6 (DHCPv6)".

[22]	Void

[23]	IETF RFC 7155: "Diameter Network Access Server Application".

[24]	IETF RFC 6733: "Diameter Base Protocol".

[25]	IETF RFC 4072: "Diameter Extensible Authentication Protocol (EAP) Application".

[26]	IETF RFC 2866: "RADIUS Accounting".

[27]	IETF RFC 5176: "Dynamic Authorization Extensions to Remote Authentication Dial In User Service (RADIUS)".

[28]	3GPP TS 23.003: "Numbering, addressing and identification".

[29]	IETF RFC 1825: "Security Architecture for the Internet Protocol".

[30]	IETF RFC 1826: "IP Authentication Header".

[31]	IETF RFC 1827: "IP Encapsulating Security Payload (ESP)".

[32]	IETF RFC 4291: "IP Version 6 Addressing Architecture".

[33]	IETF RFC 4861: "Neighbor Discovery for IP Version 6 (IPv6)".

[34]	IETF RFC 4862: "IPv6 Stateless Address Autoconfiguration".

[35]	IETF RFC 1027: "Using ARP to Implement Transparent Subnet Gateways".

[36]	802.3-2015 - IEEE Standard for Ethernet.

[37]	IETF RFC 5281: "Extensible Authentication Protocol Tunneled Transport Layer Security Authenticated Protocol Version 0 (EAP-TTLSv0)".

[38]	3GPP TS 23.380: "IMS Restoration Procedures".

[39]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[40]	3GPP TS 29.502: "5G System; Session Management Services; Stage 3".

[41]	3GPP TS 29.229: "Cx and Dx interfaces based on Diameter protocol; Protocol details".

[42]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[43]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[44]	IETF RFC 7761: "Protocol Independent Multicast - Sparse Mode (PIM-SM): Protocol Specification (Revised)".

[45]	IETF RFC 3973: "Protocol Independent Multicast - Dense Mode (PIM-DM): Protocol Specification (Revised)".

[46]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces Stage 3".

[47]	IETF RFC 2132: "DHCP Options and BOOTP Vendor Extensions".

[48]	IETF RFC 3925: "Vendor-Identifying Vendor Options for Dynamic Host Configuration Protocol version 4 (DHCPv4)".

[49]	Void

[50]	3GPP TS 29.274: "3GPP Evolved Packet System. Evolved GPRS Tunnelling Protocol for EPS (GTPv2)".

[51]	CableLabs WR-TR-5WWC-ARCH: "5G Wireless Wireline Converged Core Architecture".

[52]	BBF WT-470: "5G FMC Architecture".

[53]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[54]	BBF TR-456: "AGF Functional Requirements".

[55]	CableLabs DOCSIS MULPI: "Data-Over-Cable Service Interface Specifications DOCSIS 3.1, MAC and Upper Layer Protocols Interface Specification".

[56]	IETF RFC 7542: "The Network Access Identifier".

[57]	IETF RFC 2661: " Layer Two Tunneling Protocol "L2TP".

[58]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane of EPC Nodes; Stage 3".

[59]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[60]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[61]	3GPP TS 26.502: "5G Multicast-Broadcast User Service Architecture".

[62]	IETF RFC 6603: "Prefix Exclude Option for DHCPv6-based Prefix Delegation".

[63]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[64]	IETF RFC 9298: "Proxying UDP in HTTP"

[65]	IETF RFC 9114: "Hypertext Transfer Protocol Version 3 (HTTP/3)"

[66]	IETF RFC 9868: "Transport Options for UDP".

[67]	draft-ietf-masque-quic-proxy: "QUIC-Aware Proxying Using HTTP".

Editor's note:	The above reference will be revised to RFC when finalized by IETF.

[68]	IETF RFC 9000: "QUIC: A UDP-Based Multiplexed and Secure Transport"

[69]	draft-ietf-moq-transport: "Media over QUIC Transport".

Editor's note: The above reference will be revised to RFC when finalized by IETF.

[70]	3GPP TS 26.522: "5G Real-time Media Transport Protocol Configurations".

[71]	IEEE 802.1Q-2022: "IEEE Standard for Local and metropolitan area networks--Bridges and Bridged Networks".

[72]	IETF RFC 9297: "HTTP Datagrams and the Capsule Protocol"

[73]	IETF RFC 5705: "Keying Material Exporters for Transport Layer Security (TLS)"

==============Next change==============

## 10.1	General

In current LAN environments the most commonly used configuration protocol is DHCP (Dynamic Host Configuration Protocol, IETF RFC 2131 [18]) and DHCPv6 (Dynamic Host Configuration Protocol for IPv6, IETF RFC 9915 [21]). It provides a mechanism for passing a large set of configuration parameters to hosts connected to a TCP/IP network (IP address, sub-net mask, domain name, MTU, etc.) in an automatic manner. Moreover, DHCP may assign IP addresses to clients for a finite lease time, allowing for sequential reassignment of addresses to different users.

The lease time is chosen by the administrator of the DHCP server (in the external network), and is therefore out of the scope of the present document.

The 3GPP network may obtain IP address via external DHCP server during the PDU establishment procedure, the SMF or SMF+PGW-C acts a DHCP server towards the UE and it acts as a DHCP client towards the external DHCP server.

In the following cases the PDU session associated with the allocated IPv4 address or IPv6 prefix shall be released:

-	if the DHCP lease expires;

-	if the DHCP renewal is rejected by the DHCP server;

-	if the IP address is changed during the renewal process. Usually when the lease is renewed, the IP address remains unchanged. However, if for any reason (e.g. poor configuration of the DHCP server), a different IP address is allocated during the lease renewal process the associated PDU session shall be released.

A RG may request DHCP singalling for a UE behind the RG as specified in 3GPP TS 23.316 [43].When handling DHCP signalling coming from the wireline BBF access, the SMF or SMF+PGW-C shall support the DHCP signalling as described in BBF TR-456 [54].

==============Next change==============

### 10.2.3	IPv6 Prefix allocation via IPv6 stateless address autoconfiguration via DHCPv6

When the IPv6 prefix is allocated from the external DN, the SMF or SMF+PGW-C is responsible to obtain the IPv6 prefix for external DN, allocate and release the IPv6 prefix. The SMF or SMF+PGW-C may use DHCPv6 to obtain the IPv6 prefix from the external DN. In this context, the SMF or SMF+PGW-C shall act as a DHCP client as per IETF RFC 9915 [21] towards the external DHCPv6 server.

The SMF or SMF+PGW-C may allocate a second IPv6 prefix for routing traffic via a second UPF or UPF+PGW-U to enable simultaneous access via remote and local networks or to enable SSC mode 3 (i.e. make-before-break) mobility, as described in clause 4.3.5.3 of 3GPP TS 23.502 [3].

The following system procedure describes the signalling flows for the IPv6 Stateless Address Autoconfiguration procedures for 5G system. The procedures are based on the descriptions in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3].

1.	UE initiates the PDU Session Establishment procedure, indicating IPv6 address is required.

2.	The AMF sends PDU Session Establishment Request in Nsmf_PDUSession_CreateSMContext to the SMF or SMF+PGW-C.

3.	The SMF or SMF+PGW-C may retrieve IPv6 prefix using DHCPv6 mechanism. This procedure is performed when an external DN allocates an IPv6 prefix, the signaling between the SMF or SMF+PGW-C and external DN is exchanged via UPF or UPF+PGW-U which is omitted in the figure 10.2.3-1.

4.	The SMF or SMF+PGW-C sends PDU Session Establishment Accept included in Namf_Communication_N1N2MessageTransfer to the AMF. It includes the IPv6 prefix.

5.	The AMF sends PDU Session Establishment Accept message to the UE without the IPv6 prefix. The UE shall ignore the IPv6 prefix if it receives it in the message.

6.	The UE may send a Router Solicitation to the SMF or SMF+PGW-C via the UPF or UPF+PGW-U to solicit a Router Advertisement message.

7.	The SMF or SMF+PGW-C sends a Router Advertisement message to the UE via the UPF or UPF+PGW-U, solicited or unsolicited. It shall include an IPv6 prefix in Prefix Information option field of the message. The prefix is the same as the one in the PDU Session Establishment Accept message, if it is provided during the previous PDU Session Establishment procedure.

8.	At any time after PDU session establishment, the SMF or SMF+PGW-C may trigger the establishment on an alternative route via UPF2 or UPF+PGW-U2 for access to a local data network or for SSC mode 3 mobility.

9.	Like step 3, the SMF or SMF+PGW-C may retrieve a second IPv6 prefix using DHCPv6 mechanism.

10.	The SMF or SMF+PGW-C sends a Router Advertisement to the UE via UPF2 or UPF+PGW-U2 to update the UE. Note that this will occur without a Router Solicitation since the UE is unaware of the network's decision to form an alternative Route.

11.	Specific to the case of SSC mode 3 mobility, the SMF or SMF+PGW-C sends a Router Advertisement to the UE via UPF1 or UPF+PGW-U1 with zero value in the preferred lifetime field and a value in the valid lifetime field according to IETF RFC 4862 [34]. The UE shall update the valid lifetime of the old IPv6 prefix to the signalled value, regardless of the remaining lifetime. The signalled lifetime value indicates how long the SMF or SMF+PGW-C is willing to keep the old IPv6 prefix.

NOTE:	Alternative routes can be established repeatedly through additional UPFs or UPF+PGW-Us and old routes can be terminated when required by the SMF or SMF+PGW-C. More complex scenarios are not described here for the sake of simplicity.

Figure 10.2.3-1: IPv6 Stateless Address Autoconfiguration

==============Next change==============

### 10.2.4	IPv6 parameter configuration via stateless DHCPv6

A UE that has obtained an IPv6 address may use stateless DHCP to request other configuration information such as a list of DNS recursive name servers or SIP servers.

For 3GPP networks, when an external DHCPv6 server in a DN is used to obtain the requested parameters, the SMF acts as a DHCPv6 client towards the external DHCPv6 server while acting a DHCPv6 server towards the UE.

The IPv6 parameter configuration via stateless DHCPv6 function in the UE, the SMF and the external DHCPv6 Server shall be compliant to IETF RFC 9915 [21]. The following system procedure describes the signalling flows for the IPv6 parameter configuration via stateless DHCPv6 procedures for 5G system. All messages in the following steps between the UE and the SMF are sent via the UPF.

1)	A Router Advertisement with the O-flag set, is sent from SMF to UE to indicate to it to retrieve other configuration information.

2)	The UE sends an INFORMATION-REQUEST message with the IP destination address set to the All_DHCP_Relay_Agents_and_Servers multicast address defined in the DHCPv6 IETF RFC 9915 [21]. The source address shall be the link-local address of the UE. The DHCP relay agent in the SMF shall forward the message.

3)	DHCP servers receiving the forwarded INFORMATION-REQUEST message, reply by sending a RELAY-REPLY message, with the "Relay Message" option including a REPLY message with the requested configuration parameters.

The UE chooses one of the possibly several REPLY messages and extracts the configuration information.

Figure 10.2.4-1: DHCPv6 Other configuration signal flow

==============Next change==============

#### 10.2.5.1	General requirement

SMF or SMF+PGW-C may (depending on the configuration for the DNN/S-NSSAI) assign a single network prefix shorter than the default /64 prefix to a PDU Session from the internal IPv6 Prefix pool or via DHCPv6. In this case, the /64 default prefix used for IPv6 stateless autoconfiguration shall be allocated from this network prefix, except for RG may have different shorter IPv6 prefix(es) delegated via DHCPv6 not including the allocated /64 default prefix; the remaining address space from the network prefix may be delegated to the PDU Session using prefix delegation after the PDU Session establishment and IPv6 prefix allocation via IPv6 stateless address autoconfiguration as defined in clause 10.2.3.

Depending on configuration, the SMF may obtain the prefix from a locally provisioned pool, from the PSA UPF or from the external DN.

If the UE had indicated that it supports prefix exclusion and the prefix to be delegated to the UE includes the /64 prefix that was allocated to the PDU Session, the SMF or SMF+PGW shall utilize the prefix exclusion feature as specified for DHCPv6 Prefix Delegation in RFC 6603 [62].

The UE uses DHCPv6 to request additional IPv6 prefix (i.e.: prefix in addition to the default prefix) from the SMF or SMF+PGW-C after completing stateless IPv6 address autoconfiguration procedures. The UE acts as a "Requesting Router" as described in RFC 9915 [21] and inserts one or more IA_PD option(s) into a DHCPv6 Solicit message sent from the UE to the SMF or SMF+PGW-C via the user plane and the UPF or UPF+PGW-U. The SMF or SMF+PGW-C acts as the DHCP server and fulfils the role of a "Delegating Router" according to RFC 9915 [21]. The UE optionally includes the RAPID_COMMIT option in the DHCPv6 Solicit message to trigger two-message DHCPv6 procedure instead of the four-message DHCPv6 procedure. The UE shall include OPTION_PD_EXCLUDE option code in an OPTION_ORO option to indicate support for prefix exclusion. In response to the DHCPv6 Solicit message, the UE receives a DHCPv6 Reply message with one or more IA_PD prefix(es) for every IA_PD option that it sent in the DHCPv6 Solicit message. The SMF or SMF+PGW-C delegates a prefix excluding the default prefix with help of OPTION_PD_EXCLUDE, except for RG as UE when different shorter IPv6 prefix(es) is delegated via DHCPv6. Prefix exclusion procedures shall follow RFC 6603 [62].

The additional requirement for RG connecting to 5GC with IPv6 Prefix Delegation via DHCPv6 is defined in clause 10.2.5.2.

==============Next change==============

## 10.3	3GPP Vendor-Specific Options

This clause describes 3GPP Vender-Specific Options that will be included in DHCP messages exchanged between SMF or SMF+PGW-C and DHCP Server. Other DHCP options may be used as defined in DHCP RFC(s). Unless otherwise stated, when the encoding scheme of an attribute is specified as UTF-8 encoding, this shall be interpreted as UTF-8 hexadecimal encoding.

The DHCPv4 vendor specific option is encoded as per IETF RFC 2132 [47] or IETF RFC 3925 [48]. The DHCPv6 vendor specific option is encoded as per IETF RFC 9915 [21]. For DHCP vendor specific option code 17 or 125, the Enterprise Number shall be set to value 10415.

The table 10.3-1 lists the encapsulated 3GPP Vendor-Specific Options in DHCP vendor specific option (17/43/125).

Table 10.3-1: List of the encapsulated 3GPP Vendor-Specific sub-options

| Sub-opt # | Sub-option Name | Presence |
| --- | --- | --- |
| 1 | 3GPP-IP-Pool-Info | Optional |

1 – 3GPP-IP-Pool-Info

| DHCPv4 |  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets |  | 8 |  | 7 | 6 | 5 | 4 | 3 |  | 2 | 1 |
| 1 |  | VS option code = 1 |  |  |  |  |  |  |  |  |  |
| 2 |  | VS option length |  |  |  |  |  |  |  |  |  |
| 3-m |  | IP address pool ID |  |  |  |  |  |  |  |  |  |

| DHCPv6 |  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets |  | 8 |  | 7 | 6 | 5 | 4 | 3 |  | 2 | 1 |
| 1-2 |  | VS option code = 1 |  |  |  |  |  |  |  |  |  |
| 3-4 |  | VS option length |  |  |  |  |  |  |  |  |  |
| 5-m |  | IP address pool ID |  |  |  |  |  |  |  |  |  |

VS option code: 1

Length: m-2 or m-4

The IP address pool ID is of Octet String type.

The SMF or SMF+PGW-C may determine an IP address pool ID based on UPF ID, S-NSSAI, DNN, and IP version as described in clause 5.8.2.2.1 in 3GPP TS 23.501 [2] and includes the IP address pool ID within 3GPP Vendor-Specific-Option and send it to the DHCP server. The DHCP server assigns IPv6 prefix or IPv4 address from the requested IP address pool. Multiple 3GPP-IP-Pool-Info sub-options may be sent in a DHCP request message. The DHCP server shall return the selected IP address pool ID within 3GPP Vendor-Specific-Option to the SMF or SMF+PGW-C in the DHCP successful response message (e.g. DHCPACK).

==============End of change==============