3GPP TSG CT WG3 Meeting #145	C3-260288

# Goa, India, 9 - 13 February, 2026

| CR-Form-v12.5.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.482 | CR | 0026 | rev | - | Current version: | 19.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to AIMLES_AIMLEClientDiscovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_App |  |  |  |  |  | Date: |  |  | 2026-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | In the AIMLES_AIMLEClientDiscovery API, the clients attribute is defined as mandatory in the ClientDiscResp data type, which is consistent with the Stage‑2 requirement specified in TS 23.482 clause 8.8.3.2, where the list of AIMLE client IDs information element is mandatory. However, in the corresponding OpenAPI file, the clients attribute is not marked as required. Therefore, the OpenAPI file needs to be corrected to indicate the clients attribute as required. Also the clNumber attribute in the GET query needs to be corrected to cl-number attribute in the GET query parametsrs align with OpenAPI definition. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes to correct the above-mentioned issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistency between data model and OpenAPI definition. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.3.2.3.1, A.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible correction to the following APIs: TS29482_AIMLES_AIMLEClientDiscovery.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** First Change ***

6.1.6.3.2.3.1	GET

This method shall support the URI query parameters specified in table 6.1.6.3.2.3.1-1.

Table 6.1.6.3.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| filt-criteria | ClientDiscCriteria | M | 1 | Represents the AIMLE Client(s) filtering criteria. |
| cl-number | Uinteger | O | 0..1 | Represents the required number of the discovered AIMLE Clients. |
| supported-features | SupportedFeatures | C | 0..1 | Contains supported features information, used to negotiate the applicability of optional features. This query parameter shall be present only if feature negotiation needs to take place. |

This method shall support the request data structures specified in table 6.1.6.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.6.3.2.3.1-3.

Table 6.1.6.3.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.6.3.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ClientDiscResp | M | 1 | 200 OK | Successful case. The response body contains the result of the search over the list of AIMLE Clients. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.6.3.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

Table 6.1.6.3.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

# *** Second Changes ***

# A.5	AIMLES_AIMLEClientDiscovery API

openapi: 3.0.0

info:

title: AIMLES_AIMLEClientDiscovery

version: 1.0.0

description: |

API for AIMLES Client Discovery Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.482 v19.0.0; Artificial Intelligence Machine Learning Enablement

(AIMLE) Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.482/

servers:

- url: '{apiRoot}/aimles-disc/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

security:

- {}

- oAuth2ClientCredentials: []

paths:

/clients:

get:

summary: Retrieve an existing the Individual AIMLE Data Management Assistance Subscription resource.

operationId: GetAimleClients

tags:

- AIMLE Clients (Collection)

parameters:

- name: filt-criteria

in: query

description: >

Represents the AIMLE Client(s) filtering criteria.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ClientDiscCriteria'

- name: cl-number

in: query

description: >

Represents the required number of the discovered AIMLE Clients.

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

- name: supported-features

in: query

description: >

Contains supported features information, used to negotiate the applicability

of optional features.

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: >

OK. The response body contains the result of the search over the list of AIMLE Clients.

content:

application/json:

schema:

$ref: '#/components/schemas/ClientDiscResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

ClientDiscCriteria:

description: Represents the AIMLE Client discovery criteria.

type: object

properties:

serviceReq:

$ref: '#/components/schemas/ServiceRequirement'

mlModelTypes:

type: array

items:

$ref: '#/components/schemas/MlModelType'

minItems: 1

aimlOpers:

type: array

items:

$ref: '#/components/schemas/AimlOperationRole'

minItems: 1

clientAppCap:

$ref: '#/components/schemas/ClientAppCapability'

datasetReq:

$ref: '#/components/schemas/DatasetRequirement'

clientTaskCap:

$ref: '#/components/schemas/ClientTaskCapability'

clientVel:

$ref: '#/components/schemas/ClientVelocity'

location:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

clientQosReqs:

type: array

items:

$ref: 'TS29548_SDD_TransmissionQualityMeasurement.yaml#/components/schemas/TransQualMeasCriteriaSet'

minItems: 1

required:

- serviceReq

- aimlOpers

- clientAppCap

ServiceRequirement:

description: Represents the AIMLE service requirements.

type: object

properties:

valServId:

type: string

permLevel:

$ref: '#/components/schemas/ServicePermLevel'

required:

- valServId

ClientAppCapability:

description: Represents the AIMLE Client compute capability type.

type: object

properties:

appType:

$ref: '#/components/schemas/MlAppType'

avail:

type: array

items:

$ref: 'TS29122_CpProvisioning.yaml#/components/schemas/ScheduledCommunicationTime'

minItems: 1

dropOfRate:

type: integer

minimum: 0

maximum: 100

required:

- appType

DatasetRequirement:

description: Represents the dataset requirements.

type: object

properties:

avail:

$ref: '#/components/schemas/DatasetAvailability'

cap:

$ref: '#/components/schemas/DatasetCapability'

DatasetAvailability:

description: Represents the dataset availability information.

type: object

properties:

ids:

type: array

items:

type: string

minItems: 1

age:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

size:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

features:

type: array

items:

type: string

minItems: 1

DatasetCapability:

description: Represents the dataset capability information.

type: object

properties:

dataType:

$ref: '#/components/schemas/DataType'

functions:

type: array

items:

type: string

minItems: 1

ClientTaskCapability:

description: Represents the performance capabilities.

type: object

properties:

compCap:

$ref: '#/components/schemas/ComputeCapability'

perfCap:

$ref: '#/components/schemas/PerformanceCapability'

ClientDiscResp:

description: Represents the AIMLE Client discovery response.

type: object

properties:

clients:

type: array

items:

$ref: 'TS29482_AIMLES_ContextTransfer.yaml#/components/schemas/AimleClientId'

minItems: 1

suppTasks:

type: array

items:

$ref: '#/components/schemas/ClientTaskCapability'

minItems: 1

suppFeats:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- clients

# SIMPLE DATA TYPES

#

# ENUMERATIONS

ServicePermLevel:

anyOf:

- type: string

enum:

- PREMIUM

- STANDARD

- LIMITED

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Represents the  service permision level.

Possible values are:

- PREMIUM: Indicates that the service permission level is premium.

- STANDARD: Indicates that the service permission level is standard.

- LIMITED: Indicates that the service permission level is limited.

MlModelType:

anyOf:

- type: string

enum:

- DECISION_TREES

- LINEAR_REGRESSION

- NEURAL_NETWORKS

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Represents the ML model type.

Possible values are:

- DECISION_TREES: Indicates that the ML model type is decision trees.

- LINEAR_REGRESSION: Indicates that the ML model type is linear regression.

- NEURAL_NETWORKS: Indicates that the ML model type is neural networks.

AimlOperationRole:

anyOf:

- type: string

enum:

- MODEL_TRAINING

- MODEL_TRANSFER

- MODEL_INFERENCE

- MODEL_OFFLOAD

- MODEL_SPLIT

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Represents the AIMLE operation role.

Possible values are:

- MODEL_TRAINING: Indicates that the supported AIML operation role is model training.

- MODEL_TRANSFER: Indicates that the supported AIML operation role is model transfer.

- MODEL_INFERENCE: Indicates that the supported AIML operation role is model inference.

- MODEL_OFFLOAD: Indicates that the supported AIML operation role is model offload.

- MODEL_SPLIT: Indicates that the supported AIML operation role is model split.

MlAppType:

anyOf:

- type: string

enum:

- FL

- TL

- SL

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Represents the ML model type.

Possible values are:

- FL: Indicates that the supported ML application type is Federated Learning.

- TL: Indicates that the supported ML application type is Transfer Learning.

- SL: Indicates that the supported ML application type is Split Learning.

DataType:

anyOf:

- type: string

enum:

- RAW

- PROCESSED

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Represents the type of the collected data.

Possible values are:

- RAW: Indicates that the type of the collected data is raw.

- PROCESSED: Indicates that the type of the collected data is processed.

PerformanceCapability:

anyOf:

- type: string

enum:

- GREEN_TASK

- ENERGY_EFFICIENT

- LOW_COSTS

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Represents the performance capabilities.

Possible values are:

- GREEN_TASK: Indicates that the performance capability is green task.

- ENERGY_EFFICIENT: Indicates that the performance capability is energy-efficient.

- LOW_COSTS: Indicates that the performance capability is low costs.

ComputeCapability:

anyOf:

- type: string

enum:

- LOW

- HIGH

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Represents the AIMLE Client compute capability type.

Possible values are:

- LOW: Indicates that the compute capability is low.

- HIGH: Indicates that the compute capability is high.

ClientVelocity:

anyOf:

- type: string

enum:

- MOBILE_LOW

- MOBILE_HIGH

- STATIC

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Represents the AIMLE Client velocity level.

Possible values are:

- MOBILE_LOW: Indicates that the AIMLE Client is mobile with low velocity.

- MOBILE_HIGH: Indicates that the AIMLE Client is mobile with high velocity.

- STATIC: Indicates that the AIMLE Client is static.

# *** End of Changes ***