3GPP TSG-CT3 Meeting #145	C3-260299

# Goa, India, 9th Feb 2026 - 13th Feb 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.552 | CR | 0181 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | VFL Inference procedure corrections for NWDAF or AF as VFL server |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2026-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | There are various mistakes, including wrong NF names, references, and ambiguous formulations in the VFL Inference procedures with NWDAF or AF as VFL server. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected the wrong NF names, references, and formulations in the VFL Inference procedures with NWDAF or AF as VFL server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.10.3.4.1, 5.10.3.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

##### 5.10.3.4.1	General

The inference procedures are triggered by NWDAF containing AnLF. The NWDAF containing AnLF determines the VFL server (i.e., NWDAF, trusted AF, or untrusted AF) based on the VFL server discovery procedure as described in the clause 4.4.52.2 of 3GPP TS 29.522 [10], based on local configuration, or via the NRF. The VFL server generates and provides an aggregation result to NWDAF containing AnLF based on local inference results received from the VFL client(s).

The inference procedures in this clause include the following cases:

-	subclause 5.10.3.4.2 specifies the inference procedure when NWDAF or trusted AF is acting as VFL server while VFL client(s) can be NWDAF, AF, and/or untrusted AF;

-	subclause 5.10.3.4.3 specifies the inference procedure when untrusted AF is acting VFL server while VFL client(s) can be NWDAF.

==============Next change==============

##### 5.10.3.4.2	Inference procedure for vertical federated learning when NWDAF or Trusted AF is acting as VFL server

The inference procedure when trusted AF or NWDAF is acting as VFL server may be triggered by the NWDAF containing AnLF due to a request or subscription from a 5GC consumer NF or internal service logic of the AF acting as VFL server. This procedure is used by the VFL Server (trusted AF or NWDAF) to trigger inference procedure for Vertical Federated Learning among multiple NWDAF instances and/or AF containing VFL client.

Figure 5.10.3.4.2-1: Inference procedure for Vertical Federated Learning when NWDAF or Trusted AF is acting as VFL server

0.	To send a request for analytics events, the NWDAF service consumer invokes the Nnwdaf_EventsSubscription_Subscribe service operation to NWDAF containing AnLF as described in clause 4.2.2.2 of 3GPP TS 29.520 [5], or the NWDAF service consumer invokes Nnwdaf_AnalyticsInfo_Request service operation to NWDAF containing AnLF to request the analytics information as described in clause 4.3.2.2 of 3GPP TS 29.520 [5].

1.	The NWDAF containing AnLF determines the NWDAF or trusted AF acting as a VFL server. If NWDAF is selected as VFL server, the NWDAF containing AnLF invokes the Nnwdaf_EventsSubscription_Subscribe service operation to VFL server NWDAF as described in clause 4.2.2.2 of 3GPP TS 29.520 [5]. If the trusted AF is selected as VFL server, the NWDAF containing AnLF invokes Naf_Inference_Subscribe service operation to VFL server trusted AF to perform the VFL inference as described in clause 5.5.2.2.2 of 3GPP TS 29.530 [53].

2.	The VFL server and VFL clients may perform VFL training procedures as described in clause 5.10.3.3.

3.	The NWDAF or trusted AF acting as VFL server sends the VFL inference request to each VFL client.

3a. If the VFL client is an NWDAF, the NWDAF or trusted AF acting as VFL server invokes Nnwdaf_VFLInference_Subscribe service operation as described in clause 4.11.2.2.2 of 3GPP TS 29.520 [5]. If the Subscription is successfully processed and accepted, the VFL client sends Nnwdaf_VFLInference_Subscribe Response message as described in clause 4.11.2.2.2 of 3GPP TS 29.520 [5].

3b.	If the VFL client is a trusted AF, the NWDAF acting as VFL server invokes Naf_VFLInference_Subscribe service operation as described in clause 5.3.2.2.2 of 3GPP TS 29.530 [53]. If the Subscription is successfully processed and accepted, VFL client sends Naf_VFLInference_Subscribe Response message as described in clause 5.3.2.2.2 of 3GPP TS 29.530 [53].

3c. If  the VFL client is an untrusted AF, the NWDAF acting as VFL server invokes Nnef_VFLInference_Subscribe service operation through NEF as described in clause 4.8.2.2 of 3GPP TS 29.591 [11]. The NEF invokes Naf_VFLInference_Subscribe by sending an HTTP POST request as described in clause 5.3.2.2.2 of 3GPP TS 29.530 [53]. If the Subscription is successfully processed and accepted, VFL client sends Naf_VFLInference_Subscribe response message as described in clause 5.3.2.2.2 of 3GPP TS 29.530 [53]. The NEF sends Nnef_VFLInference_Subscribe response message to VFL server as described in clause 4.8.2.2 of 3GPP TS 29.591 [11].

NOTE 1:	For the trusted AF acting as VFL server, the VFL client can only be the NWDAF.

4.	Each VFL client collects its local data. Based on the "vflCorrId" attribute received in step 1, each VFL Client determines the VFL local model to generate the intermediate local inference results.

5.	Each VFL client sends the client intermediate local results to the VFL server.

5a.	If the VFL client is an NWDAF, then it sends the response to the VFL server by invoking Nnwdaf_VFLInference_Notify service operation as described in clause 4.11.2.4.2 of 3GPP TS 29.520 [5].

5b.	If the VFL client is a trusted AF, then it sends the response to the VFL server by invoking Naf_VFLInference_Notify service operation as described in clause 5.3.2.4 of 3GPP TS 29.530 [53].

5c.	If the VFL client is an untrusted AF, then it sends the response to the VFL server to NEF by invoking Naf_VFLInference_Notify service operation as described in clause 5.3.2.4 of 3GPP TS 29.530 [53]. For each untrusted AF VFL client, the NEF converts any external identifiers to internal identifiers and the NEF invokes the Nnef_VFLInference_Notify service operation as described in clause 4.8.2.2 of 3GPP TS 29.591 [11].

6.	The VFL server combines all the intermediate local results to generate the combined inference results.

7.	If the NWDAF is the VFL server in step 1 and analytics notifications are to be sent, the VFL server NWDAF invokes Nnwdaf_EventsSubscription_Notify service operation to NWDAF containing AnLF as described in clause 4.2.2.4 of 3GPP TS 29.520 [5]. The NWDAF containing AnLF responds to the Nnwdaf_EventsSubscription_Notify service operation as described in clause 4.2.2.4 of 3GPP TS 29.520 [5].

If the trusted AF is VFL server in step 1 and analytics notifications are to be sent, the VFL server AF invokes Naf_Inference_Notify service operation to NWDAF containing AnLF as described in clause 5.3.2.4 of 3GPP TS 29.530 [53]. The NWDAF containing AnLF responds to the Naf_Inference_Notify service operation as described in clause 5.3.2.4 of 3GPP TS 29.530 [53].

8.	The NWDAF containing AnLF invokes the Nnwdaf_EventsSubscription_Notify service operation to the consumer as described in clause 4.2.2.4 of 3GPP TS 29.520 [5], or sends Nnwdaf_AnalyticsInfo_Request response message to the consumer as described in clause 4.3.2.2 of 3GPP TS 29.520 [5].

==============End of change==============