3GPP TSG CT WG3 Meeting #145	C3-260319

Goa, India, 9 - 13 February, 2026

Source:	CEWiT

Title:	Discussion paper on SMF event subscription termination

Document for:	DISCUSSION

Agenda item:	19.60

- Introduction

This discussion paper is regarding the actions to be taken corresponding to the SA2 LS reply C3-260018 for our LS out C3-254595 regarding subscription event termination indication upon PDU release.

- Background

  - SA2/ CT3 CR details

The stage 2 requirement of subscription termination indication upon PDU session release during energy information collection from SMF is captured in the agreed CR S2-2507736(CR#5508) in SA2#170. In CT3#143, an alignment CR from CEWiT C3-254035(CR#0362) was discussed. The proposal of the CR was to add the indication of subscription termination along with the cause code in the notification data type of SMF event exposure service API.

  - LS details

There was no consensus in the work group on this CR (C3-254035). There were different opinions on the interpretation of this CR content from different companies. So, it was decided to send an LS out C3-254595 to SA2 for clarification with three queries.

In SA2#172, this LS was discussed, presented on the floor by CEWiT. Among the three queries there was consensus on two queries, while on the third query of applicability of the indication to other SMF events, there were different viewpoints. After multiple discussions an LS reply C3-260018 was framed and sent to this work group.

The LS reply from SA2 says that

- In Rel 19 this indication is applicable to the event “ENERGY_USAGE_DATA”. This indication may be applied to other events, but the detailed impact should be identified by this work group and SA2 would like to be informed about it.

- The NF service consumer need not subscribe for the event “PDU_SES_REL” to get this indication.

- Upon PDU release, the SMF indicates that the subscription is terminated in the notification and the consumer need not take any further action.

- Impacts in stage 3

It is the responsibility of this work group to determine the applicability of this subscription termination and indication upon PDU release or other trigger points.

Following are the impacts based on our analysis

- All the SMF events can be subscribed per UE basis, per group of UE basis or any UE basis. So single PDU release can’t trigger subscription termination in case of subscriptions for multiple UEs or multiple PDUs of single UE.

- Adding to that, even if the subscription is per UE per PDU basis, the PDU release can’t trigger subscription termination for some specific events. One such event is “PDU_SES_EST”. For example, a consumer can create a subscription to be notified about the establishment of a PDU for some PDU setup criteria like dnn, slice or pduId. In this case upon release of the PDU if the subscription is terminated then the request of the consumer is not met fully.

- Also, if an expiry timer is provided in the subscription then terminating the subscription upon PDU release is improper unless stage 2 indicates so.

- The action of subscription termination and indication is explicitly mentioned in the procedure for energy information collection from SMF. But extending this subscription termination for all other SMF exposure events is not explicitly mentioned.

In our opinion, we conclude from our analysis, that it is best to keep the termination and its indication with the cause code to the event “ENERGY_USAGE_DATA” only. It may not be proper to extend this feature to all other SMF events considering the above analysis listed.

- Proposal

Based on the impact study, our proposal is to

- Include the subscription termination indication and cause value as PDU release in the notification data type of SMF event exposure service API.

- Provide constraints on its usage for other events. The constraints are this termination upon PDU release may not be applicable for subscriptions that are monitored across session termination(s). Also, it may not be applicable to subscriptions created multiple UEs/ PDUs.

CEWiT has provided the CR C3-260318 based on this proposal.