3GPP TSG-CT WG3 Meeting #143	C3-254368

Sophia-Antipolis, France, 13 -17 October, 2025

Source:	Ericsson, Huawei

Title:	Pseudo-CR for corrections for the AF VFL

Spec:	3GPP TS 29.530 V1.0.0

Agenda item:	19.39

Document for:	Agreement

1. Introduction

There are some mistakes in TS 29.530, including non reused data types and repeated empty clauses.

2. Reason for Change

NwdafEvent, MLModelMetric and VflIntermedTrainInfo data types are not reused by the Naf_VFLTraining API  but are present on the table of reused data types on clause 6.1.6.1.

Naf_Training Service API and Naf_Inference Service API clauses are repeated and empty.

3. Conclusions

NwdafEvent, MLModelMetric and VflIntermedTrainInfo data types must be removed from the table of reused data types on clause 6.1.6.1 since they are not directly reused by the Naf_VFLTraining API.

Repeated Naf_Training Service API and Naf_Inference Service API are misleading and must be removed.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.530 V1.0.0.

# *** 1st Change ***

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Naf_VFLTraining service-based interface protocol.

Table 6.1.6.1-1: Naf_VFLTraining API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| VflTrainingSubs | 6.1.6.2.2 | Represents a VFL Training Subscription. |  |
| VflTrainingSubsPatch | 6.1.6.2.3 | Represents the requested modifications to a VFL Training Subscription. |  |

Table 6.1.6.1-2 specifies data types re-used by the Naf_VFLTraining service-based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Naf_VFLTraining service-based interface.

Table 6.1.6.1-2: Naf_VFLTraining API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ReportingInformation | 3GPP TS 29.523 [17] | Represents the event reporting requirements. |  |
| SupportedFeatures | 3GPP TS 29.571 [16] | Represents the list of supported feature(s) and used to negotiate the applicability of the optional features. |  |
| Uinteger | 3GPP TS 29.571 [16] | Represents an unsigned integer. |  |
| Uri | 3GPP TS 29.571 [16] | Represents a URI. |  |
| VflTrainingNotify | 3GPP TS 29.520 [18] | Represents a VFL Training Notification. |  |
| VflTrainingSub | 3GPP TS 29.520 [18] | Represents a VFL Training set. |  |

# *** Next Change ***

## 6.3	Naf_Training Service API

# *** Next Change ***

## 6.4	Naf_Inference Service API

# *** End of Changes ***