3GPP TSG CT WG3 Meeting #143	C3-254370

# Sophia-Antipolis, FR, 13th – 17th October, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.591 | CR | 0247 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on the Nnef_Inference API |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  | 2025-10-06 |
| Category: | F |  |  |  |  |  | Release: |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  |  | - The inferAnaSub and targetServer attribute names in the POST, PUT and PATCH method tables are incorrect. - The InferEventSubsc and InferEventSubscPatch data types are defined in this TS and the reference in the OpenAPI file needs to be corrected. - The reference TS in clause 4.11.2.2.2, 4.11.2.2.3 needs to be updated to stage 3 TS. |  |  |  |  |  |  |
| Summary of change: |  |  | Correct the errors listed above. |  |  |  |  |  |  |
| Consequences if not approved: |  |  | Errors in the specification may lead to wrong implementation. |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.3.2.3.1, 5.8.3.3.3.1, 5.8.3.3.3.2, 5.8.6.1, 5.8.6.2.2, 5.8.6.2.3, A.9 |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |
| affected: |  |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |
| Other comments: |  | This CR introduces backwards compatible corrections to the OpenAPI descriptions of the following APIs: - TS29591_Nnef_Inference.yaml |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

5.8.3.2.3.1	POST

This method shall support the URI query parameters specified in table 5.8.3.2.3.1-1.

Table 5.8.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.8.3.2.3.1-2 and the response data structures and response codes specified in table 5.8.3.2.3.1-3.

Table 5.8.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| InferEventSubsc | M | 1 | Contains the information required for the creation of a new Individual Inference subscription resource. The "targetServerId" attribute contained in InferEventSubsc data structure shall contain the identity of the target AF VFL Server. The "exterGroupIds" and "gpsis" attributes contained in the "inferAnaSubs" attribute within this data structure are not applicable to this API. |

Table 5.8.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| InferEventSubsc | M | 1 | 201 Created | Contains the representation of the Individual Inference subscription resource. The "exterGroupIds" and "gpsis" attributes contained in the "inferAnaSubs" attribute within this data structure are not applicable to this API. |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status code for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: Failure cases are described in clause 5.8.7. |  |  |  |  |

Table 5.8.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/nnef-inference/<apiVersion>/subscriptions/{subscriptionId} |

# *** Next Change ***

5.8.3.3.3.1	PUT

This method shall support the URI query parameters specified in table 5.8.3.3.3.1-1.

Table 5.8.3.3.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.8.3.3.3.1-2 and the response data structures and response codes specified in table 5.8.3.3.3.1-3.

Table 5.8.3.3.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| InferEventSubsc | M | 1 | Modifies the existing Individual Inference subscription resource. The "exterGroupIds" and "gpsis" attributes contained in the "inferAnaSubs" attribute within this data structure are not applicable to this API. |

Table 5.8.3.3.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| InferEventSubsc | M | 1 | 200 OK | Successful case. The Individual Inference subscription resource was modified and a representation is returned. |
| n/a |  |  | 204 No Content | Successful case. The Individual Inference subscription resource was modified. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection, during subscription modification. (NOTE 3) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection, during subscription modification. (NOTE 3) |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the PUT method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: Failure cases are described in clause 5.8.7. NOTE 3: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 5.8.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NEF (service) instance towards which the request is redirected. |

Table 5.8.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NEF (service) instance towards which the request is redirected. |

# *** Next Change ***

5.8.3.3.3.2	PATCH

This method shall support the URI query parameters specified in table 5.8.3.3.3.2-1.

Table 5.8.3.3.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.8.3.3.3.2-2 and the response data structures and response codes specified in table 5.8.3.3.3.2-3.

Table 5.8.3.3.3.2-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| InferEventSubscPatch | M | 1 | Partially modify an existing inference subscription. |

Table 5.8.3.3.3.2-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| InferEventSubsc | M | 1 | 200 OK | The subscription was updated successfully. The "exterGroupIds" and "gpsis" attributes contained in the "inferAnaSubs" attribute within this data structure are not applicable to this API. |
| n/a |  |  | 204 No Content | The subscription has been successfully updated and no content is returned in the response body. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection, during subscription retrieval. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection, during subscription retrieval. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the PATCH method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 5.8.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF (service) instance towards which the request is redirected. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NEF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |

Table 5.8.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NEF (service) instance towards which the request is redirected. |

# *** Next Change ***

5.8.6.1	General

This clause specifies the application data model supported by the API.

Table 5.8.6.1-1 specifies the data types defined for the Nnef_Inference service based interface protocol.

Table 5.8.6.1-1: Nnef_Inference specific Data Types

| Data type |  | Section defined |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| InferEventSubsc |  | 5.8.6.2.2 |  | Represents a inference subscription. |  |  |  |
| InferEventSubscPatch |  | 5.8.6.2.3 |  | Represents parameters to request the modification of a inference subscription. |  |  |  |

Table 5.8.6.1-2 specifies data types re-used by the Nnef_Inference service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nnef_Inference service based interface.

Table 5.8.6.1-2: Nnef_Inference re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| InferAnaSub | 3GPP TS 29.530 [33] |  |  |
| InferNotif | 3GPP TS 29.530 [33] | Represents notification of a inference subscription. |  |
| InferReq | 3GPP TS 29.530 [33] | Represents inference requirements. |  |
| InferResults | 3GPP TS 29.530 [33] | Represents inference results. |  |
| ReportingInformation | 3GPP TS 29.523 [22] | Represents the type of reporting a subscription requires. |  |
| SupportedFeatures | 3GPP TS 29.571 [16] | Represents the list of supported features. |  |
| Uri | 3GPP TS 29.571 [16] | Represents a URI. |  |

# *** Next Change ***

5.8.6.2.2	Type InferEventSubsc

Table 5.8.6.2.2-1: Definition of type InferEventSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifCorreId | string | M | 1 | The value of Notification Correlation ID in the corresponding notification. |  |
| notifUri | Uri | M | 1 | URI at which the NF service consumer requests to receive notifications. |  |
| suppFeats | SupportedFeatures | C | 0..1 | List of Supported features used as described in clause 5.8.8. It shall be supplied by NF service consumer in the POST requests that request the creation of an AF Inference Subscriptions resource and shall be supplied by the AF in the reply of corresponding request. |  |
| inferAnaSubs | array(InferAnaSub) | M | 1..N | Identifies the inference subscription information for the subscribed analytics ID(s). |  |
| inferReq | InferReq | O | 0..1 | Represents required conditions to apply inference. |  |
| inferResults | array(InferResult) | O | 1..N | Represents inference results |  |
| reportInfo | ReportingInformation | O | 0..1 | Reporting requirement information of the inference subscription. If omitted, the default values within the ReportingInformation data type apply. |  |
| targetServerId | string | M | 1 | Target VFL Server identity. |  |

EN: 23.288 clause 11.4.1 states on EN that parameters of the Naf_Inference service operations are FFS and more will be added when procedures and content of services are agreed. This affects the parameter definition of InferEventSubsc data type.

# *** Next Change ***

5.8.6.2.3	Type InferEventSubscPatch

Table 5.8.6.2.3-1: Definition of type InferEventSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifUri | Uri | O | 0..1 | URI at which the NF service consumer requests to receive notifications. |  |
| notifCorreId | string | O | 0..1 | The value of Notification Correlation ID in the corresponding notification. |  |
| inferReq | InferReq | O | 0..1 | Represents required conditions to apply inference. |  |
| reportInfo | ReportingInformation | O | 0..1 | Reporting requirement information of the inference subscription. If omitted, the default values within the ReportingInformation data type apply. |  |

# *** Next Change ***

A.9	Nnef_Inference API

openapi: 3.0.0

info:

title: Nnef_Inference

version: 1.0.0-alpha.1

description: |

API for Nnef_Inference Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.591 V19.4.0; 5G System; Network Exposure Function Southbound Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.591/

servers:

- url: '{apiRoot}/nnef-inference/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- nnef-inference

paths:

/subscriptions:

post:

summary: Create a new Individual Inference Subscription resource.

operationId: CreateInferenceSubcription

tags:

- Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/InferEventSubsc'

responses:

'201':

description: Create a new Individual Inference Subscription resource.

content:

application/json:

schema:

$ref: '#/components/schemas/InferEventSubsc'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the

structure

{apiRoot}/nnef-inference/v1/subscriptions/{subscriptionId}.

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

myNotification:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: 'TS29530_Naf_Inference.yaml#/components/schemas/InferNotif'

responses:

'204':

description: No Content, Notification was successful

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

put:

summary: Update an existing Individual Inference Subscription

operationId: UpdateInferenceSubcription

tags:

- Individual Inference Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/InferEventSubsc'

parameters:

- name: subscriptionId

in: path

description: String identifying an Individual Inference Subscription.

required: true

schema:

type: string

responses:

'200':

description: >

The Individual Inference Subscription resource was modified

successfully and a representation of that resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/InferEventSubsc'

'204':

description: >

The Individual Inference Subscription resource was modified

successfully.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Partial update an existing Individual Inference Subscription

operationId: PartialUpdateInferenceSubcription

tags:

- Individual Inference Subscription (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/InferEventSubscPatch'

parameters:

- name: subscriptionId

in: path

description: String identifying an Individual Inference Subscription.

required: true

schema:

type: string

responses:

'200':

description: >

The Individual Inference Subscription resource was partial

modified successfully and a representation of that resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/InferEventSubsc'

'204':

description: >

The Individual Inference Subscription resource was partial

modified successfully.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Delete an existing Individual Inference Subscription.

operationId: DeleteInferenceSubcription

tags:

- Individual Inference Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: >

String identifying an Individual Inference Subscription.

required: true

schema:

type: string

responses:

'204':

description: >

No Content. The Individual Inference Subscription matching the

subscriptionId was deleted.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nnef-inference: Access to the Nnef_Inference API

schemas:

InferEventSubsc:

description: Represents an Inference subscription.

type: object

properties:

notifCorreId:

type: string

description: >

String identifying the Notification Correlation ID in the corresponding

notification.

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

suppFeats:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

inferAnaSubs:

type: array

items:

$ref: 'TS29530_Naf_Inference.yaml#/components/schemas/InferAnaSub'

minItems: 1

description: Represents inference subscription per analytics Id.

inferReq:

$ref: 'TS29530_Naf_Inference.yaml#/components/schemas/InferReq'

inferResults:

type: array

items:

$ref: 'TS29530_Naf_Inference.yaml#/components/schemas/InferResult'

minItems: 1

description: Represents Inference result.

reportInfo:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

targetServerId:

type: string

description: >

String identifying target VFL server id.

required:

- notifUri

- notifCorreId

- inferAnaSubs

- targetServerId

InferEventSubscPatch:

description: >

Represents parameters to request the modification of an Inference

subscription.

type: object

properties:

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

notifCorreId:

type: string

description: >

String identifying the Notification Correlation ID in the corresponding

notification.

inferReq:

$ref: 'TS29530_Naf_Inference.yaml#/components/schemas/InferReq'

reportInfo:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

# *** End of Changes ***